3GPP TSG-RAN WG4 Meeting # 116	R4-2510601

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.26.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM requirements for Rel-19 XR

Document for:	Discussion

# Introduction

In last RAN4 meeting, RAN4 discussed the RRM impacts of the WI with agreements and issues captured in the WF [1]. In this contribution, we further provide our views on remaining issues.

# Discussion

Regarding the type of MG which is applicable for MG cancelling, the relevant agreements are summarized as follows:

## Issue 2-1: Type 2 gaps <Agreement> - MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules. - The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules. - DCI skipping cannot be applied if any gap is configured with gapAssociationPRS - FFS the applicability of the gap cancellation if two non colliding concurrent gaps are separated by less than X ms ## Issue 2-2: NCSG <Agreement> - MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with ncsgInd-r17. ## Issue 2-3: Preconfigured measurement gaps <Agreement> - MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with preConfigInd-r17. ## Issue 2-4: Rel-18 concurrent gaps <Agreement> - MG cancelling DCI doesn’t apply to Rel-18 concurrent gaps.

One FFS point is the applicability of gap cancellation if two non-colliding concurrent gaps are separated by less than X ms. This issue was identified during the discussion in last meeting. As shown in following figure, this issue only exist for the case that the interval between two MG are less than 5ms and UE does not support the 3ms capability. The problem is how can gNB cancel MG2. Based on the agreed TP in last RAN1 meeting which is shown as follows, if gNB want to cancel MG2 in Fig.1, gNB can trigger the DCI before MG1, and the applicable earliest RRM MG that starts 3ms or 5ms after the end of PDCCH is MG2.

An XYZ measurement gap cancellation field in a DCI format [5, TS 38.212] provided by a PDCCH reception is associated to an earliest RRM measurement gap occasion that starts at least 3 msec or 5 msec, as determined by a reported UE capability, after the end of the PDCCH reception. The indication by the measurement gap cancellation field is applied to the associated RRM measurement gap occasion on all cells in the applicable range of the measurement gap occasion that include the one or more scheduled cells associated with the DCI format [10, TS 38.133] as described below.

Observation 1: Based on RAN1 spec, it is clear which MG to be cancelled for non-colliding concurrent gaps.

Fig.1 MG cancel for non-colliding concurrent gaps

Proposal 1: Based on RAN1 agreements, it is clear which MG is to be cancelled for non-colliding concurrent gaps.

Another remaining issue is about the how to formulate the RRM delay extension. The status is summarized as follows:

<Way forward> - RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Option 1: Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - Option 2: Max( 200 ms, ( 8 + Nskip )  Max(MGRP , SMTC period) )  CSSFinter - Option 3: - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms - where Tskip = Nskip  Max(MGRP , SMTC period) - Other options are not precluded - where Nskip is the number of skipped MG occasions in the measurement period. - FFS: CSSFinter

The controversial part is whether the extended number of occasions shall be also scaled by CSSF. Companies support option 1 commented that it will makes the delay extension looks shorter than option 1. However, it assumes the UE implementation that UE will keep measurement on the same carrier until enough samples are obtained. From our understanding, it only makes the delay extension look shorter for this particular carrier, and the delay for other carriers will be much longer. From UE implementation perspective, after several MG occasions are cancel, the optimized algorithm maybe not staying on the same carrier but switch to other carriers. This may related to the SMTC configurations for difference carriers and even the channel conditions. Thus, Option 2 is more reasonable which is also the approached adopted in NR-U.

Proposal 2: Use following formula to define the delay extension. The all candidate carriers using the MG shall be extended.

- Option 2:

- RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter

    - where Nskip is the number of skipped MG occasions in the measurement period.

- the delay shall be extended accordingly for all frequencies which are candidates to be measured using the gap.

Another issue is about the applicable range of MG canceling, and RAN4 achieved following agreements in last meeting.

<Agreement>: - It is RAN4 common understanding that: If UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FRs. - Further discuss whether it has already been specified in RAN1 CR, and whether/how to capture in RAN4 spec.

Companies commented that whether this has already been reflected in RAN1 spec. After checking relevant RAN1 agreements and the approved RAN1 CR, which is shown as follows. Based on the highlighted parts, we believe RAN1 is responsible to clarify the applicable range in RAN4 spec.

Conclusion It is RAN1 understanding that if measurement gap/restriction is indicated to be skipped by explicit indication by DCI, the TX/RX is enabled on all the serving cells in the applicable range of the skipped measurement gap/restriction. Note: Applicable range of measurement gap/restriction is based on RAN4 specifications A measurement gap cancellation field in a DCI format [5, TS 38.212] provided by a PDCCH reception is associated to an earliest RRM measurement gap occasion that starts at least 3 msec or 5 msec, as determined by a reported UE capability, after the end of the PDCCH reception. The indication by the measurement gap cancellation field is applied to the associated RRM measurement gap occasion on all cells in the applicable range of the measurement gap occasion that include the one or more scheduled cells associated with the DCI format [10, TS 38.133].

The TP to clarify the applicable range is as follows:

The applicable range of measurement gap cancelling includes the scheduled Cell(s) and Cell(s) in the same frequency range when UE is configured with per-FR gap; otherwise, applicable range of measurement gap cancelling applies to both frequency ranges.

Proposal 3: Clarify in RAN4 spec about the applicable frequency range as follows:

The applicable range of measurement gap cancelling includes the scheduled Cell(s) and Cell(s) in the same frequency range when UE is configured with per-FR gap; otherwise, applicable range of measurement gap cancelling applies to both frequency ranges.

# Conclusions

Observation 1: Based on RAN1 spec, it is clear which MG to be cancelled for non-colliding concurrent gaps.

Proposal 1: Based on RAN1 agreements, it is clear which MG is to be cancelled for non-colliding concurrent gaps.

Proposal 2: Use following formula to define the delay extension. The all candidate carriers using the MG shall be extended.

- Option 2:

- RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter

    - where Nskip is the number of skipped MG occasions in the measurement period.

- the delay shall be extended accordingly for all frequencies which are candidates to be measured using the gap.

Proposal 3: Clarify in RAN4 spec about the applicable frequency range as follows:

The applicable range of measurement gap cancelling includes the scheduled Cell(s) and Cell(s) in the same frequency range when UE is configured with per-FR gap; otherwise, applicable range of measurement gap cancelling applies to both frequency ranges.

# References

[1] R4-2508285 WF on RRM requirements for XR_Ph3