3GPP TSG-RAN WG4 Meeting #116	R4-2510605

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.22.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impacts for R19 A-IoT device

Document for:	Discussion

# Introduction

The study item of Ambient IoT [1] was completed with WI approve in [2] and further updated in [3]. The RRM impacts were discussed in last RAN4 meeting with agreements captured in [4]. In this contribution, we provide our views on remaining issues for Ambient-IoT.

# Discussion

In the following part of the contribution, we provide our analysis on RRM requirements for RA and transmit timing requirements based on progress from other WGs.

## Random access

It is agreed in last RAN4 meeting to include following aspects for RRM requirements for A-IoT random acces.

Issue 1-1: A-IoT random access procedure Agreement: - RAN4 to include following aspects for RRM requirements for A-IoT random access. The details of UE behaviour will be updated based on RAN2 spec. - CBRA procedure - the correct behaviour when transmitting A-IoT MSG1(Random ID transmission) - the correct behaviour when receiving A-IoT MSG2 - the correct behaviour when not receiving A-IoT MSG2 - CFRA procedure - the correct behaviour when transmitting First D2R message

Based on the endorsed running CR [5] and the running CR before Aug meeting, the correct behaviour for above procedures are summarized as follows:

For CBRA, if the RA type is indicated as CBRA in paging message and the device is considered as selected, the device initiates contention-based random access, as defined in TS 38.391 clause 5.2

- Device shall select the access occasion and initiate the transmission of Random ID message, as defined in TS 38.391 clause 5.3.1.1 and 5.3.1.2.

- Once the random ID message is transmitted, if the CBRA procedure is considered as successful, the device shall initiate D2R message transmission, as defined in TS 38.391 5.3.1.3 and 5.4.

- Upon reception of A-IoT Paging message, if CBRA procedure has not been considered as successful, the device consider that the current procedure associated with the stored Transaction ID failed, as defined in TS 38.391 5.5

For CFA, if the RA type is indicated as CRA in paging message, UE shall initiate contention-free access, and initiate D2R message transmission, as defined in TS 38.391 clause 5.2 and 5.3.2

It should be noted that the detailed device behaviour is still under discussion, and maybe further updated in next Aug meeting, the device RA behaviour shall be generic and consistent with RAN2 MAC spec, and can be future updated with RAN2 progress.

Proposal 1: RAN4 to capture correct device behaviour as follows:

For CBRA, if the RA type is indicated as CBRA in paging message and the device is considered as selected, the device initiates contention-based random access, as defined in TS 38.391 clause 5.2

- Device shall select the access occasion and initiate the transmission of Random ID message, as defined in TS 38.391 clause 5.3.1.1 and 5.3.1.2.

- Once the random ID message is transmitted, if the CBRA procedure is considered as successful, the device shall initiate D2R message transmission, as defined in TS 38.391 5.3.1.3 and 5.4.

- Upon reception of A-IoT Paging message, if CBRA procedure has not been considered as successful, the device consider that the current procedure associated with the stored Transaction ID failed, as defined in TS 38.391 5.5

For CFA, if the RA type is indicated as CRA in paging message, UE shall initiate contention-free access, and initiate D2R message transmission, as defined in TS 38.391 clause 5.2 and 5.3.2

Note: The details could be updated based on further RAN2 progress to be consistent with RAN2 MAC spec.

## Device transmit timing requirements

In last RAN4 meeting, RAN4 discussed and agreed on the basic framework of timing requirements. The status is shown as follows:

Issue 1-2-1: Device transmit timing Agreement: - RAN4 define D2R transmit timing requirements. - Define D2R transmit timing error requirements as follows: - For MSG1 X=1, - The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1. - For MSG1 X=2, follow the same principle as starting point. - Note 1: The timing error requirements for other D2R could be updated accordingly based on further RAN1 agreements. - Note 2: The end of the R2D transmission and last edge could be updated based on further RAN1 agreements if any.

Based on the latest progress in last RAN1 meeting, we provide our views on necessary updating and timing error for other D2R transmission. The relevant RAN1 agreements are shown as follows:

Agreement When the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used as follows: - Alt 1a: The content of padding is up to reader implementation and transparent to device. - The timeline determination of any timing relationship refers to the end of padding. - Note: it implies the device should be aware of the duration of padding or the last OFDM symbol boundary by implementation. - Note: the padding time could be used for extra time needed for calculating FEC/CRC for D2R (if applied) - The end chip(s) of the padding content shall follow the CP handling solution determined in RAN1, and may be affected by other agreements. - The chip(s) of the padding content shall avoid to result in SIP pattern. Agreement R2D postamble is specified with 4 ON chips corresponding to M value of the PRDCH - R2D postamble is added immediately after the PRDCH - R2D postamble has always 4 ON chips - Note: For M=24, 2 ON chips at the end of OFDM symbol for CP handling are in addition to R2D postamble, and are not part of the R2D postamble - R2D padding duration is determined after R2D postamble insertion TBS information for R2D is supported via higher layer R2D control signalling. - Send LS to RAN2 asking to include R2D TBS information (excluding CRC length) in higher layer signaling, at least for messages with variable size. Note: Exact method for determining the end of PRDCH at the device is not specified. Agreement For the time interval from the end of a R2D transmission triggering CFRA to the starting time of the first D2R message time domain resource, from the device perspective, reuse Toffset3. Note: this assumes the maximum size of the first D2R message for CFRA is 128 bits. Agreement Define Toffset3, which is the time interval from the end of a R2D transmission for Msg2 to the starting time of the corresponding Msg3 time domain resource, from the device perspective. Agreement Define Toffset4, which is the time interval from the end of a R2D transmission to the starting time of the corresponding D2R time domain resource except for Msg1 and Msg3 transmission, from the device perspective. Agreement It is up to RAN4 to define the value(s) for TD2R_min. - Note: RAN1 expects that the value(s) for TD2R_min will be defined in RAN4 specifications

One of the remaining issues is about the definition of last edge, which depends on RAN1 design on padding and postamble. As explained in last meeting, if there is a pre-defined the edge in padding or postamble can be guaranteed, UE can use the edge for synchronization. Otherwise, UE can only use the last edge of PRDCH. It could be observed from RAN1 agreement that:1) the content of padding is up to reader implementation and transparent to device; 2) the postamble is specified with 4 ON chips.

Observation 1: There is no pre-defined transition edge can be guaranteed in padding or postamble for synchronization.

Observation 2: The last edge that UE can utilized for synchronization is the last transition edge of PRDCH.

Proposal 2: The last edge of the corresponding R2D transmission is clarified as the last transition edge of PRDCH.

In last RAN4 meeting, we only discuss and define the timing error for MSG1 X=1, for other D2R transmission, similar approach can be adopted based on RAN1 agreement.

CBRA

- MSG1 X=1 (agreed in RAN4#115)

- MSG1 X=2

Working assumption For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource: - Derived based on the starting time of the first Msg1 time domain resource plus Toffset2 - FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access.

Thus, for MSG 1 X=2, the reference point for the device transmission timing is Toffset1+ Toffset2 after the end of corresponding R2D transmission as define in TS 38.291

- MSG 3

Agreement Define Toffset3, which is the time interval from the end of a R2D transmission for Msg2 to the starting time of the corresponding Msg3 time domain resource, from the device perspective.

Thus, for MSG3, the reference point for the device transmission timing is Toffset3 after the end of R2D transmission for MSG2 as define in TS 38.291

- Other D2R

Agreement Define Toffset4, which is the time interval from the end of a R2D transmission to the starting time of the corresponding D2R time domain resource except for Msg1 and Msg3 transmission, from the device perspective.

Thus, for other D2R transmission except for MSG1 and MSG3, the reference point for the device transmission timing is Toffset4 after the end of corresponding R2D transmission as define in TS 38.291

CFRA

- First D2R message

Agreement For the time interval from the end of a R2D transmission triggering CFRA to the starting time of the first D2R message time domain resource, from the device perspective, reuse Toffset3. Note: this assumes the maximum size of the first D2R message for CFRA is 128 bits.

Thus, for first D2R the reference point for the device transmission timing is Toffset3 after the end of corresponding R2D transmission as define in TS 38.291

- Other D2R

Thus, for other D2R transmission except for first D2R transmission, the reference point for the device transmission timing is Toffset4 after the end of corresponding R2D transmission as define in TS 38.291

Observation 3: The reference point for each D2R transmission is defined as follows:

- CBRA MSG1 X=2: the reference point for the device transmission timing is Toffset1+ Toffset2 after the end of corresponding R2D transmission as define in TS 38.291

- CBRA MSG3: the reference point for the device transmission timing is Toffset3 after the end of R2D transmission for MSG2 as define in TS 38.291

- CBRA other D2R: the reference point for the device transmission timing is Toffset4 after the end of corresponding R2D transmission as define in TS 38.291

- CFRA first D2R: the reference point for the device transmission timing is Toffset3 after the end of corresponding R2D transmission as define in TS 38.291

- CFRA other D2R: the reference point for the device transmission timing is Toffset4 after the end of corresponding R2D transmission as define in TS 38.291

Based on the newly released PHY spec TS 38.291, it could be observed that a generic term TR→D is used for different D2R transmission with different values. Thus, the correspond timing error can be defined as follows:

--------------------------------------------------------------Text proposal----------------------------------------------------------

The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last transition edge of PRDCH of the corresponding R2D transmission to the starting time of D2R transmission. The reference points are defined as the TR→D after the end of chip χR2D= χendR2D of the corresponding R2D transmission as defined TS 38.291.

--------------------------------------------------------------Text proposal----------------------------------------------------------

Proposal 3: The timing error requirements for D2R transmission are defined as:

--------------------------------------------------------------Text proposal----------------------------------------------------------

The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last transition edge of PRDCH of the corresponding R2D transmission to the starting time of D2R transmission. The reference points are defined as the TR→D after the end of chip χR2D= χendR2D of the corresponding R2D transmission as defined TS 38.291.

--------------------------------------------------------------Text proposal----------------------------------------------------------

Another issue which may be related to timing requirements is about TD2R_min. RAN1 agreed that it is up to RAN4 to define the value of TD2R_min which is shown as follows.

Agreement It is up to RAN4 to define the value(s) for TD2R_min. Note: RAN1 expects that the value(s) for TD2R_min will be defined in RAN4 specifications

If we take RFID as reference, TD2R_min serve the same purpose as T2 in RFID spec. In RFID spec, T2 is defined as a range. For A-Iot, only TD2R_min will be defined without TD2R_max. From device implementation perspective, it is about the minimum processing time that a device shall be ready for R2D reception after D2R transmission.

EPC® Radio-Frequency Identity Generation-2 UHF RFID Standard

Observation 4: TD2R_min is the minimum processing time that a device needs before it is ready for R2D reception after D2R transmission. After TD2R_min, the device is ready for R2D reception.

Consider some implementation capability, 10us is reasonable value.

Proposal 4: TD2R_min is defined as 10 us.

# Conclusions

Proposal 1: RAN4 to capture correct device behaviour as follows:

For CBRA, if the RA type is indicated as CBRA in paging message and the device is considered as selected, the device initiates contention-based random access, as defined in TS 38.391 clause 5.2

- Device shall select the access occasion and initiate the transmission of Random ID message, as defined in TS 38.391 clause 5.3.1.1 and 5.3.1.2.

- Once the random ID message is transmitted, if the CBRA procedure is considered as successful, the device shall initiate D2R message transmission, as defined in TS 38.391 5.3.1.3 and 5.4.

- Upon reception of A-IoT Paging message, if CBRA procedure has not been considered as successful, the device consider that the current procedure associated with the stored Transaction ID failed, as defined in TS 38.391 5.5

For CFA, if the RA type is indicated as CRA in paging message, UE shall initiate contention-free access, and initiate D2R message transmission, as defined in TS 38.391 clause 5.2 and 5.3.2

Note: The details could be updated based on further RAN2 progress to be consistent with RAN2 MAC spec.

Observation 1: There is no pre-defined transition edge can be guaranteed in padding or postamble for synchronization.

Observation 2: The last edge that UE can utilized for synchronization is the last transition edge of PRDCH.

Proposal 2: The last edge of the corresponding R2D transmission is clarified as the last transition edge of PRDCH.

Observation 3: The reference point for each D2R transmission is defined as follows:

- CBRA MSG1 X=2: the reference point for the device transmission timing is Toffset1+ Toffset2 after the end of corresponding R2D transmission as define in TS 38.291

- CBRA MSG3: the reference point for the device transmission timing is Toffset3 after the end of R2D transmission for MSG2 as define in TS 38.291

- CBRA other D2R: the reference point for the device transmission timing is Toffset4 after the end of corresponding R2D transmission as define in TS 38.291

- CFRA first D2R: the reference point for the device transmission timing is Toffset3 after the end of corresponding R2D transmission as define in TS 38.291

- CFRA other D2R: the reference point for the device transmission timing is Toffset4 after the end of corresponding R2D transmission as define in TS 38.291

Proposal 3: The timing error requirements for D2R transmission are defined as:

--------------------------------------------------------------Text proposal----------------------------------------------------------

The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last transition edge of PRDCH of the corresponding R2D transmission to the starting time of D2R transmission. The reference points are defined as the TR→D after the end of chip χR2D= χendR2D of the corresponding R2D transmission as defined TS 38.291.

--------------------------------------------------------------Text proposal----------------------------------------------------------

Observation 4: TD2R_min is the minimum processing time that a device needs before it is ready for R2D reception after D2R transmission. After TD2R_min, the device is ready for R2D reception.

Consider some implementation capability, 10us is reasonable value.

Proposal 4: TD2R_min is defined as 10 us.

# References

[1] RP‑234058 New SID: Study on solutions for Ambient IoT (Internet of Things) in NR

[2] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[3] RP-250796 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[4] R4-2508275 WF on RRM requirements for Ambient_IoT_Solutions

[5] R2-2503997 A-IoT MAC running CR	 draft TS	Rel-19	38.391	0.0.2