3GPP TSG-RAN WG4 Meeting #116	R4-2510607

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.22.7

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM performance for R19 A-IoT device

Document for:	Discussion

# Introduction

The study item of Ambient IoT [1] was completed with WI approve in [2] and further updated in [3]. The RRM impacts were discussed in last RAN4 meeting with agreements captured in [4]. The performance requirements are scheduled to started from this meeting. In this contribution, we provide our initial views on RRM performance impacts.

# Discussion

In previous meetings, the RRM impacts on core requirements are identified as follows:

## Sub-topic 1-1: A-IoT random access procedure Issue 1-1: A-IoT random access procedure Agreement: - RAN4 to include following aspects for RRM requirements for A-IoT random access. The details of UE behaviour will be updated based on RAN2 spec. - CBRA procedure - the correct behaviour when transmitting A-IoT MSG1(Random ID transmission) - the correct behaviour when receiving A-IoT MSG2 - the correct behaviour when not receiving A-IoT MSG2 - CFRA procedure - the correct behaviour when transmitting First D2R message ## Sub-topic 1-2: timing Issue 1-2-1: Device transmit timing Agreement: - RAN4 define D2R transmit timing requirements. - Define D2R transmit timing error requirements as follows: - For MSG1 X=1, - The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.291. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1. - For MSG1 X=2, follow the same principle as starting point. - Note 1: The timing error requirements for other D2R could be updated accordingly based on further RAN1 agreements. - Note 2: The end of the R2D transmission and last edge could be updated based on further RAN1 agreements if any.

For the performance part, the following-up question is whether and how to define test cases to verify these requirements. For NR RRM requirements, RAN4 defined dedicated RA and timing requirements Test cases to verify these requirements. However, for A-IoT device, the support functionalities are very limited. The only support service is inventory and command. And the supported R2D and D2R signals are also limited. As discussed in Device test method [5], which is shown as follows, it could be observed that similar behaviour/requirements (i.e. correct response and correct timing relationship) can also be verified in other requirements. For instance, the demod requirements may also be verified by correct response upon receiving R2D message. The same behaviour may also be verified in the signalling test in RAN5.

Issue 4-5-1: testing performance metric of reference sensitivity Agreement: - Use following as starting point: - Measuring device’s correct response and correct respond timing relationship between R2D and the corresponding D2R transmission with a minimum mean power applied for device antenna. - FFS on how to set the side condition including CW signal power.

Observation 1: Due to very limited supported functionalities, the same behaviour/requirements may also be implicitly verified in other requirements/test (e.g. RF/Demod requirements).

Considering that A-IoT device are extremely low-case, limiting the number of tests is also important to reduce the cost of the device.

Observation 2: Limiting the number/complexity of tests are important to reduce the cost of A-IoT device.

Proposal 1: RAN4 to discuss whether to introduce RRM test cases for A-IoT device. If the same behaviour/requirements can be (implicitly) verified by other requirements/test (e.g. RF and demod), there is no need to introduce redundant test case.

If RRM specific test cases are to be introduced, the test method also needs to be discussed. Based on the parallel discussion in test method, the OTA test is assumed due to no antenna connector for A-IoT device. The sensitivity and backscatter RF requirements are considered as the example for test method discussion. The same test method can be reused for RRM test.

Observation 3: The OTA test mothed under discussion for RF requirements can be reused for RRM test if any RRM test cases are introduced.

# Conclusions

Observation 1: Due to very limited supported functionalities, the same behaviour/requirements may also be implicitly verified in other requirements/test (e.g. RF/Demod requirements).

Observation 2: Limiting the number/complexity of tests are important to reduce the cost of A-IoT device.

Proposal 1: RAN4 to discuss whether to introduce RRM test cases for A-IoT device. If the same behaviour/requirements can be (implicitly) verified by other requirements/test (e.g. RF and demod), there is no need to introduce redundant test case.

Observation 3: The OTA test mothed under discussion for RF requirements can be reused for RRM test if any RRM test cases are introduced.

# References

[1] RP‑234058 New SID: Study on solutions for Ambient IoT (Internet of Things) in NR

[2] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[3] RP-250796 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[4] R4-2508275 WF on RRM requirements for Ambient_IoT_Solutions

[5] R4-2505230 WF on A-IOT device requirements