3GPP TSG-RAN WG4 Meeting # 116		R4-2510610

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	5.27.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on requirements maintenance for R18 NES

Document for:	Discussion

# Introduction

RAN4 discussed the maintenance issues for Rel-18 NES in previous RAN4 meeting with agreements captured in [1][2], and RAN4 sent the LS [3] to RAN2 about RAN4 agreements. In last RAN2 meeting, RAN2 achieved agreements and sent the LS reply to RAN4 [4].

# Discussion

The controversial issue in maintenance stage is about the neighbor cell measurement on SSB-less carriers, which involved both RAN2 and RAN4 discussion. Based on previous RAN4 discussion, RAN4 achieved following agreements [1][2].

Issue 1-1-1: Neighbour cells on carrier of SSB-less SCell Agreement: - From RAN4 understanding, the definition of intra-frequency/inter-frequency, whether MG is needed and which requirements apply depends on whether servingCellMO is present or not. - Based on the agreements in RAN4#113 meeting, RAN4 provide analysis for following cases: - If the servingCellMO indication is configured: - Intra/inter: The neighbor cell measurement is defined as intra-frequency when centre frequency of the SSB of the neighbour cell is same as that of SSB frequency configured in servingCellMO; otherwise, it is defined as inter-frequency - Requirements: Add clause numbers for corresponding requirements (i.e. intra/inter and with/without MG) - Intra-frequency without measurement gap: Clause 9.2.5 TS 38.133 - Intra-frequency with measurement gap: Clause 9.2.6 TS 38.133 - Inter-frequency with measurement gap: Clause 9.3.4 TS 38.133 - Inter-frequency without measurement gap: Clause 9.3.9 TS 38.133 - If the servingCellMO indication is not configured: - Intra/inter: Based current RAN4 framework on intra-f/inter-f definition, the neighbor cell measurement is defined as inter-frequency Requirements: Add clause numbers for corresponding requirements (i.e. inter and with/without MG) - Inter-frequency with measurement gap: Clause 9.3.4 TS 38.133 - Inter-frequency without measurement gap: Clause 9.3.9 TS 38.133 - Other information to RAN2: - Based on above information, it is up to RAN2 to decide whether servingCellMO can be present or not and detailed signalling design. - The potential conflict (if any) when servingCellMO is present is up to RAN2 to handle. Note: servingCellMO is based on the current RAN2 specification, it does not preclude any RAN2 discussion or signalling design for the same purpose.

Based on RAN2 discussion, RAN2 achieved following agreements [4]:

Introduce per UE capability to indicate whether the UE supports the configuration of servingCellMO for SCell that does not transmit SS/PBCH block. A UE supporting this feature shall also support NR intra-frequency measurements on neighbour cells based on servingCellMO associated with SCell that does not transmit SS/PBCH block. NW can optionally configure servingCellMO for SCell that does not transmit SS/PBCH block if UE indicate such capability.

Based on above agreements, it could be observed that:

- If UE support intraF-NeighMeasForSCellWithoutSSB and servingCellMO is configured for the SSB-less, the neighbor cell measurement on the carries is considered as intra-frequency if the SSB frequency configured in the measurement object associated with the serving and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs is also the same.

- If UE does not support intraF-NeighMeasForSCellWithoutSSB or if UE supports intraF-NeighMeasForSCellWithoutSSB but servingCellMo is not configured for the SSB-less Cell, the neighbor cell measurement on the carries is considered as inter-frequency.

Observation 1: Based on RAN2 agreements and clarifications

- If UE support intraF-NeighMeasForSCellWithoutSSB and servingCellMO is configured for the SSB-less, the neighbor cell measurement on the carries is considered as intra-frequency if the SSB frequency configured in the measurement object associated with the serving and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs is also the same.

- If UE does not support intraF-NeighMeasForSCellWithoutSSB or if UE supports intraF-NeighMeasForSCellWithoutSSB but servingCellMo is not configured for the SSB-less Cell, the neighbor cell measurement on the carries is considered as inter-frequency.

It should be noted that RAN2 made the clarification about the intra-frequency definition in TS 38.300 since Rel-15, which is not only for SSB-less operation. The corresponding change can be found as follows:

R2-2504977

Thus, RAN4 spec about intra-frequency measurement shall also be updated accordingly. The corresponding change could be found in our accompanied CR.

Proposal 1: Update RAN4 spec about intra-frequency measurement definition to align with TS 38.300.

# Conclusions

Observation 1: Based on RAN2 agreements and clarifications

- If UE support intraF-NeighMeasForSCellWithoutSSB and servingCellMO is configured for the SSB-less, the neighbor cell measurement on the carries is considered as intra-frequency if the SSB frequency configured in the measurement object associated with the serving and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs is also the same.

- If UE does not support intraF-NeighMeasForSCellWithoutSSB or if UE supports intraF-NeighMeasForSCellWithoutSSB but servingCellMo is not configured for the SSB-less Cell, the neighbor cell measurement on the carries is considered as inter-frequency.

Proposal 1: Update RAN4 spec about intra-frequency measurement definition to align with TS 38.300.

# References

[1] R4-2420121 WF on RRM requirements for NR network energy saving

[2] R4-2502585 WF on RRM requirements for Netw_Energy_NR

[3] R4-2502694 LS on neighboring cell measurement for Rel-18 SSB-less

[4] R2-2504927 Reply LS on neighboring cell measurement for Rel-18 SSB-less