3GPP TSG-RAN WG4 Meeting #116	R4-2510646

Bengaluru, India, 25 – 29 August, 2025

Title: 	Discussion on RRM requirements for NTN in Ku band

Source: 	Huawei, HiSilicon

Agenda item:	7.6.4.5

Document for:	Discussion

Introduction

RRM performance requirements for less than 5MHz for NTN are discussed in RAN4#115, and the outcomes are captured in [1]. Based on [1] the following issues need to be further discussed.

- Test case list

In this paper, we will provide our views on RRM test cases for less than 5MHz for NTN.

Discussion

Issue 2-2-2: Test cases to verify the core requirements reusing NTN Agreement: The new test cases of cell reselection for NTN less than 5MHz can be defined. And the detailed testing configuration and requirements (e.g. TSI-NR) can be further discussed: - Option 1: captured in the new sub clause in NTN clause - Option 2: an additional test configuration in the exsiting sub clause for NTN

RAN4 agreed to define new test case for cell reselection for NTN less than 5MHz. Among the 2 options for how to capture it in spec, we support option 1.

- Option 1 is following same approach as TN less than 5MHz.

- Going with option 2, RAN4 would need to discus the applicability of different test configurations, i.e. whether UE needs pass cell reselection test case for both 10MHz and less than 5MHz. Following option 1, UE needs to pass both, and of course this can be achieved also with option 2 with clear applicability, but there is then no benefit from option 2 as the amount of spec change is even larger.

Proposal 1: Capture the new test cases of cell reselection for NTN less than 5MHz in the new sub-clause in NTN clause.

Issue 2-2-3 Test cases configuations Agreements/WF: The key testing configuration parameters for the agreed test cases are summarized in the table below. Companies are encouraged to input your views on the FFS parameters in the table below in the next meeting.

There are some remaining details to be fixed for each agreed test case.

- For CHO, we support to test CHO with L3 measurement. In core part, the extension of T∆ for this case is 2*Trs, which is different from other mobility procedure. It is meaningful to have a specific test for it.

- For satellite switch, it is straightforward to use RACH based which is same as existing TC. We see no reason to change the ending point for less than 5MHz.

- For RLM, BFR and event triggered reporting test cases, we support to test non-DRX. The test requirements are more stringent and the test time is shorter.

- For event triggered reporting test cases, we support to test no-gap measurement for intra-frequency and gap based measurement for inter-frequency, both are basic features supported by all UEs.

Proposal 2: RAN4 to adopt the following detailed test setup for less than 5MHz in NTN.

- CHO: CHO with L3 measurement

- Satellite switch: RACH based

- RLM, BFR and event triggered reporting: non-DRX

- Event triggered reporting: no-gap for intra-frequency and gap for inter-frequency

Conclusions

In this paper we provided our views on RRM test cases for less than 5MHz for NTN.

Proposal 1: Capture the new test cases of cell reselection for NTN less than 5MHz in the new sub-clause in NTN clause.

Proposal 2: RAN4 to adopt the following detailed test setup for less than 5MHz in NTN.

- CHO: CHO with L3 measurement

- Satellite switch: RACH based

- RLM, BFR and event triggered reporting: non-DRX

- Event triggered reporting: no-gap for intra-frequency and gap for inter-frequency

Reference

- R4-2508261, WF on RRM requirements for NR_IoT_NTN_req_test_enh, Moderator (Xiaomi)