3GPP TSG-RAN WG4 Meeting #116	R4-2510648

Bengaluru, India, 25 – 29 August, 2025

Title: 	Discussion on RRM testing for NGSO

Source: 	Huawei, HiSilicon

Agenda item:	7.6.5.2

Document for:	Discussion

Introduction

Based on the WID [1], there is an objective in the performance part of the WI to define RRM testing with TE-emulated channel model with varying Doppler and delay shifts.

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

In this paper, we will provide our views on RRM testing for NGSO.

Discussion

Background

BDaT session has studied the TE-emulated channel model with varying Doppler and delay shifts for several meetings, and the basic procedure is

- Step 1: Set the initial ephemeris information parameters and initial UE location parameters at T0

- Step 2: Generate the estimated satellite position/velocity vector based on Kepler’s equation and equation of motion at the time T1 with the initial ephemeris information provided in step 1

- Step 3: Obtain the propagation delay and Doppler shift trajectory based on the interpolation operation at the anytime between T0 and time T1.

The details of each step have also been agreed in BDaT session, e.g. the initial ephemeris information and UE location, equations to derive estimated satellite position/velocity vector, and definition of Doppler and Delay as a function of time. In addition, RAN4 will define accuracy requirements on the generated Doppler and Delay, and TE should meet the requirements.

In RAN4#115, RRM session agreed that there will be no core requirement impact on UE uplink transmit timing or gradual timing adjustment due to use of the new channel model.

### Issue 3-1-1: UE uplink transmit timing requirements impacts Agreement: No core requirement impact on UE uplink transmit timing requirements for both NR NTN and IoT-NTN ### Issue 3-1-2: Gradual timing adjustment requirements impacts Agreement: No core requirement impact on gradual timing adjustment requirements for both NR NTN and IoT-NTN

Based on the agreement, RRM session needs to discuss how to apply the TE-emulated channel model with varying Doppler and delay shifts in test.

Test case

One issue proposed in RAN4#115 is to which test cases the new channel model should be applied. One option is to apply it to UL transmit timing test cases and the other option is to apply it to measurement and mobility test cases in addition to UL transmit timing test cases.

We support the first option.

- Procedure wise, the WID [1] is very clear that only UL transmit timing test cases are considered for the new channel model. We believe a change to the WID is needed in RAN Plenary before RAN4 extends the scope to cover more test cases.

- Technically, the main motivation to define the new channel mode is to verify that UE can perform correct time and frequency pre-compensation. UL transmit timing is the most sensitive requirement to time varying Doppler and delay shifts. If a UE can pass the UL transmit timing test, we can safely assume the UE can correctly use ephemeris information to estimate satellite location and velocity.

- It is also noted that Demod session has down selected a very limited set of test cases where the new channel model will be applied. It is better to keep aligned principle across different sessions in RAN4.

- Applying the new channel model to measurement and mobility test cases require RAN4 to define initial ephemeris for a second satellite, which has not been done in the BDaT session.

Proposal 1: The TE-emulated channel model with varying Doppler and delay shifts is applied to UL transmit timing test cases only.

The RRM test case for UL timing is defined in clause A.14.3.1 for NR NTN. There are 2 subtests one with DRX and one without DRX. Both initial transmit timing error and the gradual timing adjustment are tested. It is noted that in NGSO test, the artificial DL timing change is not applied. The test requirement is tightened compared to the core requirements by deducing half of the margin for UE GNSS error. The test is conducted under AWGN channel.

In our view, it is straightforward to apply the TE-emulated channel model with varying Doppler and delay shifts to the test case in clause A.14.3.1. The initial ephemeris information and UE location have been agreed in [2], so we can already get the TE generated time varying Doppler and delay shifts with 1ms granularity.

Issue 1-1-9 Initial ephemeris information for LEO-600 satellite - Proposals: - Option 1： Use table 1 as the initial ephemeris information for LEO-600 satellite scenario Table 1 Initial ephemeris information for LEO-600 stating from elevation angles of 30 degrees. Parameters; Unit; Values. positionX-r17; km; [-3451.1273]. positionY-r17; km; [5703.5635]. positionZ-r17; km; [2071.0809]. velocityVX-r17; km/s; [0.87360]. velocityVY-r17; km/s; [-2.06700]. velocityVZ-r17; km/s; [7.21231]. - agreement - Option 1

UE location: Longitude: 121.56076999 Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites) Altitude: 0

The generated Doppler and delay shifts can be directly applied to the AWGN channel, so the only change we need for the test setup is the “Propagation condition” which should be changed from AWGN to AWGN with time varying Doppler and delay shifts.

| Propagation condition |  | 1,2 | AWGN |
| --- | --- | --- | --- |

For the test requirements, we think the definition of NTA,adjUE should be clarified.

2) After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±(Te_NTN –TGNSS_margin) of the first detected path of DL SSB. a. The NTA_offset value (in Tc units) is 25600 b. The NTA,adjcommon value is derived from the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation. c. The NTA,adjUE value is computed by the UE based on UE position and serving-satellite-ephemeris-related higher-layers parameters. d. The Te_NTN values depend on the DL and UL SCS for which the test is being run and are given in table 7.1C.2-1 e. The TGNSS_margin counts for the margin for the GNSS position definition error considered in the core requirement, which needs to be substracted for the test requirement, due to the usuage of AT commands in the test. TGNSS_margin=327,68×Tc

NTA,adjUE is used to determine the ideal UL timing, and it is calculated by the TE, i.e. TE would act as an ideal UE. For calculation of NTA,adjUE, the right figure in Issue 1-1-3 in [2] should be considered.

Issue 1-1-3 Time instance to calculate the propagation delay - Proposals - Option 1 - For downlink , calculate delay from the satellite at time t to the UE at time t+ΔT; - For uplink, calculate the delay from UE at time t-ΔT to the satellite at the time t, where ΔT is the signal transmission time - agreement - Option 1 as baseline assumption.

Although the RRM test case does not need modelling the UL channel from UE to TE, the calculation of NTA,adjUE by the TE should be based on the case where there is an UL transmission from UE to TE. If TE calculates NTA,adjUE with a different assumption, a correct UE would get penalized (smaller margin) for the test.

Another issue for the test requirements is whether additional margin is needed for the inaccuracy of the TE generated Test tolerance Doppler and delay shifts. In our understanding, such margin may be needed depending on the accuracy requirements for TE and how significant they are compared to core requirements, but in any case, this should be considered as test tolerance to be discussed in RAN5.

For the spec inclusion, we do not see the need to capture the procedures for generating the TE-emulated channel model with varying Doppler and delay shifts. Similar to HST channel, those procedures can be captured in 38101-5 and we can use a reference in 38133.

Above discussion are based on UL timing requirements and test cases for NR NTN, the proposals also apply IoT NTN as the requirements and test cases are similar between NR and IoT.

Proposal 2: Apply the following adaptation to the test cases for NR NTN and IoT NTN for applying the TE-emulated channel model with varying Doppler and delay shifts

- Change the “Propagation condition” from “AWGN” to “AWGN with time varying Doppler and delay shifts”

- Clarify that NTA,adjUE in test requirements is calculated based on the generated UL channel with time varying Doppler and delay shifts

- Use a reference to 38101-5 and 38102 for generation of the time varying Doppler and delay shifts

- Leave the discussion on test tolerance for inaccurate Doppler and delay shifts to RAN5

Conclusions

In this paper we provided our views on RRM testing for NGSO.

Proposal 1: The TE-emulated channel model with varying Doppler and delay shifts is applied to UL transmit timing test cases only.

Proposal 2: Apply the following adaptation to the test cases for NR NTN and IoT NTN for applying the TE-emulated channel model with varying Doppler and delay shifts

- Change the “Propagation condition” from “AWGN” to “AWGN with time varying Doppler and delay shifts”

- Clarify that NTA,adjUE in test requirements is calculated based on the generated UL channel with time varying Doppler and delay shifts

- Use a reference to 38101-5 and 38102 for generation of the time varying Doppler and delay shifts

- Leave the discussion on test tolerance for inaccurate Doppler and delay shifts to RAN5

Reference

- RP-241985, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

- R4-2508261, WF on RRM requirements for NR_IoT_NTN_req_test_enh, Moderator (Xiaomi)