3GPP TSG-RAN WG4 Meeting #116	R4-2510656

Bengaluru, India, 25 – 29 August, 2025

Title: 	Simulation assumption and additional simulation results for LP-WUR measurement

Source: 	Huawei, HiSilicon

Agenda item:	7.24.5.1

Document for:	Discussion

Introduction

RRM requirements related to LP-WUR are discussed in RAN4#115, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to evaluate the link level performance of LP-WUR measurement.

In this paper, we will provide our views on the remaining issues for simulation assumption and provide simulation results.

Discussion

Simulation assumptions and parameters

Issue 2-1-5: On ideal RSRP/RSRQ in simulation

In RAN4#115, it is confirmed to use method 1 in future simulation alignment. Method 1 is from RAN4#114 and is for defining ideal RSRP.

- Method 1: The first one is the RSRP is pass the fading channel and with the perfect channel estimation and the real RSRP pass the same channel.

For defining ideal RSRQ, similar approach should be used. The RSSI part should be based on the actual total received power at the specific time/frequency resources instead an assumed total received power.

Proposal 1: For defining ideal RSRQ, the ideal RSSI is the actual total received power at the specific time/frequency resources with channel fading.

Issue 2-1-7: On interference in simulation

Our simulation results are shown in Table 1 in section 2.2 for all cases supported by RAN1, we find that the performance for M=1, L=6 is particularly poor, especially in TDL-C channel. The specific of this case is that the binary sequence for the serving cell and interfering cell are opposite to each other. It can be seen that the target accuracy of 3.5dB cannot be reached even with 4 samples in TDL-C.

For M=1, L=6 (if supported), the set of LP-SS sequence is: Serving cell: [1 0 1 0 1 0] Interference cell: [0 1 0 1 0 1]

We further checked the reason for the poor performance. We used the basic RSRP calculation method in the simulation where RSRP is calculated by total power on the ON symbols minus that on the OFF symbols.

- For AWGN channel, the interference is always 6dB lower than the signal, so ideally the RSRP error would be 10*log(0.75)=-1.25dB, and we also find this is achievable when noise is small enough.

- For fading channel, when we are looking at 5% or 95%-tile of the error CDF, the interference is quite close to the signal due to the instantaneous channel fading of the service link and interference link. In this case, the RSRP error would be close to 10*log(0)=-inf.

In our view, there are 2 approaches for to address such cases with opposite sequence.

- Alt 1: define requirements for all combinations, and the requirements are based those with opposite sequence (worst case)

- Alt 2: define the requirements without considering combinations with opposite sequence, and the requirements are not applicable for those with opposite sequence

We suggest to adopt Alt 2 because the combinations with opposite sequence are in minority, but they will drag down the requirements significantly (longer delay plus degraded accuracy).

Proposal 2: LP-SS measurement requirements are not applicable to cases where serving cell and neighbour cell are using opposite binary sequences.

Issue 2-1-8: On LP-SS with overlaid OFDM sequences

In RAN4#115, some companies suggested to re-discuss the earlier agreement to define same requirements for OOK based LR with and without configured overlaid sequence. For example, it is observed that by using the orthogonal overlaid sequence between serving and interfering cells, the performance degradation as discussed in Issue 2-1-7 can be resolved.

Our understanding is that OOK based LR is not supposed to process overlaid sequence, i.e. only the power information of received samples are used to derive RSRP and RSRQ. In this sense, the performance with and without configured overlaid sequence should be very similar. There might be minor difference as the flatness of the received power over an OFDM symbol may be different with different overlaid sequences, but the impact should be rather small.

Proposal 3: RAN4 to confirm to define same requirement for OOK based LR with and without overlaid sequence configured.

Simulation results for LP-SS

The simulation results in this section are same as we presented in RAN4#115. The parameters selected for simulations are as follows.

- Serving cell SINR: -3dB

  - Serving cell SNR = -2.4dB, interfering cell SNR = -8.4dB, i.e. 6dB power difference

- Sample number: 1, 2, 3, 4

- Sampling rate: 7.68MHz

- ADC bit width: 8bit

- Filter: 3th Order Butterworth with 3.96MHz bandwidth

- Interference type: LP-SS, interfering cell is fully synchronous to serving cell, binary sequence for LP-SS is independent for serving cell and interfering cell

The results in this section are for OOK based LR, i.e. without utilizing the overlaid sequence on LP-SS. Our updated simulation results for LP-SS are given in Table 1.

Table 1: Simulation results for LP-SS

| {M,L} | LP-SS Binary Sequence | Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  |  |  | SINR(dB) | channel | Interference Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RSRP error |  |  |  | RSRQ error |  |  |  |  |  |  |
|  |  | X=1 | X=2 | X=3 | X=4 | X=1 | X=2 | X=3 | X=4 |  |  |  |
| {1,6} | Opposite sequence | 3.42 | 2.63 | 2.49 | 2.42 | 3.47 | 2.73 | 2.58 | 2.47 | -3 serving cell SNR = -2.4dB neighbor cell SNR = -8.4dB | AWGN | LP-SS |
| {1,8} |  | 2.93 | 2.34 | 2.29 | 2.28 | 2.66 | 2.06 | 1.98 | 1.98 |  |  |  |
| {2,12} |  | 2.76 | 2.09 | 2.03 | 2.00 | 2.43 | 1.78 | 1.72 | 1.67 |  |  |  |
| {2,16} |  | 2.35 | 1.68 | 1.63 | 1.60 | 2.19 | 1.46 | 1.42 | 1.35 |  |  |  |
| {4,16} |  | 3.10 | 2.34 | 2.26 | 2.24 | 2.64 | 1.98 | 1.88 | 1.84 |  |  |  |
| {4,32} |  | 2.33 | 1.69 | 1.66 | 1.63 | 2.24 | 1.50 | 1.45 | 1.41 |  |  |  |
| {1,6} | Opposite sequence | 30.39 | 6.31 | 4.68 | 4.07 | 30.65 | 6.46 | 5.25 | 4.22 |  | TDL-C |  |
| {1,8} |  | 4.21 | 2.57 | 2.35 | 2.17 | 3.93 | 2.33 | 2.22 | 1.99 |  |  |  |
| {2,12} |  | 3.94 | 2.23 | 1.95 | 1.94 | 3.89 | 2.10 | 1.88 | 1.74 |  |  |  |
| {2,16} |  | 3.56 | 1.95 | 1.91 | 1.72 | 3.52 | 1.91 | 1.83 | 1.69 |  |  |  |
| {4,16} |  | 4.06 | 2.53 | 2.23 | 2.10 | 3.85 | 2.28 | 2.05 | 1.87 |  |  |  |
| {4,32} |  | 3.63 | 2.11 | 1.95 | 1.73 | 3.63 | 2.09 | 1.96 | 1.81 |  |  |  |

It can be seen that without considering the cases with opposite sequence for serving and interfering cell as discussed in section 2.1, the target accuracy of 3.5dB can be met for all {M,L} combinations with 2 samples, for both AWGN and TDL-C channels. We suggest to take number of samples for measurement X as [2].

Proposal 4: RAN4 to take samples for LP-SS measurement X as [2].

  - Simulation results for SSB

Our simulation results for SSB measurement are given in Table 2.

Table 2: Simulation results for SSB

| Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  |  |  | SINR(dB) | channel | Interference Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSRP error |  |  |  | RSRQ error |  |  |  |  |  |  |
| X=1 | X=2 | X=3 | X=4 | X=1 | X=2 | X=3 | X=4 |  |  |  |
| 3.28 | 2.29 | 2.04 | 1.95 | 2.88 | 2.40 | 2.16 | 2.05 | -3 serving cell SNR = -2.4dB neighbor cell SNR = -8.4dB | AWGN | SSB |
| 4.18 | 2.75 | 2.55 | 2.55 | 3.31 | 2.84 | 2.70 | 2.67 |  | TDL-C |  |

It can be seen that the target accuracy of 3.5dB can be met with 2 samples, for both AWGN and TDL-C channels. We suggest to take number of samples for measurement Y as [2].

Proposal 5: RAN4 to take samples for LP-SS measurement Y as [2].

Conclusions

In this paper we provided our views on the remaining issues for simulation assumption and provide simulation results.

Proposal 1: For defining ideal RSRQ, the ideal RSSI is the actual total received power at the specific time/frequency resources with channel fading.

Proposal 2: LP-SS measurement requirements are not applicable to cases where serving cell and neighbour cell are using opposite binary sequences.

Proposal 3: RAN4 to confirm to define same requirement for OOK based LR with and without overlaid sequence configured.

Proposal 4: RAN4 to take samples for LP-SS measurement X as [2].

Proposal 5: RAN4 to take samples for LP-SS measurement Y as [2].

Reference

- R4-2508280, WF on RRM requirements for NR_LPWUS, Moderator (vivo)