3GPP TSG-RAN WG4 Meeting #116	R4-2510657

Bengaluru, India, 25 – 29 August, 2025

Title: 	Discussion on RRM requirements for LP-WUR

Source: 	Huawei, HiSilicon

Agenda item:	7.24.5.2

Document for:	Discussion

Introduction

RRM requirements related to LP-WUR are discussed in RAN4#115, and the outcomes are captured in WF [1]. Based on [1] the following issues need further discussion.

- General aspects

- LP-WUR measurement

- MR measurement

- LP-WUR at CONNECTED mode

- Other

In this paper, we will provide our views on RRM requirements related to LP-WUR.

Discussion

General aspects

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

In RAN4#114 it is agreed that at legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation, but it is FFS whether to define requirements for LR measurement.

We do not support to define LR measurement requirements at legacy state. We understand this is a transient period where LR measurement is needed only for evaluation of the entry condition, and this is quite different from Case 1 or Case 3 where LR measurement is for serving cell evaluation that is important for mobility. In other words, we do not see much significance to define the requirements. Functional wise, UE must evaluate the LR based criterion if configured by NW for the entry condition. This evaluation is not time critical, and it can be up to UE to decide how fast it would like to complete the evaluation as it only impacts when UE can get the power saving gain.

In addition, at legacy state MR measurement is ongoing without relaxation, and it may be difficult to define LR measurement requirements considering UE is not assumed to simultaneously operate LR and MR.

Finally, it would be difficult to verify this requirement via test case since when and how to turn on LR for serving cell measurement is up to UE implementation.

Proposal 1: RAN4 not to define LR measurement requirements before entering or after exiting LP-WUS monitoring or LR measurement offloading.

Issue 1-1-15 RedCap related

In RAN4#114-bis some companies proposed to define LP-WUR related requirements for RedCap UE. We support this proposal, as RedCap UE is also an important use case for LP-WUR. RedCap UE is already considered during SI and in RAN1 agreement for WI, e.g. the following agreement from RAN1#120-bis.

Agreement For RRC idle/inactive, support Alt 1 for both Redcap and non-redcap UEs: - Alt 2: LP-WUS/LP-SS frequency resource can be out of initial DL BWP where UE receives paging but within the same carrier. - FFS whether to define the maximum frequency gap between LP-WUS/LP-SS and SSB

As RedCap and non-RedCap UE differ in MR capability, the LR requirements should be same for RedCap and non-RedCap UE. The MR requirements should be based on R17 RedCap requirements, and be adapted for LP-WUR. We understand the conclusions so far, e.g. the relaxation factor and the evaluation of entry condition, are directly applicable for RedCap UEs.

Proposal 2: RAN4 to define LP-WUR related requirements for RedCap UE.

LP-WUR measurement

Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements

In RAN4#114-bis, the target accuracy for LR measurement is agreed with the RF margin as [2 or 2.5] dB. We suggest to take 2.5 dB because it is the assumption for MR. For example, in [2] it is clearly captured that 2.5 dB is the RF margin to define L1-RSRP accuracy in FR1.

Proposal 3: Take [2.5]dB as the RF impairment margin for LR.

Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity

In RAN4#113, it was agreed that the periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity, no matter if LP-SS is configured or not. One remaining issue is whether an upper bound is needed for the measurement periodicity. It is essentially a trade-off between power saving gain and how fast UE can exit LR when the condition for LR is not met.

Assuming the measurement period for LR measurement is n samples, and m measurements are needed for determining the exit condition is met (m=1 or m>1 is discussed in Issue 1-2-3), the total time between the time point when the channel condition gets insufficient for LR to the time point when UE determines to exit LR would be

Texit = n * Tsample + (m-1) * Tsample

where Tsample is the measurement periodicity. Assuming no upper limit is defined, UE may miss (n+m-1) LP-WUS since UE may not be able to decode LP-WUS under the channel condition. We understand n and m will be in the range of {1,2,3,4}, and it should not be big issue to user experience considering the probability of actual paging is low. Therefore, we suggest not to define upper limit for the measurement periodicity.

Proposal 4: Measurement periodicity for SSB is the LO periodicity, i.e. no upper limit is defined.

Issue 1-2-4-0: On LR measurement requirement

In RAN4#114 following is agreed regarding the LR evaluation requirements.

Issue 1-2-4-2-3: On how to define LR evaluation requirements Agreement: LR evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 > x and y1>y. Number x and y are defined as measurement requirement as [x samples]*LP-SS (for OOK LR) or [y samples] *LO (for SSB LR)

The common understanding behind the agreement was to define LR requirements similar to existing MR requirements for neighbour cell. It is noted that in existing requirements for MR neighbour cell (as copied below), both Tevaluate and Tmeasure are defined, so we support to define Tmeasure.

The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra (see table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5) for intra-frequency cells that are identified and measured according to the measurement rules. The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2.

On the two options to define LR measurement requirements, we support option 1, i.e. define the minimum interval where one LR measurement shall be executed. This is same as existing requirements, and it defines how frequent LR should generate one measurement for the serving cell. It would ensure that UE would not evaluate LR threshold based on a single measurement which is unreliable due to channel fading.

As to option 2, i.e. define the minimum duration to meet accuracy requirement, we do not see much value because it effectively defines with how many samples UE must be able to get an accurate measurement result. It is a consideration factor in defining Tevaluate, but does not need to be defined explicitly. It is noted that it is not defined in existing requirements for MR either. It is clear that UE is not expected to meet the accuracy (defined as reselection margin) with single measurement occasion even Tmeasure is 1 DRX cycle, and exactly how many samples are needed to meet the accuracy is not specified in existing requirements.

Proposal 5: Define LR measurement requirement as the minimum duration where one LR measurement shall be executed.

Issue 1-2-4-2-1: On applicability LR evaluation requirements

In our view, this issue is related to Issue 1-1-9, and it is enough to clarify LR requirements are applicable when UE is in RRM relaxation mode or RRM offloading mode. It may be unnecessary and inaccurate to discuss whether LR requirements apply or not for each entry and exit condition of each mode. As agreed in earlier RAN4 meeting, the LR requirements are generic for all transition cases.

Proposal 6: RAN4 to confirm LR requirements are applicable when UE is in RRM relaxation mode or RRM offloading mode.

Issue 1-2-4-2-3: On how to define LR evaluation requirements

In RAN4#114 the framework of the LR evaluation requirements has been agreed, and the remaining issue is to decide on the exact number of x1 and y1 based on x and y which are determined via simulation.

According to our simulation results, x = y = 2 should be sufficient to meet the target BB accuracy of 3.5dB for both LP-SS (including all {M,L} combinations) and SSB, and we suggest to define x1 = y1 = 4 which allows UE to use multiple measurements for the evaluation.

Proposal 7: Define LR evaluation requirement based on x1 = y1 = [4].

MR measurement

Issue 1-2-5-1: On applicability MR evaluation requirements

Similar to Issue 1-2-4-2-1, our view is that it is enough to clarify MR evaluation requirements are applicable only when UE is in RRM relaxation mode or legacy mode.

Proposal 8: RAN4 to confirm MR evaluation requirements are applicable when UE is in RRM relaxation mode or legacy mode.

Issue 1-2-6-3: On RAN4 related requirements and other issues

RAN4 has discussed the MR wake-up delay based on RAN1 LS [3]. It is noted that the wake-up delay in [3] is the “minimum gap time between LP-WUS reception and MR to start PDCCH monitoring”. It is for the case where MR wake-up is triggered by UE detecting LP-WUS.

On the other hand, MR wake-up can also be triggered by UE determining LR exit condition is met.

- When UE determines exit condition for LP-WUS monitoring is met, UE should wake up MR for PO monitoring

- When UE determines exit condition for RRM relaxation or offloading is met, UE should wake up MR for measurement

In our view, the wake-up delay for the case where UE determines LR exit condition is met should also be defined. It is important to make sure UE will timely wake up MR when the LR exit condition is met. Without this delay requirement, it is not possible to test the LR evaluation requirements. On the exact value, it is straightforward to re-use the same for the case where UE detects LP-WUS.

Proposal 9: RAN4 to define requirements on MR wake-up delay for cases where UE determines LR exit condition is met. The wake-up delay for cases where UE detects LP-WUS is re-used.

Issue 1-2-8-1: Accuracy for normal or relaxed MR serving cell measurement

Issue 1-2-8-2: Accuracy for relaxed MR neighbour cell measurement

We support to re-use the legacy MR accuracy requirements for MR measurement with relaxation, for both serving and neighbour cell. Accuracy requirements do not depend on the measurement interval, i.e. same accuracy requirements apply regardless of whether the measurement is performed once per DRX cycle or per N DRX cycles. Here, by legacy MR accuracy requirements we mean the accuracy in clause 10 for CA/DC Idle Mode Measurements. We are also open to discuss other existing accuracy numbers in RRC_IDLE or RRC_INACTIVE.

Proposal 10: Legacy MR accuracy requirements in RRC_IDLE or RRC_INACTIVE are re-used for MR measurement with relaxation, for both serving and neighbour cell.

LP-WUR at CONNECTED mode

Issue 1-4-1: LP-WUR at CONNECTED mode

In RAN4#115, some companies proposed to define interruption requirements for LP-WUS monitoring in CONNECTED mode. There was no conclusion due to ongoing discussion in RAN1.

In RAN1#121, following agreements are reached.

Agreement As the initial reply to RAN2 LS in R1-2503616, RAN1 confirms that at least the collision with Active Time, measurement gap, and RAR window monitoring for BFR can be considered for the cases/scenarios on when the UE is not able to monitor LP-WUS. Agreement As the reply to RAN2 LS in R1-2503616, RAN1 assumes that UE is not able to operate LR and MR simultaneously in Rel-19. RAN1 understanding is that the terminology of LR and MR operations are for discussion purpose and will not be specified. - LR operation is the UE operation for LP-WUS monitoring. - MR operation is the UE operation for all other NR signals/channels transmissions/receptions in connected mode. Agreement Nominal MO duration (X1, in unit of OFDM symbols) and actual LP-WUS duration (X2, in unit of OFDM symbols) are configured for LP-WUS in connected mode. (Alt C) - A LP-WUS MO spans the nominal MO duration (i.e., the LP-WUS MO duration is the same as the nominal MO duration). - If the number of available OFDM symbols within the nominal MO duration is no less than the actual LP-WUS duration, UE monitors LP-WUS on the first X2 available symbols within the LP-WUS MO. - Otherwise, UE does not monitor LP-WUS in this MO (i.e., the MO is dropped). Agreement For the handling of at least PCell/PSCell SSB for connected mode, the SSB symbols are considered as unavailable for LP-WUS.

We have several observations from the RAN1 agreements:

- Simultaneous LR and MR operations are not supported.

- MR operation for Active Time, measurement gap, and RAR window monitoring for BFR are prioritized over LR operation. There is however no agreement on prioritization related to other MR operations.

- RAN1 defines MO dropping (when an MO is dropped, UE does not monitor LP-WUS in this MO) based on availability of OFDM symbols for LP-WUS.

- PCell/PSCell SSB are considered as unavailable for LP-WUS.

Based on above observations, RAN4 should wait for RAN1 further agreements before deciding whether there is impacts to RRM requirements due to LP-WUS monitoring in CONNECTED mode.

- We do not think interruption to LP-WUS monitoring needs to be defined in RAN4 because it will be captured in RAN1 spec in some form.

- However, there might be impacts to some MR measurement requirements, if RAN1 agrees LP-WUS monitoring is prioritized over those MR measurements.

Proposal 11: RAN4 to wait for RAN1 further agreements to discuss possible RRM impacts of LP-WUS monitoring in CONNECTED mode.

Other

Issue 1-5-1: LR based RRM with EMR in IDLE/inactive mode

In RAN4#115, some companies proposed that when T331 for EMR is running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading or relaxation condition is met or not.

In our view, the proposals are in general reasonable. EMR is for possible CA/DC setup, i.e. for better data experience. It should be of higher priority than UE power saving, especially considering the impact is limited to when T331 is running (max 5min). It is also be noted that in R16 power saving WI, EMR requirements are not relaxed even relaxation conditions are met.

As to the exact impact to the LP-WUR, our view is that RAN4 can clarify that UE should meet the existing EMR measurement requirements when in RRM relaxation or offloading mode when TR331 is running. We do prefer to impact the UE behaviour related to entry of RRM relaxation or offloading mode.

Proposal 12: Clarify that UE should meet the existing EMR measurement requirements when in RRM relaxation or offloading mode when TR331 is running.

Issue 1-5-2: eDRX related

In RAN4#115, some companies discussed possible requirements for LR in eDRX. Although RAN2 has confirmed that LP-WUS is supported with eDRX, we do not support to define requirements for this case. In our view, it is not a strong use case. When UE is in eDRX, the power saving gain from using LR is marginal because the baseline power consumption is already quite low. On the other hand, defining the requirements for eDRX is effort consuming and will complicate the requirements. As this is the last meeting of the WI, we suggest RAN4 to focus on the essential use case.

Proposal 13: RAN4 to de-prioritize defining LR related requirements with eDRX.

Conclusions

In this paper we provided our views on RRM requirements related to LP-WUR.

Proposal 1: RAN4 not to define LR measurement requirements before entering or after exiting LP-WUS monitoring or LR measurement offloading.

Proposal 2: RAN4 to define LP-WUR related requirements for RedCap UE.

Proposal 3: Take [2.5]dB as the RF impairment margin for LR.

Proposal 4: Measurement periodicity for SSB is the LO periodicity, i.e. no upper limit is defined.

Proposal 5: Define LR measurement requirement as the minimum duration where one LR measurement shall be executed.

Proposal 6: RAN4 to confirm LR requirements are applicable when UE is in RRM relaxation mode or RRM offloading mode.

Proposal 7: Define LR evaluation requirement based on x1 = y1 = [4].

Proposal 8: RAN4 to confirm MR evaluation requirements are applicable when UE is in RRM relaxation mode or legacy mode.

Proposal 9: RAN4 to define requirements on MR wake-up delay for cases where UE determines LR exit condition is met. The wake-up delay for cases where UE detects LP-WUS is re-used.

Proposal 10: Legacy MR accuracy requirements in RRC_IDLE or RRC_INACTIVE are re-used for MR measurement with relaxation, for both serving and neighbour cell.

Proposal 11: RAN4 to wait for RAN1 further agreements to discuss possible RRM impacts of LP-WUS monitoring in CONNECTED mode.

Proposal 12: Clarify that UE should meet the existing EMR measurement requirements when in RRM relaxation or offloading mode when TR331 is running.

Proposal 13: RAN4 to de-prioritize defining LR related requirements with eDRX.

Reference

- R4-2508280, WF on RRM requirements for NR_LPWUS, Moderator (vivo)

- R4-1904820, Way forward on L1-RSRP accuracy requirements, Ericsson, Nokia, CMCC

- R1-2501624, LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode, RAN1