3GPP TSG-RAN WG4 Meeting #116	R4-2510660

Bengaluru, India, 25 – 29 August, 2025

Title: 	Discussion on RRM requirements for RedCap UE in NTN

Source: 	Huawei, HiSilicon

Agenda item:	7.27.5.1

Document for:	Discussion

Introduction

RRM requirements for RedCap UE in NTN are discussed in RAN4#115, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to further discuss the following issues.

- RRM requirements

- Impact of HD-FDD

- Other

In this paper, we will provide our views on RRM requirements for RedCap UE in NTN.

Discussion

RRM requirements

Issue 1-2-2: SMTC adjustment in idle/inactive <Way Forward>: - Proposal 1: SMTC adjustment in idle/inactive is kept mandatory for (e)RedCap UEs with NTN. - Proposal 2: RAN4 will not make any modifications on the feature of SMTC adjustment in idle/inactive for RedCap UE with NTN. - Proposal 2a: Maintain the same principle used in NR NTN for RedCap devices: the requirements are transparent to whether the UE adjusts or not the SMTC window in IDLE/INACTIVE mode. - Proposal 3: For SMTC adjustment in idle/inactive, we recommend keeping it optional, yet we would support defining it as mandatory if companies have strong concern on this.

In May meeting, RAN2 confirmed that SMTC adjustment in idle/inactive is an optional feature for normal UE. On the other hand, regardless of whether the feature is optional or mandatory, we do not see clear reason to change the optional/mandatory for RedCap UE compared to normal UE.

Proposal 1: RAN4 not to make any modifications on the feature of SMTC adjustment in idle/inactive for RedCap UE with NTN.

Impact of HD-FDD

Issue 1-3-5: The impact on NR Handover due to HD-FDD <Way Forward>: - Proposal 1: For the impact on NR handover for HD-FDD, the principle in option 1 is accepted. - Proposal 1a: Regarding Issue 3-2-1 (The impact on NR Handover due to HD-FDD), adopt Option 1 with 'availability' defined as 'no collision with UL transmission at the UE'. - Proposal 2: Adopt option 2 (no spec impact) for HO, CHO and satellite switch for HD-FDD UE.

We support option 2. Technically, SSB is always prioritized over UL following RAN1 agreement (Case 5), and during those mobility procedures, we do not see any reason for UE to prioritize UL over SSB. If we define those applicability rules, it may be mis-interpreted as SSB may be dropped due to collision with UL, which is not the intention. The applicability of those requirements should be same as full duplex UE, i.e. no additional condition is needed.

Conclusion For collision cases 1, 2, 5 and 6, the existing priority rules can be reused for a HD-FDD (e)RedCap UE in NTN.

Proposal 2: Adopt option 2 (no spec impact) for HO, CHO and satellite switch for HD-FDD UE.

Other

Issue 1-4-1: General principle for designing the UE capabilities for (e)RedCap UE with NTN <Way Forward>: - Option 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). (CATT, CMCC, ZTE, Xiaomi, E///, Samsung - compromise, Apple, QC) - FFS: The Rel-19 requirement can be release independent from Rel-17. - Option 2: RAN4 to introduce new capability to declare support over RedCap over NTN. (vivo, Samsung) - Option 3: to wait for RAN1 to confirm the working assumption (UE that supports RedCap and NTN shall support RedCap over NTN). (Nokia, QC, MTK)

We support option 1. The problem with option 2 is that it would lead to 2 types of UEs,

- UE type 1: UEs supporting RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17), but not supporting the new capability

- UE type 2: UEs supporting RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17), and in addition supporting the new capability

In general, it would be preferable to avoid many UE types from capability perspective, especially when the difference is only about requirement applicability. In last meeting, some companies mentioned that R19 UE supporting RedCap + NTN needs to additionally support new HD-FDD collision handling defined by RAN1. This is true, but RAN1 has already agreed to introduce an optional feature (FG 65-2-1) for this functionality.

In last meeting, some companies also mentioned that option 1 would not allow UE “to operate as a RedCap-TN UE or an NTN non-RedCap UE”. In our view, a RedCap-NTN can still operate as RedCap-TN UE if it supports TN bands. It is true that following option 1, a RedCap-TN UE cannot operate as NTN non-RedCap UE, but it would be very unlikely that a UE is a RedCap UE for TN but becomes a normal UE in NTN.

Proposal 3: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap and support of NTN.

Conclusions

In this paper we provided our views on RRM requirements for RedCap UE in NTN.

Proposal 1: RAN4 not to make any modifications on the feature of SMTC adjustment in idle/inactive for RedCap UE with NTN.

Proposal 2: Adopt option 2 (no spec impact) for HO, CHO and satellite switch for HD-FDD UE.

Proposal 3: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap and support of NTN.

Reference

- R4-2508287, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)