3GPP TSG-RAN WG4 Meeting #116	R4-2510682

Bangalore, India, 25 Aug - 29 Aug, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM aspects of UE-initiated beam management of R19 NR MIMO Phase 5

Agenda Item:	7.20.4.1

Document for:	Discussion

- Introduction

Around the objective of UE-initiated/event-driven beam management, during last RAN4 meeting, some progress were achieved in last meeting and summarized in [1]. In this paper, we provide our further analysis around the suspending issues of UE-initiated/event-driven beam management.

- Discussion

Ever since Rel-15, NR has supported intra-cell beam management. During Rel-17, inter-cell beam management(ICBM) was studied and specified. So actually until now, both intra-cell and inter-cell beam management are allowed. While all the legacy intra-cell and inter-cell beam management are NW-initiated. In terms of the beam management resource configuration, periodic, semi-persistent and aperiodic measurement resource are candidates.With respect to the reporting, similarly, periodic, semi-persistent and aperiodic reporting are allowed with some principle defined between the mapping of resource and reporting. All types of reporting are NW-initiated, in other words, only NW could trigger the beam measurement results reporting. Consequently, even the UE detects no need to update the current beam based on the measurement results, once the reporting is configured or activated or indicated by NW, the UE has to report the quantities configured in the corresponding report configuration.

Actually the factors which would lead to beam switching including UE movement and rotation, all these factors are not controlled by NW, so only UE is aware of these. So no matter for the sake of beam management latency reduction or to save the reporting overhead, it seems the UE-initiated beam management is more efficient and effective.

The components of Event triggered measurement reporting delay

Regarding the measurement reporting delay, it has confirmed that the component of UE waiting for the 1st PUCCH is counted on. While regarding the component of L1 measurement period, how to capture this component, it has been approved as below in last meeting.

Agreement: - If the measurement periods are different for current beam and new beam, the TL1_measurement period is not larger than the maximum of the L1 measurement periods to measure both current beam and new beams. - Note: The agreement does not imply that all measurements on individual reference signals are after the event is met over air. - UE shall perform L1-RSRP measurement based on current beam and new beams L1-RSRP measurement period by assuming TReport =0.

For Event 2, RAN1 has defined two types of trigger-event detection for reporting as below in [2][3][4]:

- Case 1: Basic solution(mandatory), once the L1-RSRP of the new beam becomes a threshold value better than the current beam(i.e. Event-2), UE initiated beam report occurs.

- Case 2: Optional solution(subject to UE capability), f within a time window, the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

- For the basic solution, which is one-shot measurement based reporting, while for the optional solution, the sample number M is configured by NW signalling, and a timer X should also be considered. Other scaling factor such as collusion with other measurement or Rx beam sweeping in FR2 should also be considered.

Proposal 1: For Event-2, both the basic Case 1 and the optional Case 2 should be considered when defining the evaluation period. For the optional Case 2, the sample number M and timer X should be considered.

The requirements of TCI state switching

RAN1 held some discussion on the reduction of TCI state switching latency after UEIBR in RAN1#119 meeting and the following progress were achieved:

Even no final conclusion between the 3 alternatives, it can be seen that the common point of all alternatives lies in: activate new beam(s) without additional SSB reception.

From RAN4 perspective, in last meeting, it is still suspending whether consider additional enhancements on top of existing requirements for TCI state switching delay and companies are divided into two camps as below:

- Option 1: - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management if no further RAN1 agreement on reduce beam application latency - Option 2: - UE is assumed to store the QCL properties of the SSB associated with new beam, the legacy TCI state switching components of TL1-RSRP and Tfirst-SSB+TSSB-proc are ignored.

To our understanding, after UE reports the target beam, actually UE would maintain the beam info of the target beam since it is probably indicated by NW through TCI state switching command in a short while. From NW side, after receiving the report and the target beam is involved in, as long as NW expects good beam performance, the TCI state switching decision is expected sent by NW and it would be indicated in time in case the target beam is not as good as reported. So when UE receive the TCI state switching command and the new beam is indicated by and command, since UE has already maintain the new beam, the components of TL1-RSRP and Tfirst-SSB+TSSB-proc are ignored.

Proposal 2: UE is assumed to store the QCL properties of the SSB associated with new beam in the latest report. When UE receives the TCI state switching command and the new beam is indicated by the command, the legacy TCI state switching components of TL1-RSRP and Tfirst-SSB+TSSB-proc are ignored.

Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time

This issue has been raised by companies for several meetings, the following options were supported by different companies:

- Option 1: - No need to capture this scenario in RAN4 requirements - Option 2: - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart. - Option 3: - Add the applicability rule of event triggered L1 measurement: - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE. - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

To our understanding, at least for Event-1 and Event-2, the measurement result of current beam is involved in the evaluation. If the current beam is changed by the TCI state switching command during the L1 measurement time, which implies the reference beam is changed, to guarantee the evaluation consistent with the new current beam, it is better to restart the evaluation and take the updated beam as the new current beam.

Actually this issue is in common case provided the periodic RS resource is configured, UE is always performing the periodic measurements on the configured RS resources, so actually for each TCI state indication, UE is in the L1 measurement time. We approved the note for the L1 measurement period as below, which demonstrates this.

Issue 1-2: Components of Event triggered measurement reporting delay? Agreement: - If the measurement periods are different for current beam and new beam, the TL1_measurement period is not larger than the maximum of the L1 measurement periods to measure both current beam and new beams. - Note: The agreement does not imply that all measurements on individual reference signals are after the event is met over air. - UE shall perform L1-RSRP measurement based on current beam and new beams L1-RSRP measurement period by assuming TReport =0.

Technically we prefer Option 2, and it could be captured into the evaluation requirement part. Furthermore, the relevant issue was also discussed in RAN1 and the consensus have already been reached until RAN1#120bis meeting as below:

Agreement(RAN1#120bis) Regarding triggering event determination for Event 2, on resetting the counting, the following modification on the agreed Candidate #2 in RAN1#120 is supported. - Candidate#2: [The measured current beam RS is updated based on] indicated TCI state is updated; - In such case, the UE needs to reset the counting for all new beams. - FFS: Further details on the update (if necessary)

Proposal 3: Confirm the case that the indicated TCI state is changed during the L1 measurement time. The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

- Conclusion

In this contribution, we have the following observations and proposals:

Proposal 1: For Event-2, both the basic Case 1 and the optional Case 2 should be considered when defining the evaluation period. For the optional Case 2, the sample number M and timer X should be considered.

Proposal 2: UE is assumed to store the QCL properties of the SSB associated with new beam in the latest report. When UE receives the TCI state switching command and the new beam is indicated by the command, the legacy TCI state switching components of TL1-RSRP and Tfirst-SSB+TSSB-proc are ignored.

Proposal 3: Confirm the case that the indicated TCI state is changed during the L1 measurement time. The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

- Reference

- R4-2508271, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung, RAN4#115

- Final_Minutes_report_RAN1#116_v100

- Final_Minutes_report_RAN1#116b_v100

- Final_Minutes_report_RAN1#117_v020