3GPP TSG-RAN WG4 Meeting #116	R4-2510692

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Discussion on other RRM requirements for MIMO ph5

Agenda item:	7.20.4.2

Document for:	Discussion

Introduction

In this document, we present Nokia’s views regarding the core requirements that RAN4 needs to discuss in relation to the WID RP-242394, in particular about asymmetric DL sTRP/UL mTRP deployment scenarios (Objective 5) and the aspects related to CJT calibration reporting (Objective 3).

Discussion on two TAs

It was agreed in RAN4#115 meeting to further discuss the following

Issue 2-1: For Scenario#1, whether to update RACH requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Way forward: FFS Issue 2-2: For Scenario#1, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Way forward: - Option 1: Gradual timing adjustment applies for each TAG and shall be met for each TAG. - Option 2: The gradual timing adjustment requirements are only applicable to TRP with both UL and DL. Issue 2-3: For Scenario#1, how to define MRTD and MTTD requirements? Way forward: FFS

As per the RAN1 LS reply is as follows:

Question: Based on RAN1 LS (R1-2410870), RAN4 discussed timing requirement for two TA enhancement for asymmetric DL sTRP/ UL mTRP. RAN4 would like to check with RAN1 whether RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s). Answer: The RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are NOT applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s). And RAN1 made the following new agreement for further clarification: Agreement: - For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing - For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs: - The UE maintains two downlink reference timings; - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length. - The reference point for PRACH transmission is indicated as follows: - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG - Above applies for the case UE is configured with SSB-MTC-additionalPCI.

PRACH timing

Regarding Issue 2-1:  For Scenario#1, whether other updates are needed for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?, in an asymmetric DL sTRP / UL mTRP scenario the issue has recently been observed in R4-2503935 in RAN4 #114 bis. With an UL-only TRP that does not transmit RSs such as SSBs (Scenario#1), the UE uses a single DL reference timing for both the DL/UL capable TRP (TAG1) and the UL-only TRP (TAG2). Because of that, the UE transmission timing errors toward the UL-only TRP tends to be larger when compared to a scenario, like in Rel-18, where the UE is required to monitor a DL reference timing per TAG. This timing error is then compensated via Timing Advance Commands (TACs) by the network. On the other hand, when the UE transmits a RACH preamble, this transmission is not time advanced to align with the uplink reception time at the gNB receiver, as TACs does not apply to PRACH.

When the UE transmits a RACH preamble, this transmission is not time advanced to align with the uplink reception time at the gNB receiver, as TACs does not apply to PRACH.

For example, during transmission of a preamble as part of the PRACH procedure, as specified in TS 38.133, the reference point for the UE initial transmit timing control requirement is the downlink timing of the reference cell. So, when PRACH is triggered toward an UL-only TRP that does not transmit RSs such as SSBs, as the UE doesn’t have the downlink reference timing from the UL-only TRP, the UE transmits a RACH preamble toward the UL-only TRP based on:

  - the DL reference timing from the DL/UL capable TRP

  - a timing advance offset n-TimingAdvanceOffset/n-TimingAdvanceOffset2 optionally configured by the network.

In this “asymmetric DL sTRP / UL mTRP scenario”, exactly because the UE may not be able to have a DL reference timing based on a RS sent by the UL-only TRP, the error on the transmission timing of the PRACH preamble may be too large, causing a) interference and b) the network not being able to detect such RACH preamble. Because of the guard period (GP) at the end of the RACH preamble (in case of most preamble formats), a late arrival seems to be not such a big issue, as the GP after the preamble sequence ensures that the preambles when received by the gNB receiver do not cause interference with other uplink transmissions, but early arrival can be problematic. In R4-2503935, it has been proposed to have a default timing offset depending on the Time Alignment Error (TAE), more specifically 0.5*TAE ≈ 1.5 µs (TAE is defined in 38.104 such that maximum TAE shall not exceed 3 µs).  On the other hand, such solution is in our view not enough, because it tackles just the TAE, which relates to the imperfect time synchronization among the two TRPs, but it does not tackle the difference in propagation delays from the two TRPs. Note that such 1.5 µs means roughly a gap that would allow handling a propagation delay difference without any TAE of about 450 meters, which could be enough in FR2, but not really in FR1.

Similar to the Rel-18 enhancements, the transmission timing offset of PRACH can be controlled by the network with the higher layer parameters n-TimingAdvanceOffset (and n-TimingAdvanceOffset2). More specifically, the network can configure the UE with two different PRACH transmission timings for the two TAGs. On the other hand, the network control of such PRACH transmission timings is quite rough, as n-TimingAdvanceOffset (and n-TimingAdvanceOffset2) can take only limited values. In addition, there could also be a timing offset/delta associated with these parameters to improve the granularity of the configurable PRACH transmission timing and be better adjusted for different deployments.

For Scenario#1, for 2TA enhancement for asymmetric DL sTRP/ UL mTRP, there is uplink interference due to a misaligned RACH preamble transmission toward an UL-only TRP which does not transmit any RS.

For an asymmetric DL sTRP/UL mTRP deployment with two TAs, RAN4 to introduce a further timing offset/delta in addition to n-TimingAdvanceOffset/n-TimingAdvanceOffset2 when PRACH is transmitted by the UE.

MTTD requirements for Scenario#1

In RAN4#115 we have reached agreement regarding MRTD/MTTD for Scenario#2. It is still pending if the same can be reused for Scenario#1. The agreement for Scenario#2 is as follows:

Issue 2-6: For Scenario#2, if the UE is configured with SSB-MTC-additionalPCI, how to define MRTD and MTTD requirements? Agreement: - Reuse Rel 18 values of MRTD and MTTD for Rel 19.

For scenario #1, the biggest difference is that, since there is no DL in one of the TRPs, there is no meaning in defining MRTD. However, we have UL transmissions for both TRPs, and for this reason MTTD needs to be defined.

Reuse Rel 18 MTTD values for Scenario #1.

In Rel-18, MIMO_evo has defined a capability for operation with MRTD>CP. However, since this feature name is related to “receive” time difference, it would be unclear to reuse the feature as it is defined. Therefore, we propose that a new feature is defined for the operation with larger MTTD.

- Rel 18 MIMO_evo has introduced optional UE capability for operation with MRTD > CP.

- UE capability rxTimingDiff-r18 cannot be reused for Rel 19 UL only TRP Scenario #1.

Define a new optional UE capability to support a case scenario when Tx timing difference between two TAGs is larger than CP length.

  - Level 1:

    - MTTD < CP length + 1.6 us for FR1

    - MTTD < CP length + 0.5 us for FR2

  - Level 2:

    - MTTD < 34.6 us for FR1

    - MTTD < 8.5 us for FR2

Autonomous timing adjustments for Scenario#1

In RAN4#115 we have reached agreement regarding autonomous timing adjustments for Scenario#2 as follows:

Issue 2-5: For Scenario#2, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Agreement: - Autonomous timing adjustment applies to both TAGs or TRPs. - Two independent gradual timing adjustments with potentially different values should be applied to both TAGs.

Regarding Issue 2-2: For Scenario#1, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?, in Scenario#1, when a UE needs to perform autonomous timing adjustments, monitoring a single downlink reference signal from the DL/UL capable TRP and using that for autonomous timing adjustments for the UL-only TRP may result in increased timing errors towards the UL-only TRP, see also Figure 1.

On the other hand, if the UE does not perform any autonomous timing adjustment towards the UL-only TRP, then the network would be forced to send a greater number of timing advance commands to the UE resulting in larger signaling overheads.

While performing autonomous timing adjustments, the likelihood of timing errors is lower when the UE independently monitors and measures the DL RS from each TRP (Scenario#2) when compared to when the UE uses only one DL reference timing to perform autonomous timing adjustments to two TRPs (Scenario#1).

If in Scenario#1 the UE doesn’t do autonomous timing adjustments towards the UL-only TRP, then the network will need to send a relatively larger number of timing advance commands than it would when the UE performs autonomous timing adjustment.

(a)

(b)

Figure 1 Timing error in asymmetric DL sTRP/UL mTRP deployment with one DL reference timing monitored by the UE (Scenario#1): (a) UE autonomous timing adjustment for TRP1 is applicable also for TRP2, (b) UE autonomous timing adjustment for TRP1 is NOT applicable for also TRP2.

In general, it would be beneficial to define in Scenario#1 conditions to determine if UE autonomous timing adjustment applies to just TRP1 or to both TRP1 and TRP2. For example, that condition can be based on some UE measurements, although other options are not precluded. At least we can consider the case where low mobility is detected at the UE, for example reusing the “Relaxed measurement criterion for low mobility” already defined in clause 5.7.13.1 of TS 38.331. Using that criterion, the threshold SSearchDeltaP-Connected can be used to determine if the UE is satisfying the low mobility condition or not. If the mobility is high, i.e., the RSRP variation measured at the UE is large, autonomous timing adjustment towards the UL-only TRP is disabled, otherwise, if the mobility is low, i.e., the RSRP variation measured at the UE is small, autonomous timing adjustment towards the UL-only TRP is enabled.

With asymmetric DL sTRP/UL mTRP, in case the UE is configured with PL offset (Scenario#1), specify conditions under which UE autonomous timing adjustment applies to just the DL/UL TRP or also to the UL-only TRP.

With asymmetric DL sTRP/UL mTRP, in case the UE is configured with PL offset (Scenario#1), autonomous timing adjustment towards the UL-only TRP is disabled when RSRP variation measured at the UE is large.

Reporting for CJT calibration

Regarding asymmetric UE reporting enhancement for CJT calibration, the following was agreed and discussed in the last RAN4#115 meeting [1]:

Issue 1-1: How to define measurement period requirements for delay and frequency offset reporting for CJT calibration? Agreement: - RAN4 to discuss the achievable accuracy in terms of delay and frequency offsets based on 1 sample and [3 or 5] samples in the next meeting. - RAN4 to conclude the delay requirement for delay and frequency offset reporting in the next meeting. The delay requirement for phase offset is deprioritized in Rel-19. Issue 1-2: How to define scheduling restriction requirements for delay and frequency offset reporting for CJT calibration? Way forward: - Option 1: The scheduling restriction is specified as: - For the UE is configured with CJTC reporting, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for CQI on the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting, and on one data symbol before and one data symbol after the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting. - Option 2: No scheduling restrictions needed to be applied for CJT reporting. Issue 1-3: How to define measurement restriction requirements for delay and frequency offset reporting for CJT calibration? Way forward: - Option 1: Reuse the legacy measurement restriction in FR1 for delay and frequency offset measurement. - Measurement restriction does apply, - If UE does not support simultaneousRxDataSSB-DiffNumerology and - SSB for L1-RSRP with different SCS as PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI and they are fully or partially overlapped on symbols. - Measurement restriction does not apply, - If UE support simultaneousRxDataSSB-DiffNumerology or - SSB for L1-RSRP with the same SCS as CSI-RS/PDSCH/PDCCH. - Option 2: If CJTC measurement collides with other L1 measurement, follow legacy principle, UE is required to measure one of but not both of different measurements when measurement resources are at the same time.

Nokia’s viewpoint regarding the remaining open issues on the topic of UE reporting enhancement for CJT calibration and the simulation results as per the agreements are presented below.

Required accuracy for CJT calibration offsets reporting

In this section we present our views regarding CJT reporting requirements. To conduct this analysis, we start with link simulation results that show how time and frequency offset errors influence the link throughput. Parameters for these LLS simulations are slightly different from the assumptions for CJT calibration reporting simulations that we will show later and can be found in Appendix A.

Figure 2 shows that LLS throughput starts degrading due TO with different rate when in different SNR conditions:

- for SNR = -3dB from 0CP what is equivalent to 0 steps when M_D is equal to 256

- for SNR = 0dB from 0.1CP what is equivalent to 26 steps when M_D is equal to 256

- for SNR = 3dB from 0.2CP what is equivalent to 51 steps when M_D is equal to 256.

LLS spectral efficiency starts degrading significantly only when CFO is above 50 to 100Hz, what is equivalent to 18 to 37 steps when M_CFO is equal to 256. We also demonstrated that CJT performance is more sensitive to TO compared to CFO. Simulations show that CJT performance degradation due to CFO and TO depends on the SNR. .

LLS throughput starts degrading due TO with different rate when in different SNR conditions:
- for SNR = -3dB from 0CP what is equivalent to 0 steps when M_D is equal to 256
- for SNR = 0dB from 0.1CP what is equivalent to 26 steps when M_D is equal to 256
- for SNR = 3dB from 0.2CP what is equivalent to 51 steps when M_D is equal to 256.

LLS spectral efficiency starts degrading significantly only when CFO is above 50 to 100Hz, what is equivalent to 18 to 37 steps when M_CFO is equal to 256.

Figure 2: LLS CJT throughput degradation due to TO (left) and CFO (right) between 2 TRPs

Simulations show that CJT performance is more sensitive to TO compared to CFO.

Simulations show that CJT performance degradation due to CFO and TO depends on the SNR.

RAN4 to consider no averaging as long as CJTC-D error is below 0,1 CP and CJTC-F error is below 100 Hz.

In our previous contribution [5], we showed SLS results demonstrating that CJT providing better gains in cell edge than cell centre. Based on our new LLS results and the SLS observation from our previous contribution, it could be recommended to use SNR=-3 dB for defining the requirements which emulate the operating conditions for a UE at the cell edge.

Use SNR=-3 dB for defining the requirements.

Core requirements for CJT calibration offsets reporting

Issue 1-2: How to define RRM core requirements for delay and frequency offset reporting based on the simulation results for CJT calibration? Agreement: - Define core requirements for delay and frequency offset reporting for - Measurement period (FFS number of samples) - Scheduling restriction (if identified)

In a realistic scheduling scenario, the symbols not carrying TRS are typically also used for PDSCH/PDCCH, therefore it could be recommended to use data on all the OFDM symbols; except for the resource elements carrying TRS which matches with the general operational conditions of a UE for the test setup.

Additionally, Table 3 shows results comparing CFO accuracy with and without scheduling restrictions. From those results, a minor difference can be observed, only 2 quantization levels for 1 sample cases. Therefore we can conclude that scheduling restrictions are not necessary for CJT reporting.

No scheduling restrictions needed to be applied for CJT reporting.

Link level simulation results

Carrier frequency offset (CFO) requirements

Table 1. Simulation assumptions for the CFO reporting simulations.

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Carrier Frequency | 3.5GHz |
| SCS | 15kHz, 30kHz |
| Channel Model | TDLC300-10 |
| TRP number | 2 |
| UE antenna configuration | 1x2 |
| UE speed | 3km/h |
| CSI-RS Bandwidth | 48 RB |
| CSI-RS configuration | 4 TRSs per TRP |
| CSI-RS periodicity | 40, 20 slots |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRPs | 100ppb |
| AFO | 200ppb |

Simulation results for carrier frequency offset estimation are provided on Table 2.

The simulation results shown below are without considering scheduling restrictions. i.e. Data on all the OFDM symbols; except for the resource elements carrying TRS.

Table 2: Carrier frequency offset estimation error.

| SCS | SNR | Frequency offset (FO) b/w two TRPs | FO estimation error in AFO = 0.2 ppm and MFO = 256 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Max (CDF5%, CDF95%) # levels 256 |  |  |
|  |  |  | # samples |  |  |
|  |  |  | #1 | #3 | #5 |
| 15KHz | -3 | 350 Hz | 24 | 18 | 5 |
| 15KHz | 0 | 350 Hz | 16 | 4 | 4 |
| 15KHz | 3 | 350 Hz | 11 | 2 | 4 |
| 30KHz | -3 | 350 Hz | 42 | 18 | 19 |
| 30KHz | 0 | 350 Hz | 27 | 19 | 19 |
| 30KHz | 3 | 350 Hz | 17 | 2 | 5 |

Table 3: Carrier frequency offset estimation error comparison of performance with scheduling restrictions and without scheduling restrictions.

| FO error max(P5,P95) for SNR=0dB |  | scenario 1: with scheduling restrictions | scenario 3: without scheduling restrictions |
| --- | --- | --- | --- |
| 15 kHz SCS | 1 sample | 15 | 16 |
|  | 3 samples | 4 | 4 |
|  | 5 samples | 4 | 4 |
| 30 kHz SCS | 1 sample | 29 | 27 |
|  | 3 samples | 19 | 19 |
|  | 5 samples | 17 | 19 |

Time offset (TO) requirements

Table 4. Simulation assumptions for the TO reporting simulations.

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Carrier Frequency | 3.5GHz |
| SCS | 15kHz, 30kHz |
| Channel Model | TDLC300-10 |
| TRP number | 2 |
| UE antenna configuration | 1x2 |
| UE speed | 3km/h |
| CSI-RS Bandwidth | 48 RB |
| CSI-RS configuration | 4 TRSs per TRP |
| CSI-RS periodicity | 40 slots for 30kHz; 20 slots for 15kHz |
| CSI-RS offset | Slot 10 and 11 for 15kHz , slot 20 and 21 for 30KHz |
| SNR | -3dB, 0dB, 3dB |
| Time offset between TRPs | CP/2 |
| AD | CP |

Simulation results for time offset estimation are provided on Table 5.

The simulation results shown below are without considering scheduling restrictions. i.e. Data on all the OFDM symbols; except for the resource elements carrying TRS.

Table 5: Time offset estimation error.

| SCS | SNR | Time offset (TO) b/w two TRPs | TO estimation error in AD = CP and MD = 256 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Max (CDF5%, CDF95%) # levels 256 |  |  |
|  |  |  | # samples |  |  |
|  |  |  | #1 | #3 | #5 |
| 15KHz | -3 | 0.5CP | 8 | 2 | 1 |
| 15KHz | 0 | 0.5CP | 8 | 2 | 1 |
| 15KHz | 3 | 0.5CP | 5 | 2 | 1 |
| 30KHz | -3 | 0.5CP | 8 | 2 | 1 |
| 30KHz | 0 | 0.5CP | 8 | 2 | 1 |
| 30KHz | 3 | 0.5CP | 8 | 1 | 1 |

Conclusion

The following Observations and Proposals were made:

Observation 1: When the UE transmits a RACH preamble, this transmission is not time advanced to align with the uplink reception time at the gNB receiver, as TACs does not apply to PRACH.

Proposal 1: For an asymmetric DL sTRP/UL mTRP deployment with two TAs, RAN4 to introduce a further timing offset/delta in addition to n-TimingAdvanceOffset/n-TimingAdvanceOffset2 when PRACH is transmitted by the UE.

Proposal 2: Reuse Rel 18 MTTD values for Scenario #1.

Rel 18 MIMO_evo has introduced optional UE capability for operation with MRTD > CP.

UE capability rxTimingDiff-r18 cannot be reused for Rel 19 UL only TRP Scenario #1.

Proposal 3: Define a new optional UE capability to support a case scenario when Tx timing difference between two TAGs is larger than CP length.

a.	Level 1:

i.	MTTD < CP length + 1.6 us for FR1

ii.	MTTD < CP length + 0.5 us for FR2

b.	Level 2:

i.	MTTD < 34.6 us for FR1

ii.	MTTD < 8.5 us for FR2

Proposal 4: With asymmetric DL sTRP/UL mTRP, in case the UE is configured with PL offset (Scenario#1), specify conditions under which UE autonomous timing adjustment applies to just the DL/UL TRP or also to the UL-only TRP.

Proposal 5: With asymmetric DL sTRP/UL mTRP, in case the UE is configured with PL offset (Scenario#1), autonomous timing adjustment towards the UL-only TRP is disabled when RSRP variation measured at the UE is large.

Observation 5: LLS throughput starts degrading due TO with different rate when in different SNR conditions: - for SNR = -3dB from 0CP what is equivalent to 0 steps when M_D is equal to 256 - for SNR = 0dB from 0.1CP what is equivalent to 26 steps when M_D is equal to 256 - for SNR = 3dB from 0.2CP what is equivalent to 51 steps when M_D is equal to 256.

Observation 6: LLS spectral efficiency starts degrading significantly only when CFO is above 50 to 100Hz, what is equivalent to 18 to 37 steps when M_CFO is equal to 256.

Observation 8: Simulations show that CJT performance degradation due to CFO and TO depends on the SNR.

Proposal 6: RAN4 to consider no averaging as long as CJTC-D error is below 0,1 CP and CJTC-F error is below 100 Hz.

Proposal 7: Use SNR=-3 dB for defining the requirements.

Proposal 8: No scheduling restrictions needed to be applied for CJT reporting.

- References

- R4-2504903, WF on RRM requirements for NR_MIMO_Ph5_Part2

- R4-2502667, LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP/ UL mTRP, CMCC

- R4-2310178, WF on NR MIMO evolution RRM requirements, Samsung

- R4-2307185, Discussion on timing requirements for UL multi-DCI multi-TRP with two Tas, Nokia, Nokia Shanghai Bell

- R4-2507386, Other RRM requirements for MIMO ph5, Nokia

# Appendix A	LLS assumptions for CJT throughput analysis

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Carrier Frequency | 3.5GHz |
| SCS | 15kHz |
| Channel Model | TDLA30-10 |
| TRP number | 2 |
| UE antenna configuration | 1x4 |
| BS antenna configuration | 1x8 |
| UE speed | 3km/h |
| CSI-RS Bandwidth | 48 RB |
| CSI-RS configuration | 4 TRSs per TRP |
| CSI-RS periodicity | 20 slots |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRPs | from 0 to 100ppb |
| Time offset between TRPs | from 0 to CP/2 |