3GPP TSG RAN WG4 Meeting #116	 R4-2510701

Bengaluru, India, 25 Aug. – 29 Aug. 2025

Agenda Item:	7.20.4.1

Source: 	Ericsson

Title:	RRM core requirements for UE-initiated beam management

Document for:	Discussion

# Introduction

In this contribution we analyzed the UE initiated beam reporting (UE IBR) requirements for NR MIMO Phase 5.

# Discussion

Measurement delay

The measurement period for event-triggered reporting has been discussed over several meetings, and the following agreement was reached in the last meeting.

- If the measurement periods are different for current beam and new beam, the TL1_measurement period is not larger than the maximum of the L1 measurement periods to measure both current beam and new beams. - Note: The agreement does not imply that all measurements on individual reference signals are after the event is met over air. - UE shall perform L1-RSRP measurement based on current beams and new beams L1-RSRP measurement period by assuming TReport =0.

One open issue in the measurement period definition is whether to define the requirements when the measurement window is configured for the event triggered reporting.

Measurement window/counter consideration in UE initiated beam report:

RAN4 reused the principles of L3 event triggered reporting in defining the UE IBR requirements. In the event triggered reporting for L3 measurements, RAN4 did not consider TTT in the requirements definition. However, we think there is a slight difference between event triggered reporting for L3 and UE IBR. Based on the RAN1 agreement (from 120bis) below, the main difference from the L3 event triggered reporting is events do not need to be met throughout the entire window duration, but it is sufficient to be met M number of times. In the legacy event triggered reporting for L3 measurements, the UE can assess the event at the end of the TTT to determine if the event criteria have been met.

In the UE IBR context, the UE shall evaluate the event N  times to confirm that it is met for M   times, where M is a RRC configurable parameter and represented by the IE eventInstanceCount-r19. This functionality appears more stringent than the TTT for L3, where the UE can simply check at the end of the TTT to see if the event condition remains satisfied. Therefore, we suggest defining RAN4 requirements to ensure that UE measurement behaviour is tested accurately.

RAN1#120bis agreement: Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4. - Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: Timer is new beam specific. - Note: T_window is the agreed time window parameter for measurement. - Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam. Above agreement is captured in RAN1 specifications.

- RAN4 to define UE IBR requirements when the eventInstanceCount-r19 is configured.

Measurement period when time window (eventInstanceCount-r19) is configured:

We think we do not need to define the measurement period in terms of the equation and instead we can specify it as a text saying that after the first event entering condition is met, UE starts the timer with T_window and after the T_window+ TFirst UL channel, UE should send the indication on the first UL channel if the event has been met M number of times in the T_window. We think we need not specify the requirement in the equation because within the T_window, there could be N number of measurement occasions and M out of N number occasions needs to be satisfying to trigger the UEI BM report.

- RAN4 to agree on following as the method to specify the requirement when the eventInstanceCount-r19 is configured:

  - After the first event entering condition is met, UE starts the timer with length T_window and after the T_window+ TFirst UL channel, UE should send the indication on the first UL channel if the event has been met M number of times in the T_window

Other components:

In our last meeting, we reached the following agreement:

The event-triggered measurement reporting delay includes the TFirst UL channel.

- TFirst UL channel refers to the time from the RS that triggered the L1 reporting to the next first PUCCH.

We suggest one clarification, with the motivation explained below:

Once the event is triggered, the UE must send an indication on the first UL channel to notify the NW that the event has occurred, within "TFirst UL Channel." After the event is triggered, the UE may require time to prepare the new UCI for transmission on the first UL channel. Typically, the preparation time for the PUCCH or PUSCH is defined in the RAN1 specification, but it is crucial to account for this in the RAN4 requirement design. This is important because the UE might miss the first occasion of the first UL channel if the interval between the RS that triggers the event, and the first UL channel grant is insufficient for the UE to prepare and send the new UCI on the first UL channel.

- RAN4 to agree TFirst UL Channel as the first occasion of the first UL channel after the Tnew_UCI_prep time, where Tnew_UCI_prep time is the time to prepare new UCI, which may be specified in RAN1.

Measurement period for events 1 and 7

RAN4 started the requirements discussion for Event 2 and agreed to use them as the baseline for Events 1 and 7. However, we believe that few additional clarifications may be necessary based on the RAN1 definition of Event 7. For reference, the definitions of the events are provided below

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality.

  - Q is RRC configured with subjective to UE capability signalling

    - UE may only indicate a single candidate value or not support Event-7.

RAN1 defined the events based on the quality of the current beam, the quality of the new beam, and the quality of the reference signal (RS) derived from the activated TCI state with the Q-th best quality. While defining the requirements for Event 2, we have already clarified the measurement periods for obtaining the quality of the current beam and the new beam. We believe it is necessary to also clarify the measurement period for the RS derived from the activated TCI state with the Q-th best quality. For reference, the measurement periods for the current and new beams are provided below.

- the L1-RSRP measurement period of the current beam refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-2 assuming TReport. = 0, and TSSB is the periodicity of the SSB-Index of the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state for the current beam. - the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-2 assuming TReport. = 0, and TSSB is the periodicity of the SSB-Index of the reference signal configured by the newBeamResourceSet-r19.

As per RAN1 agreement, for Event 7, the scheme-1 and scheme-2 for deriving ‘RS for current beam’ on Event-2 is reused with the following further interpretation:

- Scheme-1: ‘RS for current beam’ is the QCL RS in the activated TCI state with the Q-th best quality.

- Scheme-2: ‘RS for current beam’ is the SSB which is QCLed with the QCL RS in the activated TCI state with the Q-th best quality.

- Basic feature of the triggering event determination for Event-7: Once quality of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality, UE initiated beam report occurs.

For Event 7, to identify the Qth best quality, UE needs to measure all the activated TCI states as the Qth best activated TCI state may change over time. That means the activated TCI state measurement delay shall include all the activated TCI states measurement delay. When the measurement period of all activated TCI states is not same, UE may calculate the Qth best activated TCI state after every activated TCI state measurement or after completing all the activated TCI states measurement. Since the evaluation period is not defined for the other events, we prefer defining the measurement period for each of the activated TCI state.

- For event 7, the L1-RSRP measurement period for each of the activated TCI state refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-2 assuming TReport. = 0.

Impact on unified TCI state switching delay

RAN1 has studied the following alternatives without any agreement.

Agreement

Study the following to reduce beam application latency after a UEI/ED beam report is sent

- Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling

  - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report

  - update TCI state(s) with the reported new beam(s)

  - activate new beam(s) without additional SSB reception

- Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception.

  - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent

  - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized)

- Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report.

  - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception

- Other alternatives are not precluded

In the current TCI state switching delay requirements, if a TCI state is included in the activated TCI state list, the activation delay of the indicated TCI state only includes the QCL application delay and does not include additional delay for fine time tracking. However, if the TCI state is not in the activated list, the activation delay also includes the SSB reception time to enable fine time tracking.

In the previous meeting, there were proposals to reduce the TCI state application delay. We believe RAN1 has considered several alternatives, as outlined above, including introducing additional signaling to inform the network which TCI states QCL information is stored. In essence, this is similar to TCI state activation, where the UE stores and maintains the QCL information of activated TCI states up to date. The new signaling discussed by RAN1 appears to serve a similar purpose.

However, RAN1 has not reached a consensus on this issue. Since the number of SSBs or CSI-RSs that a UE can measure is significantly higher than the number of TCI states it can activate using the active TCI state list, it remains unclear which TCI states QCL information the UE is actually remembering. If RAN1 had agreed on signaling to indicate which TCI states QCL information is remembered, defining an enhanced switching delay would be more straightforward. Without such an agreement, the UE may not retain QCL information for all TCI states, and the network cannot determine which TCI states the UE has remembered. Hence, we think it is out of RAN4 scope to introduce the TCI state activation delay reduction without RAN1 signaling.

- TCI state activation delay enhancements are not feasible without RAN1 signalling support.

Additional applicability conditions

Another issue discussed in the last meeting was whether/how to specify applicability requirements if indicated TCI states is changed during the L1 measurement time. The proposals discussed are listed below.

- Option 1: (Nokia, Ericsson, MTK)

  - No need to capture this scenario in RAN4 requirements

- Option 2: (Xiaomi, Samsung)

  - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

- Option 3: (Apple)

  - Add the applicability rule of event triggered L1 measurement:

    - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE.

    - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

Regarding this issue, RAN1 reached following agreement, and we think it is sufficient. We do not see a need to add additional applicability rules in the RAN4 specification.

Regarding triggering event determination for Event 2, on resetting the counting, the following modification on the agreed Candidate #2 in RAN1#120 is supported. - Candidate#2: [The measured current beam RS is updated based on] indicated TCI state is updated; - In such case, the UE needs to reset the counting for all new beams. FFS: Further details on the update (if necessary)

- When the indicated TCI state is changed, counter resetting mechanism is defined in RAN1 and RAN4 not to define additional applicability rules when the indicated TCI state change.

UE capability for UE IBR

Last meeting we had discussion on how many events UE can support and if there should be a capability defined. The proposals discussed were listed below.

Further discuss the two options. Proponents of option 2 to bring proposals on the exact numbers for the capabilities. - Option 1: - For event trigger L1 reporting, do not define such capability in parallel per category like L3. - Option 2: - Define capabilities for support of UE-initiated/event-driven beam reporting on top of existing L3 events in section 9.1.4. For intra-frequency, numbers of events = X, where X is candidate value of [3 or 6 or 9].

Our understanding of the Ecat capability for L3 measurements is that it introduces restrictions on the number of instances where a particular event needs to be executed for different cells on a frequency layer. In NR, the UE supports up to 64 measurement objects, 64 reporting configuration IDs, and 64 measurement IDs. Each frequency layer may include multiple cells, and the same event may be triggered by different cells at different times.

For example, if a frequency layer includes four neighboring cells and the network configures four events, the UE may need to handle up to 16 event instances simultaneously. This number can increase depending on the number of configured reporting configuration IDs or the number of neighboring cells to monitor. To address this, the concept of reporting criterion was introduced. As specified in TS 38.133, each instance of an event—whether with the same or different event identity—is counted as a separate reporting criterion in Table 9.1.4.2-1. RAN4 introduced this concept to avoid requiring the UE to handle an excessive number of event instances per frequency layer.

Do we need this type of capability for UE-initiated beam reporting (UE IBR)?

First, we need to clarify what an event instance is from the UE IBR perspective. In UE IBR, an event can be triggered for each new beam. From this perspective, each new beam that meets the event condition can be considered a separate event instance. Therefore, it may be necessary to define how many such instances can be supported per event.

We believe this can be addressed using the new UE capability introduced by RAN1 to limit the number of timers. However, this capability has so far only been discussed for event 2, and there has been no discussion of similar capabilities for event 1 or event 7.

In addition, similar to periodic, semi-persistent, and aperiodic reporting, there may be a need to define the number of event-triggered reporting configurations the UE can support. Legacy csi-ReportFramework is copied below for reference.

csi-ReportFramework Indicates whether the UE supports CSI report framework. This capability signalling comprises the following parameters: - maxNumberPeriodicCSI-PerBWP-ForCSI-Report indicates the maximum number of periodic CSI report setting per BWP for CSI report; - maxNumberPeriodicCSI-PerBWP-ForBeamReport indicates the maximum number of periodic CSI report setting per BWP for beam report. - maxNumberAperiodicCSI-PerBWP-ForCSI-Report indicates the maximum number of aperiodic CSI report setting per BWP for CSI report; - maxNumberAperiodicCSI-PerBWP-ForBeamReport indicates the maximum number of aperiodic CSI report setting per BWP for beam report; - maxNumberAperiodicCSI-triggeringStatePerCC indicates the maximum number of aperiodic CSI triggering states in CSI-AperiodicTriggerStateList per CC; - maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report indicates the maximum number of semi-persistent CSI report setting per BWP for CSI report; - maxNumberSemiPersistentCSI-PerBWP-ForBeamReport indicates the maximum number of semi-persistent CSI report setting per BWP for beam report; - simultaneousCSI-ReportsPerCC indicates the number of CSI report(s) for which the UE can measure and process reference signals simultaneously in a CC of the band for which this capability is provided. The CSI report comprises periodic, semi-persistent and aperiodic CSI and any latency classes and codebook types. The CSI report in simultaneousCSI-ReportsPerCC includes the beam report and CSI report. The UE is mandated to report csi-ReportFramework.

Support for events 1, 2, and 7 is indicated through a separate UE feature list, meaning the number of different event types a UE can support is not a limiting factor. The key issue is how many configurations of a single event type (e.g., event 2) the network can configure per cell—such as with different thresholds or reporting configuration parameters.

If there is no requirement to configure different thresholds or parameters for the same event, we do not see a need to define the number of event configurations the UE must support, as only one reporting configuration per event would be sufficient.

To maintain consistency with the legacy Csi-ReportFramework, we believe it would be beneficial for RAN1 to define the maximum number of events that should be supported for event-triggered reporting, if such a definition is deemed necessary. Based on current discussions, we observe that this topic has not yet been addressed in RAN1.

- Do not see a need for a separate capability like the L3 event evaluation category in the RAN4 specification. If a capability for the number of events to be supported is needed, it shall be defined by RAN1.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- RAN4 to define UE IBR requirements when the eventInstanceCount-r19 is configured.

- RAN4 to agree on following as the method to specify the requirement when the eventInstanceCount-r19 is configured:

  - After the first event entering condition is met, UE starts the timer with length T_window and after the T_window+ TFirst UL channel, UE should send the indication on the first UL channel if the event has been met M number of times in the T_window

- RAN4 to agree TFirst UL Channel as the first occasion of the first UL channel after the Tnew_UCI_prep time, where Tnew_UCI_prep time is the time to prepare new UCI, which may be specified in RAN1.

- For event 7, the L1-RSRP measurement period for each of the activated TCI state refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-2 assuming TReport. = 0.

- TCI state activation delay enhancements are not feasible without RAN1 signalling support.

- When the indicated TCI state is changed, counter resetting mechanism is defined in RAN1 and RAN4 not to define additional applicability rules when the indicated TCI state changes.

- Do not see a need for a separate capability like the L3 event evaluation category in the RAN4 specification. If a capability for the number of events to be supported is needed, it shall be defined by RAN1.

# References

[1]		R4-2420096, “WF on RRM requirements for NR_MIMO_Ph5”, Samsung

[2] 	RP-234007, New WID: NR MIMO Phase 5