3GPP TSG-RAN WG4 Meeting #116	R4-2510715

# Bangaluru, India, August 25th – 29th 2025

Source:	Ericsson

Title:	Discussion on SBFD demodulation requirements

Agenda item:	         7.21.6

Document for:	Discussion

# Introduction

In RAN4#115 meeting, companies had discussions on general issue of SBFD demodulation requirements. Following agreements were reached [1].

Issue 2-1-1 whether to introduce UE demodulation requirement related with inter-UE CLI for Rel-19 SBFD aware UEs

- agreement

  - Not to introduce UE demodulation test related with inter-UE CLI for Rel-19 SBFD-aware UEs

Issue 2-1-2 whether to introduce UE CSI reporting requirement for Rel-19 SBFD aware UEs

- agreement

  - Not to introduce UE CSI reporting requirement for Rel-19 SBFD-aware UEs

Issue 2-1-4 whether to introduce UE demodulation with repetition transmission

- agreement

  - Do not introduce UE demodulation requirement with repetition transmission cases for SBFD-aware UE

Issue 3-1-4 whether to introduce new PUCCH requirement to support SBFD operation

- agreement

  - No new PUCCH requirement introduced. The legacy PUCCH test cases will be applied for SBFD capable-gNB

Issue 3-1-5 whether to introduce new PRACH requirement to support SBFD operation

- agreement

  - No new PRACH requirement introduced. The legacy PRACH test cases will be applied for SBFD capable-gNB

In this paper, we provided our view on remaining issues on SBFD demodulation requirements.

# Discussion

## 2.1	UE demodulation requirements

### 2.1.1	Requirement definition

Issue 2-1-3 whether to introduce UE demodulation with SBFD carrier (with DL sub-band and UL sub-band configured)

- Way forward

  - Option 1: Define limited number of additional SBFD cases

  - Other options are not precluded

In RAN4#115, one potential scenario is proposed by company for PDSCH demodulation requirement which is the shorter HARQ ACK timeline in SBFD. In this scenario, SBFD scheduled UE would feedback HARQ ACK in SBFD UL slot rather than legacy UL slot, see figure 2.1-1 below.

Figure 2.1-1 Difference between legacy HARQ feedback and SBFD feedback

In this scenario, the interference for DL UE could be higher than legacy MU-MIMO condition since other UE could do SBFD UL transmission (CLI UE-to-UE).  In that case, the total interference includes intra-cell inter-UE interference and CLI UE-to-UE. This kind of CLI interference level might depend on UE Tx power, antenna directivity, spatial isolation between the victim and the aggressor etc.

The current CLI UE-to-UE handling is to estimate interference level by measuring SRS-RSRP on victim UE, and report measured interference level to network, then network could optimize scheduling accordingly. It is not so relevant to demodulation capability. By CLI handling, it is not clear about the remaining interference which would impact demodulation performance. A study with system level simulations could be necessary, but there is no proper interference model since Rel-18 SBFD SI till now because the whole deployment model and propagation model are very complicated. As an agreement in the last meeting, there is no proper CLI UE-to-UE interference model in current 3GPP discussion. If no CLI UE-to-UE were considered, there would be no algorithm difference between SBFD scenario and legacy network from PDSCH demodulation perspective, but only a function difference on sub-band Tx/Rx and shorten HARQ feedback.

The PDSCH performance would not be different from legacy network if no CLI UE-to-UE interference is considered. Only function is a bit different.

The sub-band transmission/reception for UE depends on RRC and DCI configuration. For non-SBFD-aware UE, it uses dynamic TDD working flow and transmit HARQ by UL sub-band in “Flexible” symbols (TDD-UL-DL-ConfigCommon on symbol is “F”). For SBFD-aware UE, it could transmit HARQ by UL sub-band in “DL” or “F” symbol (TDD-UL-DL-ConfigCommon on symbol is “D” or “F”). There is UE capability report signaling defined to differentiate non-SBFD-aware and SBFD-aware UE. From DL demodulation perspective, two types of UEs’ behavior are the same if the SBFD symbols only in F symbol (to simplify the test setup).

Both non-SBFD-aware and SBFD-aware UE can support sub-band transmission/reception and with shortened HARQ feedback.

In current specifications, sub-band PDSCH demodulation requirements are only defined for NR-U scenario which is deployed on special bands. Similarly, SBFD could be triggered on some special TDD bands, then sub-band transmission/reception can be a mandatory function for UE supporting these bands. But it is common that UE only use partial bandwidth and depends on BS scheduling. It doesn’t show much benefit to consider a requirement for this function.

If companies agree to introduce sub-band demodulation requirements for SBFD deployment, it might lead to a situation that the requirement will be applied for UE which support SBFD TDD bands only, but demodulation requirements should be band agnostic in general understanding. Regarding both SBFD-aware and non-SBFD-aware UE have chances to be scheduled in SBFD network, how to define a proper applicability rule would be an issue.

If the demodulation requirement for sub-band transmission/reception with shortened HARQ feedback is introduced, how to define applicability rule would be an issue.

Regarding observations above, we prefer not to introduce sub-band PDSCH demodulation requirements with shortened HARQ feedback to check a simple function.

Proposal 1	Do not define sub-band PDSCH demodulation requirements with shortened HARQ feedback.

Proposal 2	If new requirement is considered, companies to check how to apply requirement for non-SBFD-aware UE.

### 2.1.2 	Test case configurations if new requirement is considered

Issue 2-2-1 Number of DL sub-band if introduced UE demodulation

- Way forward

  - Option 1

    - Consider one DL sub-band and one UL sub-band case as high priority. FFS whether to consider two DL sub-band and one UL sub-band case based on RAN1 agreements

  - Other options are not precluded

In RAN1 discussion, there is an agreement that the UL sub-band could be either at the edge or in the middle of the TDD carrier. In that case, both cases could be considered. DL sub-bands could have dedicated HARQ processes and independent ACK/NACK feedback when different TB is transmitted per sub-band. But it is not common configuration in normal PDSCH demodulation then one HARQ feedback is considered.  However, FRC for two DL sub-bands would be a bit more complicated than one sub-band configuration, and   there would be no difference per DL sub-band from demodulation point of view. Thus, we prefer only to consider one DL sub-band case to simplify the simulation and corresponding test setup.

Proposal 3	Only consider one DL sub-band and one UL sub-band case for SBFD PDSCH demodulation requirement.

Issue 2-2-2 Number of Case related TDD pattern

- Way forward

  - Option 1

    - Define limited number of additional SBFD cases

  - Other options are not precluded

Issue 2-2-3 TDD pattern

- Way forward

  - Option 1

    - 30khz: XXXXXXXXUU

    - 120KHz: XXXXU

  - Other options are not precluded

Similar as legacy demodulation requirements, SBFD demodulation requirements should also be applied for different TDD patterns. Test cases based on one typical TDD pattern would be sufficient, i.e., 7D1S2U for FR1 and 3D1S1U for FR2-1. “D” and “S” slots of the TDD pattern could be configured as “F” (flexible) slots which is compatible for both non-SBFD-aware UE and SBFD-aware UE. Further to simplify the configuration, UL sub-band transmission is from the first OFDM symbol of the chosen F slot.

Proposal 4	Define SBFD PDSCH demodulation requirements for only one typical TDD pattern and apply requirements for all other TDD patterns.

Proposal 5	Use following TDD pattern for initial simulation.

- 30kHz SCS: 8F2U

- 120kHz SCS: 4F1U

- UL sub-band transmission is from the first OFDM symbol of the chosen F slot.

Issue 2-2-7 HARQ timeline

- Way forward:

  - Option 1

    - use the HARQ timeline for XXXXXXXXUU TDD pattern for 30kHz SCS and XXXXU TDD pattern for 120kHz SCS as above Figure 2.1.2-1 and Figure 2.1.2-2 respectively

Figure 2.1.2-1 HARQ timeline for XXXXXXXXUU TDD pattern for 30kHz SCS

Figure 2.1.2-2 HARQ timeline for XXXXU TDD pattern for 120kHz SCS

  - Other options are not precluded

Issue 2-2-8 number of HARQ processes

- Way forward :

  - Option 1

    - Using 4 HARQ processes for both TDD pattern

  - Other options are not precluded

In typical SBFD scenario, heavy UL transmission is expected but it would lead to a very long simulation and test time for PDSCH demodulation. Option 1 of Issue 2-2-7 in WF could be considered for initial simulation to check the sub-band Tx/Rx function.

As for the number of HARQ processes, 4 processes is generally used for FDD PDSCH demodulation requirements while 8 or higher number of processes configuration is generally used in requirements. SBFD is typically deployed on TDD bands. Once Option 1 of Issue 2-2-7 in WF is used, both 30kHz and 120kHz SCS would have 3D2U like TDD pattern. In that case, 8 processes could be used for initial simulations.

Proposal 6	Take option 1 of Issue 2-2-7 in WF for initial simulation for SBFD HARQ timeline.

Proposal 7 	Use 8 HARQ processes for both FR1 and FR2-1 SBFD demodulation requirements.

Issue 2-2-4 Channel bandwidth & SCS

- Way forward:

  - Option 1

    - FR1 with 30KHz, 40MHz DL sub-band + 20MHz UL sub-band for 30kHz SCS

    - FR2-1 with 120KH, 100MHz DL sub-band + 100MHz UL sub-band for 120kHz SCS

  - Other options are not precluded

As mentioned above, SBFD is deployed on specific TDD bands according to RF discussion and the minimum CBW is 40MHz for FR1 (n39 is 20MHz) and 100MHz for FR2-1.

For FR1, some SBFD bands only support up to 40MHz CBW, such as n38 and n39, then 40MHz DL sub-band + 20MHz UL sub-band will be beyond the UE capability. To avoid this situation, it could be better to use 20MHz DL sub-band + 20MHz UL sub-band for FR1 30kHz SCS.

For FR2-1, all operation bands support 200MHz CBW on 120kHz SCS, so 100MHz DL sub-band + 100MHz UL sub-band can be configured for all SBFD bands.

Proposal 8	Use following CBW for SBFD PDSCH demodulation requirements:

- FR1 30kHz SCS: 20MHz DL sub-band + 20MHz UL sub-band

- FR2-1 120kHz SCS: 100MHz DL sub-band + 100MHz UL sub-band

- UL sub-band is on the lower edge of the carrier.

Issue 2-2-5 MCS & Rank for PDSCH requirements

- Way forward:

  - Option 1

    - MCS 13 and rank 2

  - Other options are not precluded

Issue 2-2-6 Propagation condition for PDSCH requirements

- Way forward:

  - Option 1

    - TDLC300-100

  - Other options are not precluded

In current normal PDSCH requirements, different numbers of Rx have different combinations of rank, MCS and channel model. To simplify the requirement, only choosing one configuration per Rx configuration from legacy PDSCH requirements could be enough.  For 8Rx, there are two types of receiver assumptions which should be both considered in SBFD requirements if introduced.

- FR1 2Rx: refer to case 1-4 in Table 5.2.2.2.1-3

- FR1 4Rx: refer to case 1-4 in Table 5.2.3.2.1-3

- FR1 8Rx: refer to case 1-1 in Table 5.2.4.2.1-3 and case 2-1 in Table 5.2.4.2.1-4.

- FR2-1 2Rx: refer to case 1-3 in Table 7.2.2.2.1-3

Proposal 9	Consider following test case configuration if new requirement is considered:

| Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern (Note 1) | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | 20 DL + 20 UL / 30 | 16QAM, 0.48 | FR1.30-1 | TDLC300-100 | 2x2, ULA Low | 70 |  |
| 1 | 20 DL + 20 UL / 30 | 16QAM, 0.48 | FR1.30-1 | TDLC300-100 | 2x4, ULA Low | 70 |  |
| 2 | 20 DL + 20 UL / 30 | 64QAM, 0.50 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B (Note 2) | 70 |  |
| 1 | 100 DL + 100 UL / 120 | 64QAM, 0.46 | FR2.120-1 | TDLA30-300 | 2x2, XPL Medium | 70 |  |
| Note 1: Set all D symbols to F symbols. Note 2: Both baseline and simplified 8Rx receivers are considered. |  |  |  |  |  |  |  |

Issue 2-2-9 Other parameters

- Way forward :

  - Option 1

    - For PDSCH demodulation performance requirements, use the parameters as above Table 2.1.2-1

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |

The proposed parameters of option 1 in WF are basically follow the normal PDSCH demodulation requirements which could be used for initial simulations.

Proposal 10	Use option 1 of Issue 2-2-9 in WF for initial simulations.

### 2.1.3	Specification impact

If new requirements are introduced for UE, the corresponding applicability rule would be a question. There is no limitation for a UE to connect with SBFD network whether it is SBFD-aware or not. The only necessary capability might be the supporting SBFD band or not, but the demodulation requirements are normally band agnostic.

In Rel-16 NR-U scenario, the applicability rule is defined on a specific parameter (periodicAndSemi-PersistentCSI-RS-r16) which is introduced for NR-U deployment so that only UE supporting NR-U bands should be tested.

In that case, companies need to check if band dependency can be considered in applicability rule, such as adding one more sentence in general applicability rule for SBFD.

#### 5.1.1.1	General

The minimum performance requirements are applicable to all FR1 operating bands defined in TS 38.101-1[6].

The minimum performance requirements in Clause 5 are mandatory for UE supporting NR operation, unless otherwise specified in the other subclauses of Clause 5.1.1.

If same test is listed for different UE features/capabilities in Clauses 5.1.1.3 and 5.1.1.4, then this test shall apply for UEs which support all corresponding UE features/capabilities.

The minimum performance requirements in 5.2.xxx are applicable to all FR1 SBFD operating bands defined in TS 38.101-1[6].

Proposal 11	Companies to check if band dependency can be considered in applicability rule for SBFD PDSCH requirements if introduced.

## 2.2	BS demodulation requirements

lssue 3-1-1 whether to introduce BS demodulation requirement with considering interference modeling to account for inter-sub-band inter-gNB CLI for SBFD-capable gNB

Issue 3-1-2 interference modeling for inter-sub-band inter-gNB CLI when introducing BS demodulation requirement for SBFD-capable gNB if considered interference modeling

Issue 3-1-3 whether to introduce new PUSCH requirement with UL resource muting configured

- Way forward

  - Introduce PUSCH demodulation requirement with UL resource muting configured with interference modelling

  - FFS on which type(s) of interference will be considered to specify the PUSCH requirement

    - gNB self-interference

    - co-site inter-sector interference

    - inter-site gNB-gNB co-channel inter-subband CLI

  - FFS on how to model interference

    - gNB self-interference

      - Option 1: modelling as additive white gaussian noise with fixed INR=-6dB targeting 1dB desense as starting pointing

      - Other options are not precluded

    - inter-sector and inter-site interference

      - Option 1: modelling as additive white gaussian noise with fixed INR = - X dB based on assumption of co-site isolation

      - Other options are not precluded

    - inter-site gNB-gNB co-channel inter-sub-band CLI

In RAN4#115 meeting, companies had tense discussions on this topic. The main controversial point is whether the UL muting feature is necessary for a requirement if there is no proper interference modeling.

Feature characters

The motivation of using UL resource muting is to let BS have a better noise/interference covariance measurement on gNB-to-gNB CLI. The consequent feature characters include:

- New rate matching pattern if RE muting is enabled.

Rate matching is not a new feature. BS always do rate matching according to scheduled REs. From demodulation perspective, it is not necessary to introduce specific requirement to test rate matching.

- Use muting REs to estimate noise/interference covariance.

This is a new capability and requires receiver to have enhanced implementation on top of the legacy IRC receiver. In RAN1 discussion, the benefit of this feature is shown comparing with no muting condition, see figure 2.2-1 as a reference.

Figure 2.2-1 Example in RAN1 study. One symbol muting after the first DM-RS symbol is used to compare with legacy 1+1 DM-RS (the overhead is similar). In RAN1 link level simulation, the CDL channel model is used for CLI (3 inter-site interferers) and limited gain (~1dB) can be observed for UL muting.

Furthermore, how to use these muted REs is not captured in RAN1 specification which means it is not mandatory for BS to use them for covariance estimation.

From RAN4 demodulation perspective, the purpose might be to verify if receiver use muting REs to estimate noise covariance. In that case, obvious gains should be seen from comparison simulations. Thus, a proper interference model is necessary. In another word, it would not be necessary to introduce a requirement if we can’t see obvious gain based on simulation and interference assumptions.

Rate matching is not a new feature.

It is not mandatory for BS to use muted RE to estimate covariance matrix.

RAN1 study focuses on the gain of enabling to disabling muting and the gain is very limited based on simulation.

RAN4 demodulation focus on the gain of using muting REs to do noise estimation compared to only using DM-RS to do noise estimation (muting enabled).

Interference modeling

There are three types of interferences that are mentioned during the discussion. In current BS RF discussion, following assumptions/approaches are agreed for SBFD Rx requirements:

- Reference sensitivity level and OTA sensitivity

  - For SBFD-capable BS type 1-H, the existing requirement for conducted reference sensitivity level is also applied to BS in SBFD symbols/slots, i.e., no sensitivity degradation is allowed.

  - For SBFD-capable BS type 1-O and 2-O, maximum allowed degradation value is 0.7dB. SBFD-capable BS OTA sensitivity requirement should be conservative enough considering the receiver is further desensitized by other sources of interference including inter-site interference and inter-sector interference.

- In-band blocking

  - For FR1/FR2 local area SBFD BS, re-use existing FR1/FR2 local area BS in-band blocking power requirements.

  - For FR1 WA and MR BS, deployment restriction (e.g., a separation distance of 288 meters, corresponding to 100% grid-shift) must be implemented. Therefore, 100% grid-shift is agreed for requirement derivation.

  - gNB-to-gNB CLI handling scheme is not considered.

- New In-channel adjacent sub-band leakage and blocking

  - Inter-site interference (same operator) and inter-sector interference will be evaluated in the new in-channel adjacent sub-band leakage and blocking requirements.

  - gNB-to-gNB CLI handling discussion is still ongoing for defining the in-channel blocking requirements.

According to RF agreements, we can have the following observations on interference models

- Intra-site intra-sector interference (Self-interference):

The remaining interference after RF cancelation should be low enough. AWGN noise-like model with small power level could be considered.

It should be noted that demodulation requirements are defined on the AWGN level which should be ~15dB higher than sensitivity level in general. In that case, 0.7dB degradation on sensitivity level should not impact on demodulation performance.

- Intra-site inter-sector interference

Inter-sector interference could be nearfield radiation and there is no corresponding statistical propagation model in 3GPP scope. It seems RF requirements would handle this kind of interference, but RF requirements can’t guarantee no interference and demodulation can’t migrate it.  It needs operators to build sites with very high isolation (e.g., 80-110dB). In that case, this kind of interference is better not considered in demodulation requirements.

- Inter-site interference

Inter-site interference from the same operator is assumed to be considered in new defined BS RF requirements, but there is no consensus on CLI among different operators.

From the demodulation perspective, we don’t differentiate interference from single operator or multiple operators scenario. What we have now are INR level and AWGN or TDL models which are similar as IRC requirement assumption. If companies want to check the performance gain by using muted RE, this method could be considered.

Self-interference migration, intra-site inter-sector interference migration would be handled by BS RF requirements and special site and deployment design. Corresponding remaining interference of these two types of interference after migration would not impact much on demodulation.

Proposal 12	Only consider co-subband inter-site interference impact for UL muting case in trial simulations if new requirement is considered.

To derive a feasible interference model, it should depend on system level simulation and proper assumptions. But RAN4 currently can only create TDL + INR type of interference model such as it in IRC receiver requirements. It can’t be used to model intra-site inter-sector interference due to nearfield effect would be dominant. To address the noise estimation with extra muting REs for inter-site interference, companies could start with TDL + INR model for trial simulations.

RAN4 currently can only create TDL + INR type of interference model.

Proposal 13	Companies use TDL + INR type of interference model for trial simulations if new requirement is considered.

As for the model derivation method, we could start with typical deployment used in system level simulation, but calculate signal or interference power level only according to simplified link propagation formula (similar as link budget calculation).

The example of deployment and node assumptions could be seen in Table 2.2-1.

Table 2.2-1 Example of deployment and node assumptions for interference model derivation

| Assumptions | FR1 |
| --- | --- |
| Center frequency (GHz) | 4 |
| Deployment grid | 3 sectors Hexagonal (7 sites) Victim cell is marked as red, potential aggressor cells are marked as orange. |
| ISD (m) | 500 |
| BS configurations | TxPowerbs=53dBm EIRPbs=78dBm Carrier bandwidth 40MHz (20M DL + 20M UL) UL max combination gain: 25dB |
| UE configurations | Tx power 23dBm 1Tx, isotropic Antenna gain: 0dBi |

- Number of interferers

  - There are 4 potential interfering cells. To simplify the simulations, consider only 2 interferences Interf1 and Interf2

  - Two conditions could be considered (green arrow is signal from UE to victim BS,

    - 1 stronger interference and 1 weaker interference: Interf1 > Interf2

    - 2 similarly strong interferences: Interf_1 ≈ Interf_2.

- INR derivation methodology

  - Calculate pathloss according to Friis formula (free space path loss)

LFS=32.45+20lgd+20lgf  dB, where the unit of d is km and unit of f is MHz.

  - Interference power per PRB (aggressing BS to victim BS): Take BS-to-Bs distance dbs2bs=ISD km. Aggressing BS use full DL CBW.

LFS_bs2bs=32.45+20lgdbs2bs+20lgf               			(2.2-1)

    - Interf1 > Interf2:

Interf1 =EIRPbs-LFS_bs2bs-10lg⁡(PRBdl)                        		(2.2-2)

It is maybe too aggressive, but could be the upper bound.

Interf2 =Interf1-10dB. 		   				(2.2-3)

It includes beamforming gain degradation at both Tx and Rx sides. The degradation could be furtherly changed.

    - Interf1 ≈ Interf2:

Interf1 =Interf2 =EIRPbs-LFS_bs2bs-10lg⁡(PRBdl)-6dB 		(2.2-4)

It includes beamforming gain degradation at Tx side due to large angle deviation from boresight. The degradation could be furtherly changed.

  - Signal power per PRB (UE to victim BS): Take UE-to-BS distance due2bs=cellRange2=ISD/3 km, UE use full power transmission on sub-band.  Victim BS use max Rx gain for UE.

LFS_ue2bs=32.45+20lgdue2bs+20lgf 					(2.2-5)

SigPower=TxPowerue-LFSue2bs+maxRxGain-10lg⁡(PRBUl) 			(2.2-6)

  - Calculate INR by interference

INR1=Interf1-SigPower, INR2=Interf2-SigPower			(2.2-7)

  - Trial simulations could try several sets of values.

- Propagation model for interference

  - The channel between UE to victim BS could use typical TDL model, such as TDLC300-100 for FR1 and TDLA30-75 for FR2-1.

  - The channel between BS to victim BS could start with LOS TDL channel with zero Doppler, and the delay spread could be large, such as TDLD300 with 0 Doppler. But current TDLD channel models are TDLD10 and TDLD30 for FR2, new model profile for TDLD300 should be derived. To reduce effort in the trial simulations, TDLD30 and TDLC300 with 0 Doppler could be used.

By the methodology and assumptions in Table 2.2-1, we can summarize INR values in Table 2.2-2.

Table 2.2-2 Example of derived INR value

| Conditions | FR1 |
| --- | --- |
| Interf1 > Interf2 | INR1 <= 17.2dB, INR2 <= 7.2dB Trial simulation could try following sets: [INR1, INR2] = {[5, -5], [9, -1], [13, 3], [17, 7]} dB |
| Interf1 = Interf2 | INR1 = INR2 <=11.2dB Trial simulation could try following sets: [INR1, INR2] = {[-1, -1], [3, 3], [7, 7], [11, 11]} dB |
| Note: The interference wave form could use PUSCH with 1+1 DM-RS and CP-OFDM to simplify simulation setup. |  |

LOS channel with 0 Doppler shift would be typical for inter-site interference modeling but there is no available LOS channel model for FR1 requirements. LOS channel model used for FR2 requirements.

Proposal 14	RAN4 consider following interference modelling method for trial UL muting simulations if new requirement is considered:

• LOS channel TDLD30 or NLOS TDLC300 with 0 Doppler shift for interference in trial simulations.

• Use INR value derivation methods from formula (2.2-1) to (2.2-7) and INR values in Table 2.2-2.

- Full bandwidth PUSCH with 1+1 DM-RS and CP-OFDM to simplify simulation setup.

The next step is to decide the metrics for trial simulations.

- The first metric should be the peak throughput can be met by a certain interference configuration.

- The second metric should be the obvious performance gain (e.g., >=3dB) can be shown by simulations.

- The third metric should be the target SNR (e.g., SNR at 70% NThp) should be feasible for both network deployment and tests.

The interference model selection and conclusion on requirement introduction should be based on three metrics above. In another words, no requirement will be introduced if no simulation results can fulfill metrics above.

Proposal 15	RAN4 to use the following metrices to choose interference model based on trial simulation results.

- The peak throughput can be achieved.

- The obvious performance gain (e.g., >=3dB) can be shown by comparing using muted REs for estimation to not using muted REs for estimation.

- The target SNR (e.g., SNR at 70% NThp) should be feasible for both network deployment and tests

Proposal 16	RAN4 don’t introduce new demodulation requirements for UL muting if no proper interference modeling can be found based on metrices above.

Muting symbol configurations

It could enable muting on 1 or 2 symbols. More muting symbols would be expected to have better estimation but also lead to larger overhead. The corresponding target SNR would be higher than lower overhead case, then the target SNR would be checked if it is aggressive for the test. Regarding the typical propagation model between gNBs as discussed above (LOS channel without Doppler shift), one muting symbol would be enough for the estimation in synchronized network, but it could be OK to consider both conditions trial simulation if companies want. 1+1 DM-RS configuration is typical in normal PUSCH demodulation requirements. In that case, 1+1 DM-RS with extra muting symbols could be considered, see figure below.

Figure 2.2-2 Proposed muting pattern for trial simulations

One muting symbol might be efficient for noise estimation under typical gNB deployment in synchronized network.

Proposal 17	Companies consider 1+1 DM-RS with extra muting RE symbols for trial simulations if new requirement is considered. One muting symbol case could be prioritized.

Issue 3-2-1 Frequency Range

- Way forward

  - Option 1

    - FR1 and FR2-1

  - Other options are not precluded

Issue 3-2-2 Channel Bandwidth

- Way forward

  - Option 1

    - prioritize the minimum bandwidth sizes of SBFD for both FR1 (40 MHz) and FR2-1 (100 MHz) NR TDD bands

    - if additional bandwidth is required, 100MHz for FR1

  - Option 2

    - FR1 with 30KHz, 40MHz DL sub-band + 20MHz UL sub-band for 30kHz SCS

    - FR2-1 with 120KH, 100MHz DL sub-band + 100MHz UL sub-band for 120kHz SCS

  - Other options are not precluded

Issue 3-2-3 TDD pattern

- Way forward

  - Option 1

    - 30 kHz SCS:  XXXXXXXXUU

    - 120 kHz SCS: XXXXU

  - Other options are not precluded

Issue 3-2-4 Waveform

- Way forward

  - Option 1

    - Both DFT-s-OFDM and CP-OFDM

  - Other options are not precluded

The FR1 and FR2-1 should be considered regarding WID.

Only one bandwidth and one type of sub-band configuration can be considered for simulations. Since 40MHz CBW is supported by all FR1 SBFD bands and 200MHz CBW is supported by all FR2-1 SBFD bands, they could be considered for the requirements. The corresponding UL sub-band will be 20MHz for FR1 and 100MHz FR2-1. The SCS can choose a typical configuration, such as 30kHz for FR1 and 120kHz for FR2-1.

Proposal 18	Take 40MHz CBW (20MHz DL + 20MHz UL sub-band) 30kHz SCS for FR1 and 200MHz CBW (100MHz DL + 100MHz UL sub-band) 120kHz SCS for FR2-1 in trial simulations if new requirement is considered.

Typical TDD configuration for normal PUSCH demodulation are DDDDDDSU for FR1 and DDDSU for FR2-1. The D and S slots can be configured to “flexible” slots for UL sub-band transmission during SBFD tests. To simplify the simulations, the UL sub-band transmission could start from the first slot. A tricky part should be noted that there is no inter-site interference (in synchronized network) within full bandwidth UL symbols. It would cause more complexities on FRC definition and interference modeling definition. In the trial simulation stage, it might be OK to skip full bandwidth UL symbols during the simulations.

There is no inter-site interference in full bandwidth UL symbols in synchronized networks.

Proposal 19 	Take the following TDD patterns for trial simulations if new requirement is considered:

- 30kHz SCS: 8F2U; 120kHz SCS: 4F1U

- UL sub-band transmission could start from the first F slot.

- Skip U slots to simplify simulation setup.

As for waveform, CP-OFDM could be prioritized. DFT-s-OFDM can be further considered if obvious gain can be seen from simulations.

Proposal 20	Prioritize CP-OFDM waveform in trial simulations.

Other configurations could reuse legacy PUSCH requirements.

Issue 3-3-1 Specification Impact

- Way forward

  - Option 1: New sub-clause for demodulation requirements under same chapter with RF requirements can be considered. If RAN4 agree Not to introduce new BS demodulation requirements for SBFD, only applicability rule can be captured.

  - Other options are not precluded.

New chapter 12.5 and 12.7 are agreed to be introduced for performance requirements of SBFD-capable BS in TS 38.104. But option 1 in WF still be valid. Companies could do trial simulations first and then consider how to modify specifications.

Proposal 21 	Keep option 1 in WF until companies have conclusions from trial simulations.

# Conclusions

In the previous sections we made the following observations:

Observation 1	The PDSCH performance would not be different from legacy network if no CLI UE-to-UE interference is considered. Only function is a bit different.

Observation 2	Both non-SBFD-aware and SBFD-aware UE can support sub-band transmission/reception and with shortened HARQ feedback.

Observation 3	If the demodulation requirement for sub-band transmission/reception with shortened HARQ feedback is introduced, how to define applicability rule would be an issue.

Observation 4	Rate matching is not a new feature.

Observation 5	It is not mandatory for BS to use muted RE to estimate covariance matrix.

Observation 6	RAN1 study focuses on the gain of enabling to disabling muting and the gain is very limited based on simulation.

Observation 7	RAN4 demodulation focus on the gain of using muting REs to do noise estimation compared to only using DM-RS to do noise estimation (muting enabled).

Observation 8	Self-interference migration, intra-site inter-sector interference migration would be handled by BS RF requirements and special site and deployment design. Corresponding remaining interference of these two types of interference after migration would not impact much on demodulation.

Observation 9	RAN4 currently can only create TDL + INR type of interference model.

Observation 10	LOS channel with 0 Doppler shift would be typical for inter-site interference modeling but there is no available LOS channel model for FR1 requirements. LOS channel model used for FR2 requirements.

Observation 11	One muting symbol might be efficient for noise estimation under typical gNB deployment in synchronized network.

Observation 12	There is no inter-site interference in full bandwidth UL symbols in synchronized networks.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Do not define sub-band PDSCH demodulation requirements with shortened HARQ feedback.

Proposal 2	If new requirement is considered, companies to check how to apply requirement for non-SBFD-aware UE.

Proposal 3	Only consider one DL sub-band and one UL sub-band case for SBFD PDSCH demodulation requirement.

Proposal 4	Define SBFD PDSCH demodulation requirements for only one typical TDD pattern and apply requirements for all other TDD patterns.

Proposal 5	Use following TDD pattern for initial simulation.

	30kHz SCS: 8F2U

	120kHz SCS: 4F1U

	UL sub-band transmission is from the first OFDM symbol of the chosen F slot.

Proposal 6	Take option 1 of Issue 2-2-7 in WF for initial simulation for SBFD HARQ timeline.

Proposal 7 	Use 8 HARQ processes for both FR1 and FR2-1 SBFD demodulation requirements.

Proposal 8	Use following CBW for SBFD PDSCH demodulation requirements:

	FR1 30kHz SCS: 20MHz DL sub-band + 20MHz UL sub-band

	FR2-1 120kHz SCS: 100MHz DL sub-band + 100MHz UL sub-band

	UL sub-band is on the lower edge of the carrier.

Proposal 9	Consider following test case configuration if new requirement is considered:

Proposal 10	Use option 1 of Issue 2-2-9 in WF for initial simulations.

Proposal 11	Companies to check if band dependency can be considered in applicability rule for SBFD PDSCH requirements if introduced.

_Toc206160018

Proposal 12	Only consider co-subband inter-site interference impact for UL muting case in trial simulations if new requirement is considered.

Proposal 13	Companies use TDL + INR type of interference model for trial simulations if new requirement is considered.

Proposal 14	RAN4 consider following interference modelling method for trial UL muting simulations if new requirement is considered:

• LOS channel TDLD30 or NLOS TDLC300 with 0 Doppler shift for interference in trial simulations.

• Use INR value derivation methods from formula (2.2-1) to (2.2-7) and INR values in Table 2.2-2.

	Full bandwidth PUSCH with 1+1 DM-RS and CP-OFDM to simplify simulation setup.

Proposal 15	RAN4 to use the following metrices to choose interference model based on trial simulation results.

	The peak throughput can be achieved.

	The obvious performance gain (e.g., >=3dB) can be shown by comparing using muted REs for estimation to not using muted REs for estimation.

	The target SNR (e.g., SNR at 70% NThp) should be feasible for both network deployment and tests

Proposal 16	RAN4 don’t introduce new demodulation requirements for UL muting if no proper interference modeling can be found based on metrices above.

Proposal 17	Companies consider 1+1 DM-RS with extra muting RE symbols for trial simulations if new requirement is considered. One muting symbol case could be prioritized.

Proposal 18	Take 40MHz CBW (20MHz DL + 20MHz UL sub-band) 30kHz SCS for FR1 and 200MHz CBW (100MHz DL + 100MHz UL sub-band) 120kHz SCS for FR2-1 in trial simulations if new requirement is considered.

Proposal 19 	Take the following TDD patterns for trial simulations if new requirement is considered:

	30kHz SCS: 8F2U; 120kHz SCS: 4F1U

	UL sub-band transmission could start from the first F slot.

	Skip U slots to simplify simulation setup.

Proposal 20	Prioritize CP-OFDM waveform in trial simulations.

Proposal 21 	Keep option 1 in WF until companies have conclusions from trial simulations.

# References

[1] 	R4-2508723, Way Forward for [115][330] NR_duplex_evo_demod, Samsung