3GPP TSG-RAN WG4 Meeting #116	R4-2510716

# Bengaluru, India, August 25th – 29th 2025

Source:	Ericsson

Title:	Discussion ATG enhancement demodulation requirements

Agenda item:	7.8.7

Document for:	Discussion

# Introduction

In RAN4#115 meeting, the Rel-19 NR ATG enhancement UE and BS demodulation part are discussed. Some agreements are achieved [1].

In this paper, we will continue discuss the UE demodulation requirements for intra-band co-located and inter-band co-located DL CA enhancement, and the necessary of UL MIMO 2Tx for BS demodulation requirements.

# Discussion

## 2.1 	UE demodulation

### 2.1.1 	DL CA

It’s been raised for discussion whether to consider the new TDD pattern 30D4S6U for Rel-19 ATG downlink CA requirements.

Issue 1-1-2-1: TDD pattern - Proposal 1: Use the legacy TDD pattern as the starting point - Proposal 2: Define new TDD pattern 30D4S6U for R-19 enhanced ATG DL CA requirement

RAN4 has already defined single carrier test cases for the purpose of verifying the new TDD pattern in Rel-18. We therefore don’t see the need to reconsider the new TDD pattern for CA tests. In addition, testing the new TDD pattern would need dependency on the RRC parameter DL-DataToUL-ACK-r18 IE and dl-DataToUL-ACK-DCI-1-2-r18 to enable the K1 value range (0...31), and UE capabilities such as outOfOrderOperationDL-r16. Plus, if we look at the table below [3], the HARQ feedback configuration (k1) is very complex when it’s configured for the CA tests of FDD+TDD.

Table 5.2A-2: Test parameters for number of HARQ processes

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 8 |
|  | TDD PCell | For CC with Rank 2: 10 For CC with Rank 8: 8 | 8 |
| FDD 15 kHz + TDD 15 kHz CA | FDD PCell | 4 | 4 |
|  | TDD PCell | 8 | 8 |
| TDD 15 kHz + TDD 30 kHz CA | 15kHz PCell | 8 | 12 |
|  | 30kHz PCell | 8 | 8 |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | 4 | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 8 | N/A |

Table 5.2A-3: Test parameters for K1 values

| The number of slots between PDSCH and corresponding HARQ-ACK information |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | {2} | {2} |
|  | TDD PCell | For CC with Rank 2: {8,7,6,5,5,4,3,11} For CC with Rank 8: {8,7,6,5,5,4,3} | {7,5,4,11,9} |
| FDD 15 kHz + TDD 15 kHz CA | FDD PCell | {2} | {2} |
|  | TDD PCell | {4,3,2,6} | {4,3,2,6,5} |
| TDD 15 kHz + TDD 30 kHz CA | 15kHz PCell | {4,3,2,6} | {4,4,3,3,2,2,6,6} |
|  | 30kHz PCell | {8,7,6,5,5,4,3,2} | {7,5,4,11} |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | {2} | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | For CC with Rank 2: {8,7,6,5,5,4,3,2} For CC with Rank 8: {8,7,6,5,5,4,3} | N/A |

Based on the reasons above, we prefer not to consider the new TDD pattern 30D4S6U for downlink CA tests.

Proposal 1	Only consider the legacy TDD pattern 7D1S2U for the Rel-19 ATG enhancement demodulation tests.

For the downlink CA, it’s agreed from the WID that the following CA will be considered for the requirements:

  - FR1 intra-band contiguous CA

    - Example band combination: CA_n79C

  - FR1+FR1 inter-band CA

    - Example band combination: CA_n3A-n39A

Since only intra-band contiguous CA and inter-band CA are within the WID, we agree with the following proposal [2] for the CA configuration selection rules:

Issue 1-1-3: CA configuration selection rules - Proposal 1: - Define the CA configuration selection rules as CA capability; Step 1; Step 2; Step 3; Step 4. CA_C; Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2.; Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1.; N/A; N/A. CA_AX; Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2.; Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1.; Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2.; Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3..

Proposal 2 	Consider the following CA configuration selection rules:

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

Together with the consideration of test applicability rules, we in general agree with proposal 1 below but with some modifications to align the format with the existing CA applicability rules in TS38.101-4.

Issue 1-1-5: Test applicability rules - Proposal 1: - Define the test rules as: Tests; CA capability where the tests apply; CA configuration from the selected CA capability where the tests apply; CA Bandwidth combination to be tested in priority order; PCell CC configuration. FDD 15 kHz + FDD 15 kHz; CA_C, CA_AX; Table TBD; Largest aggregated CA bandwidth combination; Any of CCs. TDD 30 kHz + TDD 30 kHz; CA_C, CA_AX; Table TBD; Largest aggregated CA bandwidth combination; Any of CCs. FDD 15 kHz + TDD 30 kHz; CA_AX; Table TBD; Largest aggregated CA bandwidth combination; FDD CC if supported, otherwise TDD CC.

Proposal 3	Consider the following test applicability rules for ATG DL CA;

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test x in Clause 5.2A.2.x and 5.2A.3.x | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

Regarding the rank of DL CA requirements, we propose to consider only rank 1 since rank 2 with MIMO will be introduced in Rel-19 as a new feature which has its own corresponding requirements, it is not necessary to connect two new features with one requirement. Plus, RAN4 only defined downlink requirements with rank 1 for Rel-18 ATG single carrier requirements. Therefore, we propose to consider only rank 1 for the newly introduced Rel-19 ATG downlink CA requirements.

Proposal 4	Consider only rank 1 for the downlink CA requirements

As for the MCS, it’s been agreed to evaluate the following candidates:

Issue 1-1-3-3: MCS Agreement: - 16QAM MCS 13, 64QAM MCS 22, 256QAM MCS 24 for initial simulation

Based on our simulation results below, we observed good performance with configurations of MCS13 (16QAM) and MCS22 (64QAM).

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 10 | R.PDSCH.1-2.1 FDD | 16QAM, 0.48 | AWGN | 220Hz | 2x2, ULA Low | 70 | 3.40 |
| 10 | R.PDSCH.1-3.6 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2, ULA Low | 70 | 10.62 |

Considering that the propagation channel is chosen to be as AWGN, plus higher modulations are more likely to be scheduled, we propose considering MCS22 (64QAM) for ATG downlink CA requirements.

Proposal 5	Consider 64QAM MCS22 for ATG downlink CA requirements.

### 2.1.2	DL MIMO

Another objective is to specify demodulation requirements for the support of DL MIMO for UE with limited test cases.

RAN4 only considered 1 layer for ATG PDSCH demodulation requirements in the previous release since companies had concerns regarding potential inter-layer interference brought by multiple layers transmission causing by the agreed high correlation channel. Therefore, we propose to evaluate the performance of 2-layer first, configuring with both 2Rx and 4Rx, to see the feasibility of introducing requirements for 2 MIMO layers.

Currently, there are several candidates [2] for the MCS value, together with the consideration of XP-low antenna correlation.

Issue 2-1-4: MCS Agreement: - For initial simulation, consider 16QAM Table 1 MCS13, 64QAM Table 1 MCS 22 and 256QAM Table 2 MCS 24 Issue 2-1-1: Antenna configuration Agreement: - 2T2R and 2T4R FFS the antenna correlation: - Proposal 1: XP-low

To facilitate the parameter assumption, we provided our initial simulation results based on the legacy propagation condition but different MCS and antenna configurations.

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Frequency offset | antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | 10 / 15 | 16QAM, 0.48 | AWGN (Note 1) | 220Hz | 2x2 | 70 | 6.4 |
| 2 | 10 / 15 | 64QAM, 0.65 | AWGN (Note 1) | 220Hz | 2x2 | 70 | 13.5 |
| 3 | 10 / 15 | 256QAM, 0.82 | AWGN (Note 1) | 220Hz | 2x2 | 70 | 22.46 |
| 4 | 10 / 15 | 16QAM, 0.48 | AWGN (Note 1) | 220Hz | 2x4 | 70 | 3.4 |
| 5 | 10 / 15 | 64QAM, 0.65 | AWGN (Note 1) | 220Hz | 2x4 | 70 | 10.5 |
| 6 | 10 / 15 | 256QAM, 0.82 | AWGN (Note 1) | 220Hz | 2x4 | 70 | 19.45 |

Note 1: Apply the static propagation condition specified in TS 38.101-4 B.1.

From the simulation results we can observe that the performance of configuring three different MCS are all good enough for the requirement. We also observed expected MIMO gain on the maximum throughput over the result of rank 1, see figures below.

Figure 2.2-1 Performance with 16QAM

Figure 2.2-2 Performance with 64QAM

Figure 2.2-3 Performance with 256QAM

Thus, we have verified that applying MIMO with 2 layers to ATG scenario is reasonable. For the requirement, we propose to consider 64QAM as its SNR point of 70% maximum throughput is within a preferred range.

Proposal 6	Consider 64QAM for the rank 2 requirements

Proposal 7 	Consider both 2x2 and 2x4 for the antenna configurations

## 2.2	BS demodulation

Issue 3-1-4: Antenna configuration for incremental test

- Option 1: 2T2R

- Option 2: 2T2R, 2T4R and 2T8R with X-pol low correlation as the starting point

- Option 3: 2T2R with low correlation

In Rel-15, different Rx antenna configurations have been tested for 2 layers under fading channel. According to the existing Rel-15 applicability rule, both highest and lowest supported Rx antenna configurations will be tested. We don’t think it is necessary to do it again for ATG scenario with just changing channel model regarding the complexity of 8Rx test. The 2Rx configuration could be the worst case among all Rx configurations which are better for the incremental test.

As for the correlation, both low or X-pol low are OK due to no difference from the performance perspective. Considering there is no X-pol correlation matrix definition in 38.101-4 and 38.104, we prefer only to use “low correlation” in channel model definition.

Proposal 8	Use 2T2R with low correlation for ATG incremental test.

Issue 3-1-10: MCS for incremental test

Agreement:

- 16QAM Table 1 MCS12, 64QAM Table 1 MCS 20 and [256QAM Table 2 MCS 20] for initial simulation

- Only one MCS will be used

In previous meeting, companies agreed to only consider one MCS for ATG 2Tx PUSCH requirements, but corresponding modulation level and code rate are still open. First of all, 256QAM for 2Tx PUSCH would be a bit aggressive in real network which needs very good co-polarization and high SNR. Furthermore, the PUSCH 256QAM test depends on BS manufactory declaration. If only one MCS is selected, it would be better to use 16QAM or 64QAM.

Table 2.2-1: Simulation results for PUSCH, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | MCS | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 2 | Normal | AWGN + 200Hz Low | 70 % | 16 (Table1) | pos1 | 8.7 |
|  |  |  |  |  | 20 (Table1) | pos1 | 11.7 |
|  |  |  |  |  | 20 (Table2) | pos1 | 18.44 |

ATG scenario is expected to have high capacity and relatively good propagation channel, 64QAM with lower or medium MCS would be feasible for 2Tx transmission. According to simulation results, 64QAM MCS20 would have 11dB target SNR for 70% normalization throughput. Even considering several dB performance degradation caused by HW impairment and channel distortion in real network, it seems still feasible because the UL SNR can reach >20dB range based on link budget calculation.

As we indicated in the previous meeting, following 64QAM FRC tables are used in current specification:

- A.5 64QAM MCS20 R=567/1024. Used for Rel-15 PUSCH 1 layer transmission. It can be considered.

- A.10 64QAM MCS19 R=517/1024. Used for FR2 high speed train PUSCH and UL TA. It is only used in FR2.

- A.11 64QAM MCS17 R=438/1024. Used for Rel-18 FR1 PUSCH with 4Tx. For 2Tx, higher MCS is expected.

- A.13 64QAM MCS28 R=948/1024. Used for Rel-18 ATG FR1 PUSCH 1Tx. It would too aggressive for 2Tx.

It seems new FRC tables have to be introduced if 64QAM for 2Tx is considered. The 64QAM MCS20 would be better than other 64QAM MCS index used in existing specification.

In summary, if only one MCS is chosen for ATG 2Tx PUSCH demodulation requirement, 64QAM MCS20 is a better choice.

Proposal 9	RAN4 could consider 64QAM MCS20 for ATG 2Tx PUSCH demodulation requirements.

# Conclusions

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Only consider the legacy TDD pattern 7D1S2U for the Rel-19 ATG enhancement demodulation tests.

Proposal 2 	Consider the following CA configuration selection rules:

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

Proposal 3	Consider the following test applicability rules for ATG DL CA;

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test x in Clause 5.2A.2.x and 5.2A.3.x | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

Proposal 4	Consider only rank 1 for the downlink CA requirements

Proposal 5	Consider 64QAM MCS22 for ATG downlink CA requirements

Proposal 6	Consider 64QAM for the rank 2 requirements

Proposal 7 	Consider both 2x2 and 2x4 for the antenna configurations

Proposal 8	Use 2T2R with low correlation for ATG incremental test.

Proposal 9	RAN4 could consider 64QAM MCS20 for ATG 2Tx PUSCH demodulation requirements.

# References

[1] 	R4-2508724 WF on NR_ATG_enh_demodulation requirements, CMCC, RAN4 #115, St. Julian’s, Malta