3GPP TSG-RAN4 Meeting # 116	R4-2510742

# Bengaluru, India, Aug 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5926 | rev | - | Current version: | 17.18.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_RRM_enh2-Core) CR on the requirements of SCG activation_R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Change 1: It should be ‘or’ relation instead of ‘and’ in the introduction part. ‘one’ is wrongly captured as ‘in’ in Clause 8.17.1; - Change 2: The wrong referred clause in Clause 8.17.2; - Change 3: In the condition of known TCI state, the SSB should be the SSB associated with the TCI state, it is missing in the description of Clause 8.17.2. - Change 4: Adjust the alignment format in Clause 8.17.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change 1: Change the ‘and’ into ‘or’ in the introduction part. Add the missing ‘e’ in ‘one’ in Clause 8.17.1; - Change 2: Correct the wrong referred clause in Clause 8.17.2; - Change 3: In the condition of known TCI state, the SSB should be the SSB associated with the TCI state, it is missing in the description of Clause 8.17.2. - Change 4: Adjust the alignment format in Clause 8.17.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.17.1, 8.17.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

8.17	SCG Activation and Deactivation Delay

8.17.1	Introduction

This clause defines requirements for the delay within which the UE shall be able to activate one SCG or deactivate one SCG.

The requirements shall apply for NR-DC with an NR PCell, PSCell or SCell.

8.17.2	SCG Activation Delay Requirement

The requirements in this clause shall apply for the UE configured with one deactivated SCG in NR-DC and when PSCell in one SCG is being activated.

The delay within which the UE shall be able to activate the deactivated SCG depends upon the specified conditions.

Upon receiving SCG activation command in slot n, the UE shall be capable to transmit PRACH preamble or PUCCH or PUSCH towards PSCell no later than in slot n+Tactivation_timeNR slot lengtℎ ,

where:

Tactivation_time = TRRC_delay + Tprocessing + Tsearch + T∆ + TIU + 2 ms

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2].

Tprocessing is the SW processing time needed by UE, including RF warm up period. When PSCell is activated from deactivated state, if any PSCell parameter is modified, Tprocessing = 20ms. Otherwise, Tprocessing = 5 ms.

Tsearch is the time for AGC settling and PSS/SSS detection.

For RACH based PSCell activation, if the FR2 PSCell is known, Tsearch = 0 ms. If the FR2 PSCell is unknown and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms.

For RACH-less based PSCell activation, if bfd-and-RLM is configured and TCI state is known,  Tsearch = 0 ms. There are no requirements if TCI state is unknown.

T∆ is time for fine time tracking and acquiring full timing information of the target PSCell. T∆ = 1*Trs ms.

TIU: When RACH based PSCell activation is configured, it is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TIU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3].

When RACH-less based PSCell activation is configured, it is the uncertainty in acquiring the first PUSCH transmission occasion [or SR on PUCCH].

Trs is the SMTC periodicity of the PSCell if the UE has been provided with an SMTC configuration for the target cell in SCG activation command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5.

In FR2, the PSCell is known if it has been meeting the following conditions:

-	During the last 5 seconds before the reception of the SCG activation command:

-	the UE has sent a valid measurement report for the PSCell being activated and

-	One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.2.

-	One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tactivation_time according to the cell identification conditions specified in clause 9.2.

otherwise it is unknown.

If the UE is configured to perform bfd-and-RLM while the SCG is deactivated

-	The TCI state is known if all the following conditions are met:

-	During the period from the reception of the RRC command to deactivate the PSCell to the first transmission towards PSCell on PUCCH or PUSCH:

-	UE has not detected beam failure

-	The side condition Ês/Iot ≥ -3dB is fulfilled for the RSs configured for bfd-and-RLM for the PSCell being activated

-	The SSB associated with the TCI state remains detectable according to the cell identification conditions specified in clause 9.2.

-	Otherwise, the TCI state is unknown.

The PCell interruption specified in clause 8.2 is allowed only during the RRC reconfiguration procedure [2].

<End of Change #1>