3GPP TSG-RAN4 Meeting # 116	R4-2510746

# Bengaluru, India, Aug 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5930 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_ATG-Core) CR on CSI-RS based L3 measurements of ATG_R19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-Core |  |  |  |  |  | Date: |  |  | 2025-08-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The sharing factor Klayer1_measurement is needed for ATG UE capable of antennaArrayType-r18 when the ATG UE performing L3 measurements. The value of Klayer1_measurement is up to the overlapping situation between L3 RS resource and L1 RS resource. In the CSI-RS based L3 measurement, the overlapping situation is wrongly captured as between the L3 SSB and L1 RS resource. We correct this error. Regarding how to capture Klayer1_measurement correctly for ATG, we follow the below analysis: For CSI-RS based L3 intra-frequency measurement in FR2, in legacy TN, no sharing factor Klayer1_measurement was introduced in FR2 CSI-RS based L3 measurement requirements. But for ATG scenario, since the ISD could be up to 300km, a quite wide range of time offset between UE receiving L3 CSI-RS from neighbour cell and L1 SSB/CSI-RS from serving cell at same frequency should be considered. So we apply the concept of L3 CSI-RS window to distinguish the overlapping/non-overlapping between L3 CSI-RS and L1 SSB/CSI-RS, then to capture the CSI-RS based L3 intra-frequency measurement requirements. For CSI-RS based L3 inter-frequency measurement, since it could only with gap, no need Klayer1_measurement - Besides, some unnecessary blank is removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the L3 CSI-RS based measurement requirements for ATG UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.10D.2.5, 9.10D.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

9.10D	CSI-RS based L3 measurements for ATG

9.10D.2.5	Intra-frequency measurements without measurement gaps

If a UE is configured with the higher layer parameters CSI-RS-Resource-Mobility and associatedSSB, the CSI-RS based measurement shall include PSS/SSS detection time of associatedSSB, the time period used to acquire the SFN information and CSI-RS based measurement period without gap.

-	PSS/SSS detection time of associatedSSB is the intra-frequency TPSS/SSS_sync_intra in clause 9.2D.5.1.

-	The time period used to acquire the SFN information is equal to 0 if the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise, the time period used to acquire the SFN information is TCSI-RS_SFN_intra as shown in table 9.10D.2.5-3 for FR1.

-	If the associatedSSB, which has been detectable at least for the time period Tidentify_intra_with_index defined in clause 9.2D.5.1, becomes undetectable for a period ≤ 5 seconds and then the associatedSSB becomes detectable again with the same spatial reception parameter provided the timing to that cell has not changed more than  3200/2µ Tc, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3], PSS/SSS detection time and time period used to acquire the SFN information are equal to 0.

The measurement period for CSI-RS based intra-frequency measurements without gaps is as shown in table 9.10D.2.5-1.

Additionally, for a given CSI-RS resource, if the associated SS/PBCH block is configured but not detected by the UE, or if CSI-RS is configured with associated SSB but not QCL-ed to the associated SSB, the UE is not required to monitor the corresponding CSI-RS resource.

Table 9.10D.2.5-1: Measurement period for intra-frequency CSI-RS based measurements without gaps(FR1)

| DRX cycle | T CSI-RS_measurement_period_intra |
| --- | --- |
| No DRX | max(200 ms, ceil( 5 x Kp_CSI-RS x N1Note 3 x Klayer1_measurement ) x CSI-RS period) x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x 5 x Kp_CSI-RS x N1Note 3 x Klayer1_measurement ) x max(CSI-RS period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil( 5 x Kp_CSI-RS x N1Note 3 x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: The requirements apply assuming CSI-RS configuration with {D=3 with PRBs ≥ 48}. D is frequency domain density for the 1-port CSI-RS for L3 mobility defined in clause 7.4.1 of TS 38.211 [6]. NOTE 2: Kp_CSI-RS is applicable for a UE supporting concurrent gaps NOTE 3: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. |  |

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in clause 9.1D.5.

For a UE not supporting concurrentMeasGap-r17 or for a UE is supporting concurrentMeasGap-r17 but not configured with concurrent measurement gaps,

-	if the intra-frequency CSI-RS resource does not overlap with any measurement gaps, Kp_CSI-RS=1;

-	if some occasions of the intra-frequency CSI-RS resource is overlapped with measurement gaps, Kp_CSI-RS = 1/(1- (CSI-RS resource period /MGRP)) , where CSI-RS resource period < MGRP, and the MGRP is the periodicity of the measurement gap.

-	Otherwise, when UE supports concurrent measurement gaps, and concurrent measurement gaps are configured, Kp_CSI-RS is the scaling factor for a CSI-RS frequency layer to be measured outside gap which is defined as Kp_CSI-RS = Ntotal / Navailable

For a window W of duration max(CSI-RS period,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG, and starting at the beginning of any gap occasions covering the CSI-RS resources:

Ntotal is the total number of CSI-RS resources within the window, including those overlapped with other MG occasions within the window, and

Navailable is the number of CSI-RS resources that are not overlapped with any other non-dropped MG occasion within the window W, after accounting for MG collisions by applying the selected gap collision rule.

Kp_CSI-RS = 1 when Navailable = 0

For UE supporting antennaArrayType-r18,

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions and CSI-RSs resource occasions,

Klayer1_measurement=1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

For UE not supporting antennaArrayType-r18, Klayer1_measurement=1.

Table 9.10D.2.5-2: Void

Table 9.10D.2.5-3: Time period for SFN acquisition for intra-frequency CSI-RS based measurements without gaps(FR1)

| DRX cycle | TCSI-RS_SFN_intra |
| --- | --- |
| No DRX | max(200 ms, ceil(5 x Kp_CSI-RS x N1Note 2 x Klayer1_measurement ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(2000 ms, ceil (1.5 x 5 x Kp_CSI-RS x N1Note 2 x Klayer1_measurement ) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil(5 x Kp_CSI-RS x N1Note 2 x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. |  |

<End of Change #1>

<Start of Change #2>

9.10D.3.5	Inter-frequency measurements with measurement gaps

When measurement gaps are provided, if configured with the higher layer parameters CSI-RS-Resource-Mobility and associatedSSB, the UE shall be able to identify a new detectable CSI-RS based inter-frequency cell within T CSI-RS_identify_inter,

T CSI-RS_identify_inter = (TPSS/SSS_sync + T CSI-RS_measurement_period_inter + TCSI-RS_SFN_inter) ms

Where:

TPSS/SSS_sync is the time period used in PSS/SSS detection which is determined according to TPSS/SSS_sync_inter in clause 9.3D.4,

TCSI-RS_SFN_inter is the time period used to acquire the SFN information of the cell being measured, which is shown in table 9.10D.3.5-3 for FR1,

TCSI-RS_measurement_period_inter: equal to a measurement period of CSI-RS based measurement given in table 9.10D.3.5-1.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1D.5 for measurement conducted within measurement gaps.

If a UE which supports concurrent measurement gaps has been configured with concurrent measurement gaps, Kp_CSI-RS is the scaling factor for a CSI-RS frequency layer to be measured within the associated measurement gap which is defined as Kp_CSI-RS = Ntotal / Navailable. Kp_CSI-RS = 1 for for UE not configured with concurrent measurement gaps.

-	For a window W of duration max(CSI-RS period,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR1 MG within the same FR as the CSI-RS frequency layer, and starting at the beginning of any gap occasions covering the CSI-RS resources.:

-	Ntotal is the total number of associated gap occasions covering CSI-RS resources within the window, including both dropped and non-dropped instances of the associated measurement gap within the window, and

-	Navailable is the number of non-dropped associated gap occasions covering CSI-RS resources within the window W, after accounting for MG collisions by applying the selected gap collision rule.

-	Requirements do not apply if Navailable = 0

Additionally, for a given CSI-RS resource, if the associated SSB is configured but not detected by the UE, or if CSI-RS configured with associated SSB but not QCL-ed to the associated SSB, the UE is not required to monitor the corresponding CSI-RS resource.

Table 9.10D.3.5-1: Measurement period for CSI-RS based inter-frequency measurements with gaps (FR1)

| Condition NOTE1 | T CSI-RS_measurement_period_inter |
| --- | --- |
| No DRX | Max(200 ms, ceil(8 Kp_CSI-RS x N1Note 3) Max(MGRP, CSI-RS period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil(8 1.5 Kp_CSI-RS x N1Note 3) Max(MGRP, CSI-RS period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(8 Kp_CSI-RS x N1Note 3) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: Kp_CSI-RS is applicable for a UE supporting concurrent gaps NOTE 3: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. |  |

Table 9.10D.3.5-2: Void

Table 9.10D.3.5-3: Time period for SFN acquisition for inter-frequency CSI-RS based measurements with gaps(FR1)

| Condition NOTE1 | T CSI-RS_SFN_inter |
| --- | --- |
| No DRX | Max(200 ms, ceil(5 Kp_CSI-RS x N1Note 3) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil(5 1.5 Kp_CSI-RS x N1Note 3 ) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(5 Kp_CSI-RS x N1Note 3) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: Kp_CSI-RS is applicable for a UE supporting concurrent gaps NOTE 3: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. |  |

<End of Change #2>