3GPP TSG-RAN WG4 Meeting # 116	R4-2510749

# Bengaluru, India, 25th – 29th Aug, 2025

Source:	LG Electronics

Title:	Discussion for LP-WUS/WUR REFSENS

Agenda item:	7.24.3.2

Document for:	Discussion

Introduction

In RAN4#115, WF on LP-WUS requirements was approved. Based on the WF, we would like to share our view for the requirements. [1]

Discussion

This contribution aims to share the link-level simulation results of LP-SS in FR2.

Table 1 Simulation assumption

| Parameters | Assumption |
| --- | --- |
| Waveform | OOK-1,OOK-4 |
| Carrier Frequency | 24.3GHz (FR2) |
| Bandwidth | 100/200 MHz |
| Time error | 0 us |
| CFO | 0 ppm |
| SCS | 120kHz |
| WUS RBs | 11 RBs (~3.96 MHz) |
| Filter | Butterworth (5 Order) |
| ADC | 4 bits |
| Channel propagation | AWGN, TDLC-100 |
| Antenna configuration | 1x1 Low |
| Phase noise model | TR38.803 Ex2 |
| Type receiver | OFDM |
| Payload size | 5 bits |
| Manchester coder | 2-length |
| Number of coded bits | 16,32,64 bits with MC |
| Type sequence | OFDM overlaid sequence |

Table 2 Simulation results

| Scenario | Size coded bits | OOK-1 AWGN | OOK-1 TDLC100 | OOK-4 AWGN | OOK-4 TDLC100 |
| --- | --- | --- | --- | --- | --- |
| Payload: 5 bits BW: 100 MHz w/o PN | 16 | -11.9 | 3.9 | -9.2 | 7.8 |
|  | 32 | -12.6 | 4.4 | -10.1 | 7 |
|  | 64 | -12.8 | 5.4 | -10.5 | 7.1 |
| Payload: 5 bits BW: 100 MHz with PN | 16 | -11.9 | 5.1 | -9.4 | 7.3 |
|  | 32 | -12.6 | 4.6 | -10.1 | 7 |
|  | 64 | -12.8 | 4.4 | -10.5 | 7 |
| Payload: 5 bits BW: 200 MHz w/o PN | 16 | -11.9 | 4.6 | -9.2 | 8.1 |
|  | 32 | -12.6 | 4.7 | -10.1 | 7 |
|  | 64 | -12.8 | 4.9 | -10.5 | 7 |
| Payload: 5 bits BW: 200 MHz with PN | 16 | -11.9 | 4.6 | -9.5 | 7.8 |
|  | 32 | -12.6 | 4.4 | -10.1 | 7 |
|  | 64 | -12.8 | 5.1 | -10.5 | 7 |

Observation: In FR2, SNR can vary significantly depending on channel propagation conditions. Therefore, if SNR is defined assuming an AWGN channel, as in FR1, it is reasonable to use the same SNR values as those in FR1. However, if TDLC is considered, higher SNR values may need to be applied.

Proposal 1: Reuse the same SNR value used in FR1 when evaluating REFSENS for LP-WUS in FR2.

Proposal 2: Do not consider TDLC for LP-WUS SNR in FR2.

Conclusion

Observation: In FR2, SNR can vary significantly depending on channel propagation conditions. Therefore, if SNR is defined assuming an AWGN channel, as in FR1, it is reasonable to use the same SNR values as those in FR1. However, if TDLC is considered, higher SNR values may need to be applied.

Proposal 1: Reuse the same SNR value used in FR1 when evaluating REFSENS for LP-WUS in FR2.

Proposal 2: Do not consider TDLC for LP-WUS SNR in FR2.

- References

# [1] R4-2507936, WF for [115][131] NR_LPWUS_UERF  (Vivo)

- Appendix

Simulation results for FR2 LP-SS