3GPP TSG-WG4 Meeting # 116	R4-2510755

# Bengaluru, IN, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5936 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133 corrections to core requirement for NR positioning (Rel.19) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-08-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Clause number for accuracy requirement for UE Rx-Tx time difference measurement for PRS aggregation from multiple PFLs is incomplete. - There is square bracket around SRS transmission timing for the applicability of UE Rx-Tx time difference measurement. - Incorrect use of symbol for millisecond. - Scheduling availability of RedCap UE during PRS measurement without MG and without FH needs to be updated to capture the IEs used in the LPP spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clause number for accuracy requirement for UE Rx-Tx time difference measurement for PRS aggregation from multiple PFLs is updated. - Square bracket around SRS transmission timing for the applicability of UE Rx-Tx time difference measurement is removed. - Symbol for millisecond is rectified from “msec” to “ms”. - Scheduling availability of RedCap UE during PRS measurement without MG and without FH is updated to capture the IEs used in the LPP spec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Clause number for accuracy requirement for UE Rx-Tx time difference measurement for PRS aggregation from multiple PFLs remains incomplete. - SRS transmission timing for the applicability of UE Rx-Tx time difference measurement remains ambiguous. - Symbol for millisecond is not correct. - Scheduling availability of RedCap UE during PRS measurement without MG and without FH is not aligned with the LPP spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.4.2, 9.9.8.2, 9.9A.1.3, 9.9A.4.2, and 9.9C.4.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Corresponding CAT-F CR R4-2512155. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## START OF CHANGE 1

#### 9.9.4.2	Requirements Applicability

The requirements in clause 9.9.4 apply for periodic and triggered UE Rx-Tx time difference measurements, provided:

-	UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25 are met for a corresponding band.

-	UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25A are met for PRS aggregation from multiple PFLs.

-	SRS is configured on at least one of the PCell, PSCell and SCell.

-	The UE transmits SRS within [-160, 160] msec of at least one DL PRS resource of each of the TRPs in the assistance data.

## END OF CHANGE 1

## START OF CHANGE 2

#### 9.9.8.2	Requirements Applicability

The requirements in clause 9.9.8 apply for periodic and triggered UE Rx-Tx time difference measurements, provided:

-	UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25 are met for a corresponding band.

- 	DL RSCP related side conditions given in clause 10.1.44.2 for FR1 and FR2 are fulfilled, for a corresponding band.

-	SRS is configured on at least one of the PCell, PSCell and SCell.

-	The UE transmits SRS within the range from -160 ms to 160 ms of at least one DL PRS resource of each of the TRPs in the assistance data.

## END OF CHANGE 2

## START OF CHANGE 3

#### 9.9A.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps for RedCap positioning without FH

The requirements in this clause apply for RedCap UE performing RSTD, PRS-RSRP, UE Rx-Tx time difference and PRS-RSRPP measurement without measurement gaps.

If prs-ProcessingWindowType1A-r17 UE capable of supporting priority options 1, 2 and 3 is configured with a type1A PRS processing window with priority value ‘st1’ for PRS measurement, then UE is not expected to receive PDCCH/PDSCH/CSI-RS on all symbols within PRS processing window.

If prs-ProcessingWindowType1A-r17 UE capable of supporting priority option 2 is configured with a type1A PRS processing window with priority state ‘st2’ for PRS measurement, then UE is not expected to receive PDSCH/CSI-RS on all symbols within PRS processing window but shall receive PDCCH and URLLC PDSCH within PRS processing window.

If prs-ProcessingWindowType1B-r17 UE capable of supporting priority options 1, 2 and 3 is configured with a type1B priority value ‘st1’ for PRS measurement, then UE is not expected to receive PDCCH/PDSCH/CSI-RS from serving cells in the same band as DL PRS on all symbols within PRS processing window.

If prs-ProcessingWindowType1B-r17 UE capable of supporting priority option 2 is configured with a type1B PRS processing window with priority value ‘st2’ for PRS measurement, then UE is not expected to receive PDSCH/CSI-RS from a certain band but shall receive PDCCH and URLLC PDSCH from serving cells in the same band as DL PRS on all symbols within PRS processing window.

If prs-ProcessingWindowType2-r17 UE capable of supporting priority options 1, 2 and 3 is configured with a type2 PRS processing window with priority value ‘st1’ for PRS measurement, then the UE is not expected to receive PDCCH/PDSCH/CSI-RS on the symbols overlapped with DL PRS within PRS processing window.

If prs-ProcessingWindowType2-r17 UE capable of supporting priority option 2 is configured with a type2 PRS processing window with priority value ‘st2’ for PRS measurement, then UE is not expected to receive PDSCH/CSI-RS on the symbols overlapped with DL PRS within PRS processing window but shall receive PDCCH and URLLC PDSCH on the symbols overlapped with DL PRS within PRS processing window.

When PRS is lower priority than the DL signals/channels the following applies when UEs have an activated PRS processing window with type1A or type1B:

-	If UE determines the presence of other DL signals/channels except SSB of higher priority than PRS in the PPW no later than N2 symbols, defined in clause 6.4 of [26], for the subcarrier spacing μ of the DL PRS, before the start of the PPW, UE expects to receive the DL signals/channels and drop all DL PRS in the PPW.

-	If UE determines the presence of other DL signals/channels except SSB of higher priority than PRS in the PPW later than N2 symbols, defined in clause 6.4 of [26], for the subcarrier spacing μ of the DL PRS, before the start of the PPW, UE is not required to receive the other DL signals/channels except SSB of higher priority and may receive the DL PRS in the PPW.

When PRS is lower priority than the DL signals/channels the following applies when UEs have an activated PRS processing window with type2:

-	If UE determines the presence of other DL signals/channels except SSB of higher priority than PRS on a PRS symbol no later than N2 symbols, defined in clause 6.4 of [26] for the subcarrier spacing μ of the DL PRS, before the PRS symbol, UE expects to receive the DL signals/channels and drop the PRS symbol.

-	If UE determines the presence of other DL signals/channels except SSB of higher priority than PRS on a PRS symbol later than N2 symbols, defined in clause 6.4 of [26] for the subcarrier spacing μ of the DL PRS, before the PRS symbol, UE is not required to receive the other DL signals/channels except SSB of higher priority and may receive the PRS symbol.

For inter-band case for FR2 for the DL signals/channels from a different FR2 band than the FR2 band of the DL PRS, if the same Rx beam is used for both FR2 bands and the DL PRS is determined to be higher priority, UEs are not expected to receive the DL signals/channels if they are expected to measure the DL PRS in a configured DL PRS processing window with type1B or type2.

## END OF CHANGE 3

## START OF CHANGE 4

#### 9.9A.4.2	Requirements Applicability

The requirements in clause 9.9A.4 apply for periodic and triggered UE Rx-Tx time difference measurements, provided:

-	UE Rx-Tx time difference measurement related side conditions given in clause 10.1A.18.2 are met for a corresponding band.

-	SRS is configured on at least one of the PCell, PSCell and SCell.

-	The UE transmits SRS within the range from -160 ms to 160 ms of at least one DL PRS resource of each of the TRPs in the assistance data.

## END OF CHANGE 4

## START OF CHANGE 5

#### 9.9C.4.2 Requirements Applicability

The requirements in clause 9.9C.4 apply for periodic and triggered UE Rx-Tx time difference measurements, provided:

-	UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25C are met for a corresponding band.

-	SRS is configured on the PCell.

-	The UE transmits SRS within the range from -160 ms to 160 ms of the DL PRS resource of the TRP corresponding to the PCell in the assistance data.

## END OF CHANGE 5