3GPP TSG-RAN WG4 Meeting # 116 	    R4-2510758

Bengaluru, IN, 25 – 29 August 2025

Agenda Item:	7.17.5

Source: 	Ericsson

Title:	On core requirements for AI/ML based positioning

Document for:	Approval

# Introduction

This contribution discusses the core requirements for AI/ML based positioning use case #1 in the revised WID for AI/ML for NR air interface [1].

# Discussion

In the last meeting, the following agreement was reached in the ad-hoc session based on the discussion on core requirement for positioning use case #1.

Agreement from RAN4#115 [2]:

- Discuss how to define a measurement delay requirement(reporting delay).

- Do not define any positioning accuracy requirement.

In positioning use case #1 the AI/ML model is deployed at the UE to support the AI/ML based positioning and concerns the scenario where the UE is configured by the network node to infer its location and report it to the network. To ensure that the inferred location information is reported by the UE to the network in a timely manner, reporting delay requirement for positioning use case #1 needs to be specified.

Observation 1: To ensure that the inferred location information is reported by the UE to the network in a timely manner, reporting delay requirement for positioning use case #1 needs to be specified.

Measurement reporting delay for AI/ML based positioning use case #1 should be defined as the time between the moment the position inference reporting request from the network is received by the UE and the moment when the UE starts to transmit the inferred UE location report over the air interface to be received by the network. Upon receiving position inference request from the network, the UE first needs to perform measurements on the PRS resources to generate input for its AI/ML model, infer its location as an output of its AI/ML model, and report the inferred position to the network node. The measurement reporting delay for AI/ML positioning use case #1 should therefore be defined by taking these factors into account.

Proposal 1: Measurement reporting delay for AI/ML positioning use case #1 should be defined as the time between the moment the position inference reporting request from the network is received by the UE and the moment when the UE starts to transmit the inferred UE location report over the air interface to be received by the network.

Measurement period for model input generation refers to the duration of measurements performed by the UE on PRS resources to provide input to its AI/ML model to infer the UE location. Duration of such measurements depends on the number of measured positioning frequency layers (PFLs), scaling factor associated to the measured PFL, Rx beam sweeping factor, number of samples for the PRS measurement, and the PRS processing capability of the UE. The impact of these factors on the measurement delay for model input generation, in principle, is similar to the impact on measurement period requirement for the legacy positioning measurements. Measurement period requirement for the legacy positioning measurement can therefore be used as a baseline to define the delay requirement for the model input generation for the UEs capable of supporting AI/ML positioning use case #1.

Observation 2:  Measurement delay for model input generation depends on the number of measured PFLs, scaling factor associated to the measured PFL, Rx beam sweeping factor, number of samples for the PRS measurement, and the PRS processing capability of the UE.

Proposal 2: Measurement period requirement for legacy positioning measurements is used as a baseline to define the measurement period for the AI/ML model input generation for the UE capable of supporting AI/ML positioning use case #1.

Unlike in the legacy positioning measurements, UE can perform PRS measurements for its model input with a single Rx timing error group (TEG) limiting the impact of number of Rx TEGs on the measurement period requirement for measurements performed by UE for model input in use case #1 for positioning inference. Besides, the usage of Rx TEG is also transparent from the signaling point of view for the UE based positioning method, thus adding limitation to the testing procedure to verify the impact of number of TEGs on the measurement delay requirement. Therefore, impact of the number of Rx TEGs on the measurement period requirement for measurements performed by UE in use case #1 for positioning inference should not be considered.

Proposal 3: Impact of the number of Rx TEGs on the measurement period requirement for measurements performed by UE in use case #1 for positioning inference is not considered.

# Summary

In this contribution, issues related to the core requirement for AI/ML positioning use case #1 in WID are discussed. The observations and proposals below summarize the discussions presented in this paper.

Observation 1: To ensure that the inferred location information is reported by the UE to the network in a timely manner, reporting delay requirement for positioning use case #1 needs to be specified.

Proposal 1: Measurement reporting delay for AI/ML positioning use case #1 should be defined as the time between the moment the position inference reporting request from the network is received by the UE and the moment when the UE starts to transmit the inferred UE location report over the air interface to be received by the network.

Observation 2:  Measurement delay for model input generation depends on the number of measured PFLs, scaling factor associated to the measured PFL, Rx beam sweeping factor, number of samples for the PRS measurement, and the PRS processing capability of the UE.

Proposal 2: Measurement period requirement for legacy positioning measurements is used as a baseline to define the measurement period for the AI/ML model input generation for the UE capable of supporting AI/ML positioning use case #1.

Proposal 3: Impact of the number of Rx TEGs on the measurement period requirement for measurements performed by UE in use case #1 for positioning inference is not considered.

# References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm Incorporated.