3GPP TSG-RAN WG4 Meeting # 116 	    R4-2510759

Bengaluru, IN, 25 – 29 August 2025

Agenda Item:	7.17.6

Source: 	Ericsson

Title:	On performance requirements for AI/ML based positioning

Document for:	Approval

# Introduction

In the last meeting, measurement report mapping for sample-based measurement reporting for AI/ML positioning use case 3b was discussed and the following agreement was reached. This contribution further discusses additional aspects related to the sample-based measurement reporting and addresses the FFS item identified in the RAN4#115 meeting.

Agreement from RAN4#115 [1]:

- Use the value of the legacy UL RTOA mapping for 1st sample

- FFS for the additional sample reporting

# Discussion

The following agreements have been reached in RAN1 on reporting for use case 3b.

Agreement from RAN1#116 [2]

- For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting:

- timing information.

- paired timing information and power information.

Agreement from RAN1#119 [3]

For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement,

- The measurement is composed of Nt´ values of the estimated channel response in time domain. The Nt´ values are selected from a list of Nt consecutive channel response values, which have timing granularity T.

- The timing information for the Nt´ values are reported with a timing granularity T, where T=2k×Tc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR.

  - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values.

- The timing information is defined relative to a reference time, same as the path-based measurement.

- The Nt´ selected time domain channel measurement values are expected to be those with the highest power.

- The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T.

- LMF can signal parameter values of Nt, Nt´, k to gNB via NRPPa. Candidate set values:

  - Nt´<=24. FFS: Nt´ values.

  - Nt = {32, 64, 128}

  - FFS: k

  - The gNB/TRP may use different Nt´, Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement

- FFS: whether transmit offset from gNB to LMF

Note: measurement by UE is a separate discussion.

Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

Agreement from RAN1#120 [4]

For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting:

- k = {0...5}

Agreement from RAN1#120 [4]

For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt´ values” in RAN1#119 agreement is resolved by supporting:

- Nt´ = {8, 16, 24}

Based on the agreements above, the following observations on measurement reporting from gNB for use case 3b can be made:

Observation 1: Depending on the request from LMF, gNB can report path-based measurements to the LMF. For such measurement reporting, no enhancement to Rel. 18 reporting has been introduced.


Observation 2: Depending on the request from LMF, gNB can report sample-based measurements to the LMF. For such measurement reporting, report mapping table needs to be specified in RAN4 specification.

Observation 3: For sample-based reporting, gNB may either report timing measurement, or timing and power measurements associated to Nt´ samples of the estimated channel response in time domain.

Observation 4: The minimum number of samples for which the timing, or timing and power information are reported is 8 and the maximum number of samples for which the timing or timing and power information are reported is 24.

For sample-based reporting, two new measurements have been specified in the latest version of the TS 38.215 [5]. For sample time reporting UL SRS time domain channel timing (UL SRS-TDCT) measurement has been specified and for sample power reporting UL SRS time domain channel power (UL SRS-TDCP) measurement has been specified. In the last meeting it was agreed to reuse the value of the legacy UL RTOA mapping for reporting TDCT of the 1st sample. The values for reporting TDCT of additional samples was agreed FFS. The FFS item relates to if the timing information for additional samples is reported relative to the timing of the first sample or not, mandating a need of additional report mapping tables for relative timing reporting if the timing information for additional samples is reported relative to the timing of the first sample. Since the timing information for additional samples is not reported relative to the timing of the first sample, no additional report mapping tables therefore needs to be specified. The report mapping table agreed in the last meeting shall suffice the need of report mapping for UL SRS-TDCT reporting for positioning use case 3b and is provisioned in draftCR R4-2510760 [6] for endorsement.

Reporting range and the measurement reporting values for UL SRS-TDCP reporting is yet to be discussed and agreed in RAN4. In our view similar to reusing the reporting range and values of UL RTOA for UL SRS TDCT reporting, reporting range and values from the report mapping table for UL SRS RSRPP measurement shall be reused for UL SRS-TDCP reporting. A report mapping table for UL SRS-TDCP measurement based on the report mapping table for UL SRS-RSRPP measurement is therefore proposed in Table 1 below.

Proposal 1: Reporting range for UL SRS-TDCP is defined from -156 dBm to -31 dBm with resolution 1 dB.

Table 1. UL SRS-TDCP report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| SRS_TDCP_000 | SRS-TDCP<-156 | dBm |
| SRS_TDCP_001 | -156 ≤ SRS-TDCP < -155 | dBm |
| SRS_TDCP_002 | -155 ≤ SRS-TDCP < -154 | dBm |
| SRS_TDCP_003 | -154 ≤ SRS-TDCP < -153 | dBm |
| SRS_TDCP_004 | -153 ≤ SRS-TDCP < -152 | dBm |
| SRS_TDCP_005 | -152 ≤ SRS-TDCP < -151 | dBm |
| SRS_TDCP_006 | -151 ≤ SRS-TDCP < -150 | dBm |
| SRS_TDCP_007 | -150 ≤ SRS-TDCP < -149 | dBm |
| SRS_TDCP_008 | -149 ≤ SRS-TDCP < -148 | dBm |
| SRS_TDCP_009 | -148 ≤ SRS-TDCP < -147 | dBm |
| SRS_TDCP_010 | -147 ≤ SRS-TDCP < -146 | dBm |
| SRS_TDCP_011 | -146 ≤ SRS-TDCP < -145 | dBm |
| SRS_TDCP_012 | -145 ≤ SRS-TDCP < -144 | dBm |
| SRS_TDCP_013 | -144 ≤ SRS-TDCP < -143 | dBm |
| SRS_TDCP_014 | -143 ≤ SRS-TDCP < -142 | dBm |
| SRS_TDCP_015 | -142 ≤ SRS-TDCP < -141 | dBm |
| SRS_TDCP_016 | -141 ≤ SRS-TDCP < -140 | dBm |
| SRS_TDCP_017 | -140 ≤ SRS-TDCP < -139 | dBm |
| SRS_TDCP_018 | -139 ≤ SRS-TDCP < -138 | dBm |
| … | … | … |
| SRS_TDCP_111 | -46 ≤ SRS-TDCP < -45 | dBm |
| SRS_TDCP_112 | -45 ≤ SRS-TDCP < -44 | dBm |
| SRS_TDCP_113 | -44 ≤ SRS-TDCP < -43 | dBm |
| SRS_TDCP_114 | -43 ≤ SRS-TDCP < -42 | dBm |
| SRS_TDCP_115 | -42 ≤ SRS-TDCP < -41 | dBm |
| SRS_TDCP_116 | -41 ≤ SRS-TDCP < -40 | dBm |
| SRS_TDCP_117 | -40 ≤ SRS-TDCP < -39 | dBm |
| SRS_TDCP_118 | -39 ≤ SRS-TDCP < -38 | dBm |
| SRS_TDCP_119 | -38 ≤ SRS-TDCP < -37 | dBm |
| SRS_TDCP_120 | -37 ≤ SRS-TDCP < -36 | dBm |
| SRS_TDCP_121 | -36 ≤ SRS-TDCP < -35 | dBm |
| SRS_TDCP_122 | -35 ≤ SRS-TDCP < -34 | dBm |
| SRS_TDCP_123 | -34 ≤ SRS-TDCP < -33 | dBm |
| SRS_TDCP_124 | -33 ≤ SRS-TDCP < -32 | dBm |
| SRS_TDCP_125 | -32 ≤ SRS-TDCP < -31 | dBm |
| SRS_TDCP_126 | -31 ≤ SRS-TDCP | dBm |

Proposal 2: Table 1 is used to define measurement report mapping for UL SRS-TDCP measurement.

# Summary

In this contribution report mapping for positioning use case 3b is discussed. The following observations and proposals summarize the discussion presented in this contribution.

Observation 1: Depending on the request from LMF, gNB can report path-based measurements to the LMF. For such measurement reporting, no enhancement to Rel. 18 reporting has been introduced.


Observation 2: Depending on the request from LMF, gNB can report sample-based measurements to the LMF. For such measurement reporting, report mapping table needs to be specified in RAN4 specification.

Observation 3: For sample-based reporting, gNB may either report timing measurement or timing and power measurements associated to Nt´ samples of the estimated channel response in time domain.

Observation 4: The minimum number of samples for which the timing or timing and power information are reported is 8 and the maximum number of samples for which the timing or timing and power information are reported is 24.

Proposal 1: Reporting range for UL SRS-TDCP is defined from -156 dBm to -31 dBm with resolution 1 dB.

Proposal 2: Table 1 is used to define measurement report mapping for UL SRS-TDCP measurement.

Table 1. UL SRS-TDCP report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| SRS_TDCP_000 | SRS-TDCP < -156 | dBm |
| SRS_TDCP_001 | -156 ≤ SRS-TDCP < -155 | dBm |
| SRS_TDCP_002 | -155 ≤ SRS-TDCP < -154 | dBm |
| SRS_TDCP_003 | -154 ≤ SRS-TDCP < -153 | dBm |
| SRS_TDCP_004 | -153 ≤ SRS-TDCP < -152 | dBm |
| SRS_TDCP_005 | -152 ≤ SRS-TDCP < -151 | dBm |
| SRS_TDCP_006 | -151 ≤ SRS-TDCP < -150 | dBm |
| SRS_TDCP_007 | -150 ≤ SRS-TDCP < -149 | dBm |
| SRS_TDCP_008 | -149 ≤ SRS-TDCP < -148 | dBm |
| SRS_TDCP_009 | -148 ≤ SRS-TDCP < -147 | dBm |
| SRS_TDCP_010 | -147 ≤ SRS-TDCP < -146 | dBm |
| SRS_TDCP_011 | -146 ≤ SRS-TDCP < -145 | dBm |
| SRS_TDCP_012 | -145 ≤ SRS-TDCP < -144 | dBm |
| SRS_TDCP_013 | -144 ≤ SRS-TDCP < -143 | dBm |
| SRS_TDCP_014 | -143 ≤ SRS-TDCP < -142 | dBm |
| SRS_TDCP_015 | -142 ≤ SRS-TDCP < -141 | dBm |
| SRS_TDCP_016 | -141 ≤ SRS-TDCP < -140 | dBm |
| SRS_TDCP_017 | -140 ≤ SRS-TDCP < -139 | dBm |
| SRS_TDCP_018 | -139 ≤ SRS-TDCP < -138 | dBm |
| … | … | … |
| SRS_TDCP_111 | -46 ≤ SRS-TDCP < -45 | dBm |
| SRS_TDCP_112 | -45 ≤ SRS-TDCP < -44 | dBm |
| SRS_TDCP_113 | -44 ≤ SRS-TDCP < -43 | dBm |
| SRS_TDCP_114 | -43 ≤ SRS-TDCP < -42 | dBm |
| SRS_TDCP_115 | -42 ≤ SRS-TDCP < -41 | dBm |
| SRS_TDCP_116 | -41 ≤ SRS-TDCP < -40 | dBm |
| SRS_TDCP_117 | -40 ≤ SRS-TDCP < -39 | dBm |
| SRS_TDCP_118 | -39 ≤ SRS-TDCP < -38 | dBm |
| SRS_TDCP_119 | -38 ≤ SRS-TDCP < -37 | dBm |
| SRS_TDCP_120 | -37 ≤ SRS-TDCP < -36 | dBm |
| SRS_TDCP_121 | -36 ≤ SRS-TDCP < -35 | dBm |
| SRS_TDCP_122 | -35 ≤ SRS-TDCP < -34 | dBm |
| SRS_TDCP_123 | -34 ≤ SRS-TDCP < -33 | dBm |
| SRS_TDCP_124 | -33 ≤ SRS-TDCP < -32 | dBm |
| SRS_TDCP_125 | -32 ≤ SRS-TDCP < -31 | dBm |
| SRS_TDCP_126 | -31 ≤ SRS-TDCP | dBm |

# References

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm Incorporated.

- R1-2410718, Summary #5 of specification support for positioning accuracy enhancement, Moderator (Ericsson).

- R1-2501414, Summary #5 of specification support for positioning accuracy enhancement, Moderator (Ericsson).

- R1-2403740, Summary #6 of specification support for positioning accuracy enhancement, Moderator (Ericsson).

- TS 38.215, NR Physical layer measurements (Release 19), v 19.0.0.

- R4-2510760, draftCR to 38.133 on report mapping for UL SRS-TDCT and UL SRS-TDCP measurements (Rel. 19), Ericsson.