3GPP TSG-RAN WG4 Meeting # 116 	    R4-2510764

Bengaluru, IN, 25 – 29 August 2025

Agenda Item:	7.19.3.2

Source: 	Ericsson

Title:	On requirements for event prediction for AI/ML based mobility

Document for:	Approval

# Introduction

This contribution discusses impact on RAN4 requirement for predicted event triggered measurement reporting for mobility. FFS items identified in the previous meeting(s) and insights on requirements for measurement reporting triggered by direct prediction of event are particularly discussed.

# Discussion

Requirement for predicted event triggered reporting for mobility was discussed in previous meeting(s). Based on the discussion it was agreed to study both indirect and direct event prediction use cases in RAN4 and if any of the prediction use cases is to be prioritized, then it shall be based on the RAN2 prioritization. Furthermore, potential impact on RRM requirement due to indirect event prediction was also discussed and the need to define delay and accuracy requirement was agreed FFS. Whether RAN4 should define requirement for the predicted RSRP measurements if they are reported as part of the measurement report triggered due to event prediction was also agreed FFS.

Agreement from RAN4#114bis on prioritization of event prediction use cases [1]

- RAN4 should discuss both indirect and direct cases. If any prioritization will be agreed in RAN2, RAN4 to follow RAN2 prioritization.

Agreement from RAN4#114bis on potential RRM requirement impact for indirect event prediction [1]

- Potential RRM requirement impact for event prediction use case - for indirect case:

- RAN4 to study the requirement for the predicted event triggered reporting.

  - RAN4 to study the requirement for [delay and accuracy].

  - RAN4 to discuss the metric for the event prediction.

- Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time],

  - Candidate options:

    - Option 1: RAN4 will define requirement for the predicted RSRPs.

    - Option 2: RAN4 will not define requirement for the predicted RSRPs.

Agreement from RAN4#115 on performance metric for event prediction use case [2]

- In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP.

### Reporting Delay Requirement

In legacy event triggered reporting, UE is configured with the event and the associated condition for measurement reporting. The UE continuously monitors if the event condition is met. When the event condition is met, UE starts TTT timer and starts the measurement reporting to the network over the air interface given the event condition persists until the TTT timer expires. The measurement reporting delay in this case is defined as the time between the occurrence of an event and the point in time when the UE starts to transmit the measurement report over the air interface to the network. An example timeline of legacy event triggered reporting is shown in Figure 1.

Figure 1. Elaboration of legacy event triggered reporting.

For indirect event prediction, the UE is expected to perform the prediction of an event by exploiting the RRM measurements predicted in temporal domain. The UE predicts the time when the condition for the event occurrence will potentially be met and whether the event triggered reporting condition persists until the TTT timer expires. The event condition holding true until the expiry of TTT timer is based on predictions, such that the event and event condition holding true until the expiry of the TTT timer is evaluated in advance by the UE before the actual event condition is met and the event condition has persisted until the expiry of the TTT timer. Based on the prediction, if the UE determines that the prediction of the event condition holds true until the expiry of the TTT timer, UE reports the measurement to the network. Unlike in the legacy event triggered reporting, reporting of measurements based on the event prediction, in principle, occurs earlier than the actual reporting of the measurements based on the occurrence of the actual event. In this regard, the reporting delay requirement defined for the legacy event triggered reporting does not hold true for the event prediction cases.

Observation 1: Reporting delay requirement defined for the legacy event triggered reporting may not hold true for the predicted event triggered reporting.

Proposal 1: RAN4 to consider impact on reporting delay requirement due to predicted event triggered reporting for indirect event prediction use case.

### Accuracy Requirement

Event triggered reporting was introduced for mobility in NR to allow the network to perceive real-time radio conditions at UE and the measurement reporting is only triggered by the UE upon occurrence of an event, thus reducing the signaling overhead while RRM measurement is reported by the UE. This principle needs to be kept intact when AI/ML model is used by the UE to predict event or is using the predicted RRM measurement to determine likelihood of event occurrence in the future time instances. Measurement reporting based on predicted event should not result into additional signaling overhead compared to the signaling for legacy event triggered based measurement reporting such that the measurement reporting based on event prediction does not result into the degradation of HO performance. For example, false detection of an event may result into additional signaling and missed detection of an event may result into degradation in HO performance.

To ensure additional signaling overhead is not introduced and HO performance is not degraded due to the measurement reporting based on the event prediction, relevant performance requirement for predicted event triggered reporting needs to be discussed in RAN4. The accuracy of indirect event prediction relies on the accuracy of temporal prediction of RRM measurements. Potentially no new or additional requirement, other than the accuracy of the temporal domain prediction of RRM measurement, needs to be defined for the indirect event prediction use case.

Unlike indirect event prediction use case, AI/ML model deployed at the UE may directly predict the occurrence of an event either by providing the measured RRM measurements or predicted RRM measurements to its AI/ML model as input in direct event prediction use case. During the study item a small number of companies in RAN2 supplemented their contributions with the evaluation results for direct event prediction use case and therefore in the best interest of time, RAN4 should start a detail discussion on relevant requirements for the direct event prediction use case provided that the direct event prediction is supported by the framework designed by RAN2 for AI/ML based mobility.

Observation 2: Temporal domain prediction of RRM measurements is used for indirect event prediction.

Observation 3: Accuracy of temporal domain prediction has an impact on the accuracy indirect event prediction.

Proposal 2: RAN4 to reuse accuracy requirement for temporal prediction of RRM measurement for indirect event prediction.

Proposal 3: RAN4 to consider defining requirement for direct event prediction provided that the reporting framework for direct event prediction triggered reporting is agreed to be supported by RAN2.

# Summary

In this paper FFS items identified in the previous meeting are discussed and insights on requirements for measurement reporting triggered by direct prediction of event are provided. The observations and proposals below summarize the discussion presented in this paper.

Observation 1: Reporting delay requirement defined for the legacy event triggered reporting may not hold true for the predicted event triggered reporting.

Proposal 1: RAN4 to consider impact on reporting delay requirement due to predicted event triggered reporting for indirect event prediction use case.

Observation 2: Temporal domain prediction of RRM measurements is used for indirect event prediction.

Observation 3: Accuracy of temporal domain prediction has an impact on the accuracy indirect event prediction.

Proposal 2: RAN4 to reuse accuracy requirement for temporal prediction of RRM measurement for indirect event prediction.

Proposal 3: RAN4 to consider defining requirement for direct event prediction provided that the reporting framework for direct event prediction triggered reporting is agreed to be supported by RAN2.

# References

- R4-2504901, WF on AI/ML Mobility, Moderator (Nokia).

- R4- 2508268, WF on AI/ML Mobility Part 1, Moderator (Nokia).