3GPP TSG-RAN WG4 Meeting #116				R4-2510779

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	8

Source: 	CAICT

Title: 	Rel-19 RAN4 UE feature list for NR_AIML_air

Document for:	Discussion

- Discussion

In RAN4 115, the following agreement is UE feature related:

Issue 2-3: Rx beam knowledge and TCI handling

Agreement:

- if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known.

- If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, depending on UE capability, the corresponding Rx beam may be known or unknown.

- Known Rx beam means no additional Rx beam sweeping is needed

- Unknown Rx beam means additional Rx beam sweeping is needed.

- Rx beam is known mean TCI state is known if conditions below are also met

Based on the current progress, the UE feature related shall be involved in the Rel-19 UE feature list, we provide the detail information in the following table:

58. NR_AIML_air

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | [58-1-x] | UE-side beam prediction for BM Case1 | [Knows RX beam of predicted TX beam in setA in BM-case 1 if the predicted Tx beam is not QCL Type-D to a known TX beam] | Pre-requisitng FG: 58-1-2 (RAN1 feature for BM-case-1) | yes | N/A | [In BM-case 1, network needs to transmit additional samples of reference signal corresponding to the predicted TX beam of setA so that UE can sweep its RX beams during TCI state switch process when the RS resource for predicted Tx beam is the RS in target TCI state or QCLed to target TCI state.] | [Per Band] | [TDD] | [FR2-1 only] | N/A |  | [Optional with capability signaling] |
|  | [58-1-y] | UE-side beam prediction for BM Case2 | [Knows RX beam of predicted TX beam in setA in BM-case-2 if the predicted Tx beam is not QCL Type-D to a known TX beam] | Pre-requisitng FG: 58-1-4 (RAN1 feature for BM-case-2) | yes | N/A | In BM-case 2, network needs to transmit additional samples of reference signal corresponding to the predicted TX beam of setA so that UE can sweep its RX beams during TCI state switch process when the RS resource for predicted Tx beam is the RS in target TCI state or QCLed to target TCI state. | [Per Band] | [TDD] | [FR2-1 only] | N/A |  | [Optional with capability signaling] |

Proposal 1: Input the Rel-19 NR_AIML_air UE feature to Rel-19 UE feature list.

- Conclusion

- Proposal 1: Input the Rel-19 NR_AIML_air UE feature to Rel-19 UE feature list.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | [58-1-x] | UE-side beam prediction for BM Case1 | [Knows RX beam of predicted TX beam in setA in BM-case 1 if the predicted Tx beam is not QCL Type-D to a known TX beam] | Pre-requisitng FG: 58-1-2 (RAN1 feature for BM-case-1) | yes | N/A | [In BM-case 1, network needs to transmit additional samples of reference signal corresponding to the predicted TX beam of setA so that UE can sweep its RX beams during TCI state switch process when the RS resource for predicted Tx beam is the RS in target TCI state or QCLed to target TCI state.] | [Per Band] | [TDD] | [FR2-1 only] | N/A |  | [Optional with capability signaling] |
|  | [58-1-y] | UE-side beam prediction for BM Case2 | [Knows RX beam of predicted TX beam in setA in BM-case-2 if the predicted Tx beam is not QCL Type-D to a known TX beam] | Pre-requisitng FG: 58-1-4 (RAN1 feature for BM-case-2) | yes | N/A | [In BM-case 2, network needs to transmit additional samples of reference signal corresponding to the predicted TX beam of setA so that UE can sweep its RX beams during TCI state switch process when the RS resource for predicted Tx beam is the RS in target TCI state or QCLed to target TCI state.] | [Per Band] | [TDD] | [FR2-1 only] | N/A |  | [Optional with capability signaling] |

- Reference

[1] R4-2508080	WF on NR_AIML_air_part1, Qualcomm