3GPP TSG-RAN WG4 Meeting #116	R4- 2510787

# Bangalore, India,  August 25 - 29, 2025

Agenda Item:	7.27.3.1

Source:	MediaTek Inc.

Title:	Discussion on Rel-19 NR-NTN RedCap UE RF requirements

Document for:	Approval

- Introduction

In RAN#102 plenary meeting, the Rel-19 WID of Non-Terrestrial Networks (NTN) for NR Phase 3 was approved in [1]. One of the main objectives is to support Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN band.

In RAN4#114bis meeting, the WF [2] for guiding further discussion on residual issues was proposed. In this contribution, our views on the residual issues are provided.

- Discussion

2.1		Guidance for Rel-19 phase 3 NR-NTN UE RF requirements

The Rel-19 WID [1] objectives indicate that NTN for NR Phase 3 supports Redcap and eRedcap with NR NTN operating in FR1-NTN band. The objectives are shown below.

4.1 Objective of SI or Core part WI or Testing part WI Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1] - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4] Notes for this objective: GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

2.2		Previous Meeting Agreements

In RAN4#115, the approved WF [2] provided the guidance for following works. The partial content is listed below for reference.

Topic 1: Output power enhancement for NR NTN PC3 Agreements: - Define new UE capabilities for UE to indicate support for the additional power boosting per band and network to enable the boosting - FFS for details, capabilities will be needed for feature list - When UE indicates support for boosting and network enables the boosting, UE reference power is increased, i.e. both Pcmax_L and Pcmax_H are increased - MPR is not changed for DFT-s-OFDM and QPSK and pi/2 BPSK, [all RB allocations] - For CP-OFDM and DFT-s-OFDM 16QAM and 64QAM, boosting is allowed by UE implementation and MPR is correspondingly increased by the amount of the boost - Boost is 0.5 dB - FFS on duplex modes - For information only: HD-FDD/FD-FDD support is indicated per band - This means that UE has option to e.g. be HD-FDD in band A and support boosting and be FD-FDD in band B and not support boosting - Rel-18 and Rel-19 power boosting schemes can be simultaneously supported - FFS for details - PC3 ACLR requirements apply for power boosted transmissions - Configured maximum output power and MPR updates related to power boosting need to be captured to TS 38.101-5

2.3		PC3 output power for (e)RedCap

Proposal 1: When UE indicates support for output power enhancement for NTN RedCap PC3, MPR is not changed for DFT-s-OFDM and QPSK and pi/2 BPSK with deciding the final option for RB allocations.

- Option1: All RB allocations

- Option2: Outer and Inner RB allocations

Observation 1: In WF [2], it was agreed that Rel-18 and Rel-19 power boosting schemes can be simultaneously supported. However, it is found that Rel-18 and Rel-19 power boosting may be applied to different RB allocations.

Observation 2: The Rel-18 NR-TN power boosting indicates the RB allocation belongs to the inner region defined in clause TS 38.101-1 6.2.2.

Proposal 2: Discuss the combined Rel-18 and Rel-19 power enhancement values for different RB allocations.

| Modulation: QPSK and pi/2 BPSK | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| --- | --- | --- | --- |
| Rel-18 NR Tx Pout enhancement | 0dB | 0dB | 1dB |
| Rel-19 NR-NTN Tx Pout enhancement | [0.5dB] | 0.5dB | 0.5dB |
| Total Tx Pout enhancement | [0.5dB] | 0.5dB | 1.5dB |

Proposal 3: Regarding “Output power enhancement for NR NTN PC3”, discuss the feature list below.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD diff. | Need of FR1/FR2 diff. | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62-1 | Tx PC3 output power enhancement for NR-NTN UE | Indicate the supported Tx output power enhancement for NR-NTN UE |  | Yes | N/A | NTN Legacy output power limitation still apply | Per band: | No | FR1 only | N/A | The requirements for output power enhancement are defined in clause 6.2.4 in TS 38.101-5. | Optional with capability signalling |

3 Conclusion

In this paper, our views on NTN-RedCap UE RF requirements are provided as follows:

Proposal 1: When UE indicates support for output power enhancement for NTN RedCap PC3, MPR is not changed for DFT-s-OFDM and QPSK and pi/2 BPSK with deciding the final option for RB allocations.

- Option1: All RB allocations

- Option2: Outer and Inner RB allocations

Observation 1: In WF [2], it was agreed that Rel-18 and Rel-19 power boosting schemes can be simultaneously supported. However, it is found that Rel-18 and Rel-19 power boosting may be applied to different RB allocations.

Observation 2: The Rel-18 NR-TN power boosting indicates the RB allocation belongs to the inner region defined in clause TS 38.101-1 6.2.2.

Proposal 2: Discuss the combined Rel-18 and Rel-19 power enhancement values for different RB allocations.

| Modulation: QPSK and pi/2 BPSK | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| --- | --- | --- | --- |
| Rel-18 NR Tx Pout enhancement | 0dB | 0dB | 1dB |
| Rel-19 NR-NTN Tx Pout enhancement | [0.5dB] | 0.5dB | 0.5dB |
| Total Tx Pout enhancement | [0.5dB] | 0.5dB | 1.5dB |

Proposal 3: Regarding “Output power enhancement for NR NTN PC3”, discuss the feature list below.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD diff. | Need of FR1/FR2 diff. | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62-1 | Tx PC3 output power enhancement for NR-NTN UE | Indicate the supported Tx output power enhancement for NR-NTN UE |  | Yes | N/A | NTN Legacy output power limitation still apply | Per band: | No | FR1 only | N/A | The requirements for output power enhancement are defined in clause 6.2.4 in TS 38.101-5. | Optional with capability signalling |

4 Reference

- RP-234078, “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”

- R4-2508764, “Way Forward for [115][309] NR_NTN_Ph3_General_UE_SAN_RF”, Qualcomm