3GPP TSG-RAN WG4 Meeting #116	   R4-2510789

# Bangalore, India, 25th – 29th , August , 2025

Agenda Item:	7.14.6

Source: 	Ericsson

Title:	On performance requirement for Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

# Introduction

From RAN #103 meeting the WID for RRM enhancement phase 5 is listed in [1]. The RAN4#114 bis WF is listed in [2].  In this paper we provide our view for the remaining open issues.

Background: WID objective #2

- Fast SCell activation for UE supporting Rel-18 EMR

- Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting

- Apply fast scell activation in FR1 and FR2-1

- Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

Issue 2-2-1: Test case list Agreement: - Scenario: - Normal SCell activation for FR1 and FR2 - Direct SCell activation at SCell addition for FR1 - PUCCH SCell activation for FR1 and FR2 - FFS: - Direct SCell activation at SCell addition for FR2 - UE capabilities and network configurations: - Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured - Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured - FFS: - Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

The Rel-19 fast scell activation enhancement is based on the motivation that a valid EMR report can replace a normal L3 measurement report for unknown target cell. This will further enhance the unknown target cell activation requirement.

Due to the beam sweeping for FR2, any unknown target cell for activation at FR2 takes particularly long time. We see the importance of having the FR2 scenario being included since it is high gain from UE sending a valid EMR report in FR2 to save the cell search time regardless of activation signaling is MAC CE based or RRC based.

- Proposal 1: Direct Scell activation at Scell addition for FR2 shall be considered as one of the test scenarios.

From UE capabilities and network configuration point of view, the rel-19 activation requirement will be based on a new Rel-19 UE capability. The prerequisite of the Rel-19 capability is the UE either support Rel-18 or Rel-16 EMR capabilities.

The enhancement of the Rel-18 capability is:

- Supporting the cell reselection measurement

- Measurement results validation methods

For UE supporting Rel-16 capability only, when the timer T331 does not expire at the transition moment which indicates that the UE have continuous measurement and maintain relatively good sync with the carriers being configured, this is a valid case for the activation requirement enhancement. We think it is reasonable to support this configuration.

- Proposal 2: Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode shall be considered as one of the test configuration.

Issue 2-2-2: Details of test case Issue 2-2-3: Applicability rules of test case

From the UE procedure aspect, performance evaluation begins when the UE receives the measurement configuration for EMR measurement and concludes when UE sent the CSI report for a successful Scell activation. Details can be seen from figure 1.

Figure 1 signal flow of the Fast scell activation for UE supporting Rel-18 EMR

From a performance perspective, if the UE sends a valid measurement report 5 seconds prior to receiving the Scell activation command, the target cell is considered known. The known condition will apply, and specific delay activation requirements will be verified during the test. Conversely, if the UE does not send a valid measurement report before the activation command, the target cell is treated as unknown, and the unknown activation requirements will be verified during the test case.

From a test case timeline perspective, fast Scell activation for UEs supporting Rel-18 EMR requires a combination of two existing test cases, as illustrated in Figure 2.

Figure 2. Test case timelines to combine existing test cases.

- Observation 1: The EMR/Cell reselection test case and the Scell activation test case follow relatively independent timelines.

Since the two feature sets follow independent timelines, the performance of Rel-19 fast Scell activation for UEs supporting Rel-18 EMR can be evaluated using a configuration that combines the two relevant test cases. This is feasible because there is no overlap in test behavior or verification requirements between them.

If the EMR or cell reselection behavior under verification involves the UE sending a valid measurement report, then the known Scell activation test cases should be used to evaluate the activation requirements.
Conversely, if the UE behavior does not include sending a valid measurement report, the unknown Scell activation test cases should be used for evaluation.

Based on different EMR reporting UE behavior and the Scell known or unknown condition listed in Table 1. We think the test case can be configured by combining the two test cases and following the suggested timeline from figure.2 The EMR report will enhance the unknown target cell activation, so the case can be verified from different test configuration is that when there is an EMR report being sent combined with unknown target cell activation.

The test configuration can be seen from table.1

|  | Scenario | Test case configuration |  | Expected test outcome |
| --- | --- | --- | --- | --- |
| Rel-19 fast Scell activation FR1 | EMR UE report + unknown Scell activation | A6.6.9.1 | A6.5.3. 3 | Activation delay will be enhanced. |
| Rel-19 fast Scell activation FR1 | EMR UE report + unknown PUCCH Scell activation/deactivation | A6.6.9.1 | A6.5.3.7 | Activation delay will be enhanced. |
| Rel-19 fast Scell activation FR1 | Cell reselection UE report + unknown Scell activation | A6.6.9.4 | A6.5.3. 3 | Activation delay will be enhanced. |
| Rel-19 fast Scell activation FR1 | Cell reselection UE report + unknown PUCCH Scell activation/deactivation | A6.6.9.4 | A6.5.3.7 | Activation delay will be enhanced. |
| Rel-19 fast Scell activation FR2 | Cell reselection UE report + unknown Scell activation | A7.6.23.3 | A5.5.3.5 | Activation delay will be enhanced. |
| Rel-19 fast Scell activation FR2 | Cell reselection UE report + unknown PUCCH Scell activation/deactivation | A7.6.23.3 | A5.5.3.10 | Activation delay will be enhanced. |

- Proposal 3: The performance of Rel-19 fast Scell activation for UEs supporting Rel-18 EMR can be evaluated by combining the existing Scell activation test case with the Idle state fast CA/DC reporting test case such as take table.1 as test configuration.

# Conclusion

	Proposal 1: Direct Scell activation at Scell addition for FR2 shall be considered as one of the test scenarios.

	Proposal 2: Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode shall be considered as one of the test configuration.

	Proposal 3: The performance of Rel-19 fast Scell activation for UEs supporting Rel-18 EMR can be evaluated by combining the existing Scell activation test case with the Idle state fast CA/DC reporting test case such as take table.1 as test configuration.

# References

- RP-240830 “New WID: WI resulting from discussion on RRM enhancements”, VIVO, March 2024

- R4-2508266, WF for NR_RRM_Ph5_part 2, CATT