3GPP TSG-RAN WG4 Meeting #116	   R4-2510793

# Bangalore, India, 25th – 29th , August , 2025

Agenda Item:	7.21.5

Source: 	Ericsson

Title:	Discussion on RRM performance requirements for SBFD

Document for:	Discussion

# Introduction

In this paper we provide our view on RRM performance requirement within related WID [1] objectives:

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

# Discussion

#### Issue 3-2-1: L1-SRS-RSRP – side condition on time offset - Proposals - Proposal 1 (Apple, Samsung, CATT, Nokia): - Time offset condition is defined without cell phase error. - Proposal 2 (CMCC, HW): - For 15kHz SCS and 30kHz SCS, time offset condition is defined with cell phase error - For SCS larger than or equal to 60kHz, time offset condition is defined without cell phase error - Recommended WF - Discuss the 2 options based on P1 and P2.

RAN1 LS reply for which scenario is supported for L1-CLI measurement implies both intra-cell and inter-cell scneario are supported but intra-cell scneario is considered to be prioritized. Regarding whether to account the 3us cell phase error into the accuracy requirement, for inter-cell scenario, this cell phase error should be considered.

Based on the analysis from the simulation results R4-2508307, we see different degradation of SRS-RSRP accuracy for different SCS between case 1 (with cell phase error) and case2 (without cell phase error).

When SCS is 15KHz or 30KHz the difference between cas1 and case 2 for both AWGN and TDL-C channels are relatively small, it is around 1dB.

When the SCS is larger than 60KHz. For FR1 and FR2 with SCS 60 KHz, the difference between case 1 and case 2 can be around 2dB for AWGN and TDL-C channel. For FR2 with 120KHz SCS, the difference between case1 and case 2 can be around 4-5 dB.

Eventhough it will be simpler to have 1 set of requirement cover all the scenario, however, we do not see it is possible with the simulation results.

- Proposal 1: For 15kHz SCS and 30kHz SCS, time offset condition is defined with cell phase error. For SCS larger than or equal to 60kHz, time offset condition is defined without cell phase error

#### Issue 3-2-2: L1-SRS-RSRP – side condition on SRS BW #### Issue 3-2-3: L1-SRS-RSRP – side condition on SRS Es/Iot

From network total performance point of view, the side condition not only impacts the accuracy requirement however also impact the requirement coverage. Due to the flexible configuration of the SBFD for DUD and DU, the BW of the UL subband needs to be declared by the base station. Based on RF session agreement and existing RB configuration table from TS 38.104 table 5.3.2-1, we see there is case that the SRS BW cannot be guaranteed for more than 48 RBs for FR1.

Based on the simulation results that submitted by all companies, we think the SNR of the SRS can be set at 1dB without degrading the accuracy.

- Proposal 2: The side condition on SRS BW for L1-SRS-RSRP shall be based on ≥24RBs and the SRS Es/Iot  can be set to ≥1dB.

#### Issue 3-2-4: L1-SRS-RSRP – accuracy requirements

Based on RAN1 answer on the LS, the intra-cell CLI scenario is being prioritized. However from setting requirement point of view, due to the legacy Rel-16 CLI requirements was being set for both intra and inter cell scenario. We think we have set both requirement will not do harm to the specification.

The requirement should be based on the agreed side condition and simulation results accordingly.

- Proposal 3: Define L1-SRS-RSRP accuracy requirements based on simulation results for the agreed side conditions.

#### Issue 3-2-5: L1-CLI-RSSI – accuracy requirements

When comparing the L1-CLI-RSSI with the rel-16 L3-CLI-RSSI, it is both singel sample measurement. Even the resource typle configuration for L1-CLI-RSSI is more flexible, however it is still within the DL active BWP. Since all L1-RSSI measurement was agreed to be based on 1 sample one shot, we don’t see the need to differentiate requirement for different resource type and methods.

- Proposal 4: Re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and resource type#2, as well as for Method#3.

#### Issue 3-3-1: Absolute report mapping

For the L1-SRS-RSRP and L1-CLI-RSSI, the absolute report mapping will not be diverge from the value with legacy, we think the Rel-16 CLI report value can be reused.

- Proposal 5: Re-use Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1 for reporting mapping of absolute L1-SRS-RSRP and L1-CLI-RSSI, respectively.

#### Issue 3-3-2: Differential report mapping

Rel-16 CLI have also introduce the differential value for SRS-RSRP and CLI-RSSI, we think same table can be reused for differential report mapping.

- Proposal 6: Use Table 2 and Table 3 (based on Table 10.1.6.1-2) for reporting mapping of differential L1-SRS-RSRP and L1-CLI-RSSI, respectively

### Sub-topic 3-4: Test cases #### Issue 3-4-1: General test setup #### Issue 3-4-2: Test case list #### Issue 3-4-3: Test case details and test purpose #### Issue 3-4-4: Test case scenarios

Scenarios

The Rel-19 SBFD RRM core requirement mainly focus on two objectives, the L1-CLI handling mechanism and the RRM core requirement for SBFD operation.

The L1-CLI measurement due to SBFD operation have similarity with the Rel-16 L3-CLI measurement from measurement procedure point of the view. However certain differences can be observed for the L1-CLI measurement for SBFD.

The L1-CLI measurement resource can be configured with either DL subband or UL subband. While the L3-CLI measurement resource was configured with the active BWP.

The measurement timing and measurement period for L1-CLI measurement will be different from the Rel-16 L3-CLI measurement. On top of that the scheduling restriction is also different for L1-CLI-RSSI measurement.

The L1-SRS-RSRP scheduling restriction was specified based on different SCS, different number of symbol will be restricted before the SRS resource.

The legacy L3-CLI measurement test case has been specified both in FR1 and FR2, considering SA and DC scenario. While for SBFD, the current working assumption is that the SBFD operation only with single cell.

Also to mention the Legacy L3-CLI measurement test case combined the scheduling restriction within the test case for different SCS.

- Proposal 7: The SBFD RRM performance requirement for L1-CLI handling and other RRM measurement impact due to SBFD operation need to take consideration for both FR1 and FR2 for single cell operation.

- Proposal 8: RAN4 further discuss how to specify L1-CLI measurement test case such as potential re-use of the L3-CLI measurement test configuration.

Test methodology and configuration

The SBFD operation impact on the RRM measurement requirement mostly reflected on the CSI-RS based L1-RSRP/SINR measurement, the RLM/BFD/CBD measurement and the L3 RRM measurement due to the fundamental difference between a SBFD symbol and non-SBFD symbol.

Currently the impact on CSI-RS based L1-RSRP/SINR measurement requirement is being discussed for the core requirement, however the measurement requirement impact is most likely on the measurement accuracy and measurement period due to the minimum number of CSI-RS resource will be impacted due to punctuation of CSI-RS resources. Another impact is that the UE will perform the CSI-RS based measurement during SBFD operation based on network indication of valid symbol type.

- Proposal 9: RAN4 further discuss how to specify CSI-RS based L1-RSRP/SINR measurement test case due to the CSI-RS resource punctuation and the valid symbol type indication.

Regarding the RLM/BFD/CBD measurement, the impact due to CSI-RS punctuation is quite similar as the L1-RSRP/SINR, the difference is that the valid symbol type indication is not applicable to the RLM/BFD/CBD measurement.

The legacy RLM/BFD test case have been specified for both SA and DC scenario, for SBFD the working assumption for Rel-19 is considering SBFD operation for single cell.

The legacy test procedure for Out of sync and In sync for RLM/BFD can be reused, the details for the test requirement for how long the time duration should be and how UE should behave due to SBFD impact on CSI-RS resources needs to be further discussed when the core requirement is stable.

- Proposal 10: RAN4 further discuss how to specify CSI-RS based RLM/BFD/CBD test case and test requirement due to the CSI-RS resource punctuation for SBFD symbols.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: For 15kHz SCS and 30kHz SCS, time offset condition is defined with cell phase error. For SCS larger than or equal to 60kHz, time offset condition is defined without cell phase error

	Proposal 2: The side condition on SRS BW for L1-SRS-RSRP shall be based on ≥24RBs and the SRS Es/Iot  can be set to ≥1dB.

	Proposal 3: Define L1-SRS-RSRP accuracy requirements based on simulation results for the agreed side conditions.

	Proposal 4: Re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and resource type#2, as well as for Method#3.

	Proposal 5: Re-use Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1 for reporting mapping of absolute L1-SRS-RSRP and L1-CLI-RSSI, respectively.

	Proposal 6: Use Table 2 and Table 3 (based on Table 10.1.6.1-2) for reporting mapping of differential L1-SRS-RSRP and L1-CLI-RSSI, respectively

	Proposal 7: The SBFD RRM performance requirement for L1-CLI handling and other RRM measurement impact due to SBFD operation need to take consideration for both FR1 and FR2 for single cell operation.

	Proposal 8: RAN4 further discuss how to specify L1-CLI measurement test case such as potential re-use of the L3-CLI measurement test configuration.

	Proposal 9: RAN4 further discuss how to specify CSI-RS based L1-RSRP/SINR measurement test case due to the CSI-RS resource punctuation and the valid symbol type indication.

	Proposal 10: RAN4 further discuss how to specify CSI-RS based RLM/BFD/CBD test case and test requirement due to the CSI-RS resource punctuation for SBFD symbols.

# References

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June ,2024

- R4-2508274 ‘WF on RRM requirements for NR_Duplex_evo’, Huawei, May,2025