3GPP TSG-RAN WG4 Meeting #116																		R4-2510805

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.17.3

Source: 	OPPO

Title: 	CSI reporting requirement and testing framework for CSI prediction

Document for:	Approval

- Discussion

In RAN4#114bis meeting, following agreements have been made,

Codebook to be used in the tests

In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined

Use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions

Requirement baseline

Requirement to be introduced for Dopler domain basis N4=1. N4>1 can be further discussed in the future if needed

Test metric

Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4)

- tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/MLusing the precoders configured according to the UE reports,

- trnd is the throughput measured at SNRue_AI/ML with random precoding.

  - trnd is obtained with Rel-15 type I single panel codebook

- Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4

In RAN4#115 meeting, following agreements have been made,

Requirement considerations

The performance of AI based CSI prediction should not be worse than R16 etypeII.

- The exact test procedure and configurations are FFS

- The impact of different timeline between AI and R16 etype II should be also carefully analysed

For γAI/ML requirement, in terms of the relationship to legacy requirements, what we define in RAN4 is the minimum performance requirement, which is not to verify how much performance gain an AI/ML function/model can bring. The assessment of performance gain should be considered when RAN1 introduces and evaluates CSI features, rather than when RAN4 defines test cases.

In the last meeting, it has been agreed that the performance of AI based CSI prediction should not be worse than R16 etypeII. Clarification is needed regarding the “not be worse than R16 eTypeII”. Does this mean: Case a: Compared to performing CSI measurement and reporting using R16 eTypeII at T1  (with “sample and hold”, i.e., the reported result is applied at T2), the performance of AI/ML-based CSI prediction should not be worse? Or Case b: Compared to performing CSI measurement and reporting using R16 eTypeII at time T2  (the reported result is applied at T2), the performance of AI/ML-based CSI prediction should not be worse? Our understanding is Case a, because:

- It would be hard to require CSI prediction for T2 to outperform actual CSI measurement performed at T2.

- Examining R18 eTypeII codebook requirements for predicted PMI (=1.8) also reveals they are not stricter than those for R16 eTypeII codebook(=2.2).

Therefore, for AI-based CSI prediction performance:

- When compared to R16 eTypeII using 'sample and hold' reporting, it should not be worse than R16 eTypeII.

- When compared to R18 eTypeII codebook for predicted PMI, it should not be worse than R18 eTypeII codebook.

Proposal 1: For AI-based CSI prediction performance:

- When compared to R16 eTypeII using 'sample and hold' reporting, it should not be worse than R16 eTypeII.

- When compared to R18 eTypeII codebook for predicted PMI, it should not be worse than R18 eTypeII codebook.

As a minimum requirement for RAN4 tests, for 20Hz Doppler spread, we could follow the minimum requirement that defined for “Enhanced Type II codebook for predicted PMI”, i.e., Table 6.3.2.1.8-2 in 38.101-4. For other values, e.g., 50Hz Doppler spread, we can define a new value based on further evaluations.

Proposal 2: Reuse the legacy PMI requirement (γ=1.8 for Enhanced Type II codebook for predicted PMI @ 20Hz Doppler spread), FFS γ value for 50Hz Doppler spread.

Minimum requirement for AI/ML based CSI prediction

| Parameter | Test 1 (20Hz Doppler spread) | Test 2 (50Hz Doppler spread) |
| --- | --- | --- |
| γAI/ML | 1.8 | TBD |

For  γAI/ML calculation, it could be obtained within a CSI prediction window. If a CSI prediction window contains more than one CSI prediction instance, γAI/ML could be (1) calculated for each predicted CSI and then do the average within the CSI prediction window, or (2)  tue_AI/ML and trnd could be calculated within the CSI prediction window first and then get the averaged γAI/ML value.

Proposal 3: γAI/ML could be obtained within a CSI prediction window. If a CSI prediction window contains more than one CSI prediction instance,

Option1: γAI/ML is calculated for each predicted CSI and then do the average within the CSI prediction window

Option2: tue_AI/ML and trnd are calculated within the CSI prediction window first and then get the averaged γAI/ML value.

Figure 1 Reference block diagram for 1-sided model

Figure 1 provides the reference block diagram for 1-sided model as agreed previously. For CSI prediction, following steps could be considered:

- Step1: TE sends CSI-RS to DUT, within a CSI observation window (signal generating and transmitting)

- Step2: DUT conducts the CSI measurement within the CSI observation window (CSI-RS receiving)

- Step3: DUT conducts the CSI prediction within a CSI prediction window (Inference)

- Step4: DUT reports the predicted CSI (CSI reporting)

- Step5: TE calculates the γAI/ML for performance [and calculates the measurement/inference latency for LCM]

Proposal 4: For CSI prediction, following test steps could be considered:

- Step1: TE sends CSI-RS to DUT, within a CSI observation window (signal generating and transmitting)

- Step2: DUT conducts the CSI measurement within the CSI observation window (CSI-RS receiving)

- Step3: DUT conducts the CSI prediction within a CSI prediction window (Inference)

- Step4: DUT reports the predicted CSI (CSI reporting)

- Step5: TE calculates the γAI/ML for performance [and calculates the measurement/inference latency for LCM]

- Conclusions

In this contribution, we have discussed the CSI reporting requirement and testing framework for CSI prediction, and got the following proposals:

Proposal 1: For AI-based CSI prediction performance:

- When compared to R16 eTypeII using 'sample and hold' reporting, it should not be worse than R16 eTypeII.

- When compared to R18 eTypeII codebook for predicted PMI, it should not be worse than R18 eTypeII codebook.

Proposal 2: Reuse the legacy PMI requirement (γ=1.8 for Enhanced Type II codebook for predicted PMI @ 20Hz Doppler spread), FFS γ value for 50Hz Doppler spread.

Minimum requirement for AI/ML based CSI prediction

| Parameter | Test 1 (20Hz Doppler spread) | Test 2 (50Hz Doppler spread) |
| --- | --- | --- |
| γAI/ML | 1.8 | TBD |

Proposal 3: γAI/ML could be obtained within a CSI prediction window. If a CSI prediction window contains more than one CSI prediction instance,

Option1: γAI/ML is calculated for each predicted CSI and then do the average within the CSI prediction window

Option2: tue_AI/ML and trnd are calculated within the CSI prediction window first and then get the averaged γAI/ML value.

Proposal 4: For CSI prediction, following test steps could be considered:

- Step1: TE sends CSI-RS to DUT, within a CSI observation window (signal generating and transmitting)

- Step2: DUT conducts the CSI measurement within the CSI observation window (CSI-RS receiving)

- Step3: DUT conducts the CSI prediction within a CSI prediction window (Inference)

- Step4: DUT reports the predicted CSI (CSI reporting)

- Step5: TE calculates the γAI/ML for performance [and calculates the measurement/inference latency for LCM]

- References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-250308, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2