3GPP TSG-RAN WG4 Meeting #116                                                          R4-2510820

Bengaluru, India, 25th – 29th August, 2025

Source: 	NTT DOCOMO, INC., KDDI

Title:	Discussion on how to capture the modifiedMPR-behaviour for n1

Agenda item:	4.2.1

Document for:	Approval

- Introduction

Due to changes in the requirements for protecting PHS from n1, reevaluating A-MPR values for n1 is currently discussed. RAN4 agreed that it can use modifiedMPR-behaviour and reuse NS_05 and NS_05U instead of introducing new NS to introduce new smaller A-MPR for protecting DECT. Companies may have different understandings of how to describe modifiedMPR-behaviour in the specifications of each release. Therefore, this paper shares our views to clarify these.

- Background

Based on regulatory changes in Japan, it is necessary to protect DECT instead of PHS for n1 newly. The requirements for protection are relaxed, so RAN4 is currently discussing reevaluating A-MPR values for n1. Regarding NS value to be used to indicate the new A-MPR requirements, the agreement at RAN4#113 is captured in WF [1] as follows.

< Issue 1-1-3: NS value to be used to indicate the new A-MPR requirements > Way Forward: - Introduce new A-MPR in Rel-19 spec and reuse NS_05/NS_05U instead of introducing new NS. UE can support it as optional before Rel-19.

NS_05 and NS_05U have been introduced since Rel-15 for protecting PHS from n1. Based on the above agreement, RAN4 can use modifiedMPR-behaviour and reuse NS_05 and NS_05U instead of introducing new NS to introduce new smaller A-MPR for protecting DECT. However, although it was agreed at the previous meeting that "may" would be used to indicate optional support from Rel-15 to Rel-18, the decision of whether to use "shall" to indicate mandatory support in Rel-19, and the detailed wording, was not finalized [2].

- 1.5 modifiedMPR-Behaviour < Issue 1-1-9: How to capture the modifiedMPR-behaviour > Agreement: - For Rel-15 to Rel-18 specs, the modifiedMPR bit may be set to 1 to indicate the application of newly defined A-MPR values for NS_05 and NS_05U in R19 - [for Rel-19 the modifiedMPR bit shall be set to 1.] - The detailed wording can be further discussed over CR revision

Therefore, clarifying these points will help reuse existing NS and introduce the new smaller A-MPR for n1.

- Discussion

As organized in our contribution [3], when referencing the TS38101-1 specifications for each release, it is noted that in the modifiedMPR Table defining the field modifiedMPR-behaviour, "shall" is used when it is mandatory to meet the new MPR requirement, while "may" is used when it is                                                               optional. As an example, the following is excerpt from the modifiedMPR table in TS38.101-1 v18.8.0.

Table 3-1: Excerpt from modifiedMPR table in TS38101-1 v18.8.0 (Table L.1-1: Definitions of the bits in the field modifiedMPR-Behaviour)

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n71 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)71AA UE EN-DC |
| n34 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |

Observation 1: In the modifiedMPR Table, "shall" is used when it is mandatory to satisfy the new MPR, and "may" is used when it is optional.

In the draft CRs submitted at RAN4#115 [4][5], "shall" is used for Rel-19 and "may" is used for releases prior to Rel-18. According to the above Observation 1, this indicates that it is mandatory to use a smaller new A-MPR value from Rel-19 onwards, while allowing the UE to optionally support the new MPR requirements for releases prior to Rel-18. This appears to be consistent with previous agreements made in RAN4.

Table 3-2: modifiedMPR Table for release prior to Rel-18 [4]

Table 3-3: modifiedMPR Table for Rel-19 [5]

Moreover, from the perspective of network implementation, this approach is preferable. The network can easily understand that the UE meets the new MPR requirements for Rel-19 when the UE sets the modifiedMPR-behaviour bit to 1, or the old MPR requirements when it is set to 0. If it were not mandatory to set the modifiedMPR-behaviour bit to 1 for Rel-19, the network implementation would become more complex. In this case, when the modifiedMPR-behaviour bit is set to 0, the network would need to check which release the UE supports (e.g., accessStratumRelease) to determine whether the old or new MPR requirements are met. To avoid complicating the network implementation, it is preferable to use "shall" for Rel-19.

Observation 2: By making it mandatory for UEs supporting Rel-19 to set the modifiedMPR-behaviour bit to 1, the network eliminates the need to verify which release the UE supports, thus allowing for easier understanding of which MPR requirements the UE meets.

Proposal 1: To avoid complications for the network, the modifiedMPR Table for n1 uses "shall" to mandate setting the bit to 1 for Rel-19, and "may" to allow for it to be optional for releases prior to Rel-18.

On the other hand, discussions are ongoing in parallel to have a common understanding of the ModifiedMPR. The issue of how to clean up the existing bits has been captured in WF [6].

Among these issues, how to handle legacy bits for which “shall” was used when requirements are changed several times seems to be causing concern regarding the use of "shall" for the n1 bit in Rel-19.

This issue is due to the possibility of multiple bits being set to 1 for the same requirement. Such situations could potentially be avoided by ensuring that when one bit is set to 1, any other relevant bits are set to 0 or absent. This solution aligns with our intention, as it allows the NW to determine which requirements the UE supports without reading the release information supported by the UE.

Observation 3: When multiple bits can be set to 1 for the same requirement, confusion can be avoided by ensuring that all but one bit are set to 0 or absent.

- Check the difference between bit ‘0’ and absent from BS and UE perspective. - Check if the spec version number can be removed from next release. - Check if the requirements are changed several times, how to handle the legacy bits. - See n41 n41; 0 (leftmost bit); EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0; - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC. 1; EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0; - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC. 2; EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 MPR or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR; -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC.

Including solutions like the one mentioned above, this issue could be addressed as part of a general cleanup discussion. Multiple bits for n1 will not be set simultaneously, and we believe that a general discussion should not block requirements for specific bands.

Observation 4: RAN4 can discuss the issues arising from the existing ModifiedMPR Table as a general agenda item, separate from the discussion on n1.

- Conclusion

Here we summarize our proposals:

Observation 1: In the modifiedMPR Table, "shall" is used when it is mandatory to satisfy the new MPR, and "may" is used when it is optional.

Observation 2: By making it mandatory for UEs supporting Rel-19 to set the modifiedMPR-behaviour bit to 1, the network eliminates the need to verify which release the UE supports, thus allowing for easier understanding of which MPR requirements the UE meets.

Proposal 1: To avoid complications for the network, the modifiedMPR Table for n1 uses "shall" to mandate setting the bit to 1 for Rel-19, and "may" to allow for it to be optional for releases prior to Rel-18.

Observation 3: When multiple bits can be set to 1 for the same requirement, confusion can be avoided by ensuring that all but one bit are set to 0 or absent.

Observation 4: RAN4 can discuss the issues arising from the existing ModifiedMPR Table as a general agenda item, separate from the discussion on n1.

References

- R4-2420481, “WF on PHS protection update”, KDDI, OPPO, RAN4#113

- R4-2508023, “WF on PHS protection update”, KDDI, OPPO, RAN4#115

- R4-2501736, “Clarification of modifiedMPR-behaviour for n1, NTT DOCOMO, INC., RAN4#114

- R4-2507230, “CR to 38.101-1 Rel-18. to add indication of modified MPR behavior [NR_newRAT-Core]”, Ericsson, Samsung, RAN4#115

- R4-2507231, “CR to 38.101-1 Rel-19. to add indication of modified MPR behavior [NR_newRAT-Core]”, Ericsson, Samsung, RAN4#115

- R4-2507957, “WF on Modified MPR Behavior”, Xiaomi, RAN4#115