TSG-RAN Working Group 4 (Radio) meeting #116	R4-2510825

Bengaluru, India, 25th to 29th August 2025

Source:	Ericsson

Title:	Discussion on Improvements for Annex I - TRP measurement procedures

Agenda item:	7.9.4.2.3

Document for:	Approval

# Introduction

One of the work areas of the work item on NR base station (BS) RF requirement evolution for FR1/FR2 and testing simplification, targets the BS TRP estimation methods in Annex I in TS 38.141-2. The goal is to improve applicability and description. As this annex is normative, it is imperative to clearly describe how the manufacturer performs the TRP measurement, the applicability conditions for each method and what are the criteria for deciding compliance to the TRP limits.

The current Annex I of TS 38.141-2 contains alternatives of measurement grids that are supposed to produce similar outcomes in terms of pass/fail (with negligible uncertainty relative  to the MU analysis in the TR 37.941). Some of the methods aim for an accurate estimation of the TRP, while others aim to an overestimation that would provide a safe margin to the limit.  There are probably even more alternatives which are not included at this moment. Due the lack of precise instructions on how each method can be applied, regulators have difficulties assessing the risk of passing a failing device. It is also from the current description not clear how different methods relate to each other. For this reason, there is a need for establishing a “reference method” which would be the one providing an accurate estimation of TRP within the given MU. Results using other methods can then be bench-marked towards the reference method, either for accuracy or margin of over-estimation.

Moreover, the poor description and shortcomings of the pre-scan method and a way forward are addressed.

In this contribution we discuss a few proposals for improving the Annex I in TS 38.104 by adding applicability conditions, reference TRP method and preferred methods.

At the end of this contribution a text proposal is attached for approval.

# Discussion

Starting with the agreements in the WF [1] we have the following proposals for going forward with the revision of Annex I in TS 38.141-2 (and similar annex in other specifications).

Proposal 1: Adopt a “reference method” for measuring the TRP of any signal regardless power level or frequency.

We propose that there should be one reference method, preferably the one called the “Spherical equal angle grid” with angular steps not exceeding the “reference steps”. The equi-angular spherical grid is well understood and widely studied in literature. The analysis is based on Spherical wave expansions wherein the dimensions of the device in terms of minimum sphere and cylinder determines the needed angular sampling. The reference steps correspond to λ/2 sampling on the minimum sphere and cylinder, respectively. Using reference steps on a spherical grid gives accurate TRP results.

In Annex I we find a few other grids, e.g. Fibonacci, that can be demonstrated to generate an accurate value of TRP if the reference angular steps are employed. In general, all methods based on using reference angular steps can be considered as accurate as the one based on equal angle grid.

This approach is also in line with IEEE 149-2021 [2] where it is indicated that for TRP measurements of general wireless handheld devices, the 3D sphere is often sampled at constant angle sampling grid. In case an irregular angular grid is used, appropriate weighting factors on each angular term can be used.

Proposal 2: Adopt Spherical Equal Angle Grid with reference angular steps (I.2.2) as the “reference method” for TRP measurements.

| Reference angular step | Nr. of samples | Approx. test time (rounded up) | Example |
| --- | --- | --- | --- |
| 1/8 | 4147200 | 48 days | SpEm: D = 0.6 m, f = 230 GHz SpEm D: 0.8m, f = 200 GHz |
| 1/4 | 1036800 | 12 days |  |
| 1/2 | 259200 | 3 days | SpEm: D = 100 cm, f = 35 GHz |
| 1 | 64800 | 18 h | SpEm: D = 100 cm, f = 17 GHz SpEm: D = 1.5 m, f = 12,5 GHz Carrier: D = 0.6 m, f = 28 GHz |
| 2 | 16200 | 4.5 h |  |
| 3 | 7200 | 2 h | Carrier: D = 0.8 m, f = 7,1 GHz Carrier: D = 1,5 m, f = 3,8 GHz |
| 4 | 4050 | 1.1 h |  |
| 5 | 2592 | 44 min | Carrier: D = 100 cm, f = 3.5 GHz |
| 6 | 1800 | 30 min |  |
| 9 | 800 | 14 min |  |
| 10 | 648 | 11 min | Carrier: D = 1.9 m, f = 900 MHz |
| 12 | 450 | 8 min |  |
| 15 | 288 | 5 min |  |
| 20 | 162 | 2.5 min | Carrier: D = 1 m, f = 900 MHz |
| 30 | 72 | 1 min 12 sec |  |
| 45 | 32 | 32 sec |  |

Table 2-1 Spherical equal angle grid: number of measurement points (samples) on the sphere and test time as a function of angular step, assuming 1 sec per sample.

Significant reduction in testing time can be reached by capturing raw data, and avoid processing and displaying the data while measuring, and using efficient file open/close handling.

Observation 1:  Annex I is called “TRP measurement methods” but we observe that we have both methods and procedures for estimating a TRP value, but also methods and procedures to avoid estimation of a TRP value (e.g. I.11 Peak method, I.13 Pre-scan). The latter provides pass/fail a criteria based on an overestimation of TRP, with a significant margin

Observation 2:  The description of the pre-scan method is quite limited and even suggesting using a handheld antenna. In case of BS this is not common practice in modern labs. Moreover, for increased reliability and repeatability it would be advantageous to introduce a pre-scan procedure that avoids at all manual positioning of antennas.

Observation 3: It is not needed to find the true Peak EIRP in the pre-scan.

To really be sure to find the peak of an emission, the entire sphere needs to be searched using angular steps that are significantly smaller than the reference steps. This is equivalent to the situation where much fewer samples are needed to find the average value (here TRP) than the number of points needed to find the max value (here Peak EIRP). If the direction of Peak EIRP is unknown this can present a formidable task. However, the task here is to estimate the TRP which implies that finding the true Peak EIRP is not really needed. Therefore, the pre-scan procedure can be relaxed while being described in a more well-defined way.

A pre-scan method can be made more precise and aligned to estimating TRP by observing that:

- To identify that an emission is at risk of failing compliance with a TRP limit, it is enough to find one EIRP sample (or nearfield equivalent, see below) that is higher than the TRP limit. Such a value is denoted “critical EIRP sample”.

Note: if a spherical surface is sampled using reference steps or smaller roughly half of the EIRP samples will have a value larger than the TRP value, i.e., they will be “critical EIRP samples”.

- Finding the true Peak EIRP is strictly useful only when the final goal is to measure the Peak EIRP.

- The angular grid needed to find a “critical EIRP sample” is probably much sparser than one corresponding to a grid of equiangular reference steps.

- Beam sweeping will smear out the EIRP pattern and probably lead to an increased probability to find a “critical EIRP sample” within a given angular grid.

- Continuous movement of the EUT (AC case) and stirrers (RC case) will speed up the test

Some proposals to be considered for pre-scan are:

- measurement distance can be shorter than the farfield distance, given that measurement antenna directivity is low enough (i.e. wide lobe)

- measurements can be fast, without averaging for many frames

- beam scanning can be used together with max hold readings on the receiving instrument

- should allow a continuous smooth movement of the positioner in the Anechoic Chamber (AC) case or stirrers in the Reverberation Chamber (RC) case.

If such a pre-scan method can be agreed upon, the peak method and the Equal sector methods become obsolete or can be reformulated. The procedural steps of finding the peak and maximizing the polarization match become obsolete and these methods become simpler and better aligned to other TRP methods.

Note that the nearfield equivalent of EIRP is

PEIRP=4πd2PD

Here d is the measurement distance and PD is power density [W/m2]. The TRP can then be calculated as

TRP=14π∬PEIRP |sin⁡(θ)|dθdϕ

The only difference being that PEIRP varies with distance d whereas EIRP does not.

Observation 4: An emission that is at risk of failing must have at least one EIRP value above the corresponding TRP limit.

Observation 5: Finding one EIRP sample above the corresponding TRP value can probably be done using sparse sampling and will be more reliable if beam sweeping is used.

Observation 6: Nearfield measurements can be used in the pre-scan.

A structured and well-defined pre-scan method can then be designed either for an anechoic chamber or a reverberation chamber

- Anechoic chamber alternative:

  - For each test beam [TBD] and for each emission frequency, measure the EIRP (sum of two orthogonal polarizations) in the pre-scan grid [to be decided, fixed points or continuous trace]

  - To demonstrate conformance the maximum EIRP value (over directions and beams) shall not exceed the TRP_limit – [X] [dBm/MHz].

- Reverberation chamber alternative:

  - For each test beam and for each emission frequency, measure the TRP (sum of two orthogonal polarizations) with a reduced number of independent samples [to be decided]

  - To demonstrate conformance the maximum measured TRP value (over samples and beams) shall not exceed TRP_limit – [X] [dBm/MHz]

Proposal 3: Study what angular sampling is needed to find “critical EIRP samples” for any type of emission

Proposal 4: Study how the RC method for TRP can be relaxed (fewer independent samples) to provide a reasonable pre-scan method.

Proposal 5: Study what type of beam steering to use in the pre-scan

Proposal 6: Study and agree on what type of emission pattern models (harmonics, adjacent, OBUE, spurious etc) to use for the pre-scan investigation in terms of array sizes, antenna element correlation, beams, etc

Proposal 7: Remove the Equal sector with peak average I.12, for the reason that it was defined related to I.11 and it is very unclear how the number of sectors K is chosen. It also does not bring any advantage compared to full sphere with sparse sampling.

Let us now analyse an example of test procedure, here is the one for OBUE in TS 38.141-2

Directions to be tested: As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex I).

##### 6.7.4.4.2	Procedure

The following procedure for measuring TRP is based on the directional power measurements as described in Annex I. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber. If so, follow steps 1, 3, 4, 6 and 9. When calibrated and operated within the guidance of 3GPP TR 37.941 [29] the measurement methods are applicable and selected depending on availability at the test facility.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	The measurement devices characteristics shall be:

-	measurement filter bandwidth: defined in clause 6.7.4.5.

-	detection mode: true RMS voltage or true power averaging.

As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity, efficiency and avoiding e.g. carrier leakage, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table 4.1.2.2-1 for FR1 and Table 4.1.2.2-2 for FR2.

4)	For single carrier operation, set the BS to transmit according to the applicable test configuration in clause 4.8 using the corresponding test model(s) in clause 4.9.2 at manufacturers declared rated carrier output power (Prated,c,TRP).

For a BS declared to be capable of multi-carrier and/or CA operation, use the applicable test signal configuration and corresponding power setting specified in clause 4.7.2 and 4.8 using the corresponding test model(s) in clause 4.9.2 on all carriers configured.

5) Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex I).

6)	Sweep the centre frequency of the measurement filter in contiguous steps and measure emission power within the specified frequency ranges with the specified measurement bandwidth.

7)	Repeat step 5-6 for all directions in the appropriated TRP measurement grid needed for TRPEstimate (see annex I).

8)	Calculate TRPEstimate using the measurements made in step 6.

9) For BS type 1-O and multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

Observation 7: In the test procedure for different requirements there is a reference to “appropriate TRP grid”. Use of word “appropriate” throws a large uncertainty over who is making the choice of the grid: the manufacturer, testing lab, the regulators?.

Observation 8: The test procedures always a mention TRP measurement grid and TRPEstimate as the value to compare to the limit. Some methods are not based on a spherical grid and neither on estimation of TRP.

Observation 9: Aligning the EUT is in general not needed for TRP assessment. Steps 2 and 5 in the OBUE test method can be omitted, for example. The important aspect is that the EUT is placed within the qualified test zone of the chamber.

# Summary

# References

[1]	R4-2504740 Way Forward for [114bis][304] NR_BS_RF_Part3_OTA_TRP

[2]	IEEE 149-2021 Recommended Practice for Antenna Measurements