3GPP TSG-RAN WG4 Meeting #116	R4-2510833

Bangaluru, India, 25 August – 29 August 2025

Agenda Item:	7.6.5.1

Source:	Ericsson

Title:	TE-emulated channel model for NTN UE tests

Document for:	Discussion

# 1	Introduction

RAN4#115 agreed with the way forward on the TE-emulated channel modeling for NGSO NTN testing [1]. This contribution discusses the remaining open issues.

# 2	NGSO NTN test environment overview

Figure 1	Assumed test environment for NGSO NTN UE testing.

Figure 1 is the overview of the NGSO NTN test environment. During the test, DUT (UE) estimates the Doppler shift and propagation delay based on the satellite position/velocity (ephemeris) information and UE’s location information at a given time. The UE location is stationary during the test, and the UE location information is pre-configured or provided via AT command, as specified in TS 38.508 (See C.1). The UE uses the estimated Doppler shift and propagation delay to perform the pre-compensation before the baseband processing.

In the test environment, TE should update the Doppler shift and propagation delay once per 1ms. The TE applies the updated Doppler shift and propagation delay to the transmit signals before the fading simulator, but the TE does not need to align the updating timing with the slot boundary.

To derive the Doppler shift and propagation delay, the TE needs to calculate the satellite position/velocity based on the satellite propagator model. TE may use the Kepler’s equation or equation of motion. TE should update the satellite position/velocity every Tsat (sec), for example, every 1.024 seconds. During the period between the satellite potion update times, the TE may calculate the Doppler shift and propagation delay by interpolation once per 1ms. TE can choose the interpolation algorithm as far as the accuracy of Doppler shift and propagation delay is satisfied. The UE and TE should share the same UE location information to derive the Doppler shift and propagation delay.

During the test, the TE requires to generate the satellite ephemeris information specified in SIB19 ephemerisInfo-r17 for NR or SIB31 ephemerisInfo-r17 for IoT-NTN. TE needs to update the satellite ephemeris information and send to the DUT together with epochTime every Tsib (sec), where Tsat < Tsib. RAN4 assumes the ephemeris information is same as the satellite position/velocity calculated from the propagator model used for the Doppler shift and propagation delay calculation.

Figure 2 shows the update timing of the satellite ephemeris information, that is, ephemerisInfo, Doppler shift and propagation delay.

Figure 2	Update timing of satellite position/velocity, epehmerisInfo, Doppler shift, and propagation delay.

| Parameters | Unit | Values |
| --- | --- | --- |
| Tsat | sec | 1.024 |
| Tsib | sec | 10.24 for RF and demodulation tests 2.56 for RRM tests |

# 3	Discussion

## 3.1	Propagation delay model

Propagation delay model with considering earth’s rotation during signal flight time Agreement - Downlink propagation delay model at UE reception time t_R with considering the Earth's rotation during signal flight time DtRUE=ρtRECEF/c1+1cvtRECI⋅ρtRECEFρtRECEF - FFS whether and how to consider the propagation delay model for UL delay error for RRM uplink timing error cases - Option 1 ΔtUL=RULtTc1-1cVSVtT⋅RULtTRULtT Where SV (Satellite): Receiver UE: Transmitter ∆tUL=tR-tT: Uplink total propagation delay tT: Time when signals are transmitted from UE tR: Time when signals are received by satellite rR: SV position vector on ECI Frame at tT. rT: UE position vector on ECI Frame at tT. VSV: Satellite velocity vector on ECI Frame RUL=rR-rT: Distance between satellite and UE at either tT or tR depending on the reference point. c = 2.99792458 x 108 [m/s]

RAN4#115 agreed the DL propagation delay model, but it was FFS for UL propagation delay model. We think RAN4 also needs to specify the UL propagation delay model, because RRM UL transmission timing test needs the UL propagation delay model.

For the DL propagation delay model, the parameter of the DL delay model is the UE reception time, because TE will apply the delay to the DL signal at the UE receive antenna connector. For UL propagation delay model, on the other hand, the parameter of the UL delay model should be the UE transmission time, because we assume UE estimates and compensate the UL propagation delay based on the UE transmission time.

If we consider the UE (DUT) transmits the signal at time tTUE and the satellite (TE) receives this signal at time tRSat, the propagation delay is the difference tRSat-tTUE, as shown in Figure 3. If both the satellite and UE do not move, the satellite delay is simply derived from the distance between the UE position at tTUE (UEtTECEF) and the satellite position at tRSat (rtRECEI), that is,

Delay=tRSat-tTUE=rtRECEF-UEtTECEFc

where c is the speed of light.

Snice UE does not know the satellite position at tRSat, UE needs to derive the delay based on the UE position, satellite position and satellite velocity at transmit time tTUE, as shown in Figure 3 (b). The propagation delay based on the UE transmission time is given as follows.

DtTUL=ρtTECEF/c1-1cvtTECI⋅ρtTECEFρtTECEF

where ρtT ECEF=rtTECEF-UEtTECEF

See appendix A.1 on how to derive this formula.

| (a) Illustration of signal path | (b) Propagation delay estimated at Satellite at time tT |
| --- | --- |

Figure 3	Signal path from the time the UE transmits signal at tT to the time the satellite receives the signal at tR.

Figure 4 (a) shows the UL propagation delay DtTUL. Figure 4 (b) also shows the difference between two delay models: DtTUL - ρtTECEF/c

| (a) UL propagation delay estimated at UE transmit time | (b) UL propagation delay error |
| --- | --- |

Figure 4	UL propagation delay.

Proposal 1: RAN4 should specify the UL propagation model with DtTUL=ρtTECEF/c1-1cvtTECI⋅ρtTECEFρtTECEF.

# 4	Summary

Proposals:

Proposal 1: RAN4 should specify the UL propagation model with DtTUL=ρtTECEF/c1-1cvtTECI⋅ρtTECEFρtTECEF.

# References

- R4-2508625, “Way Forward for [115][324] NTN_testing_NGSO_channel_model_and_demod”, Samsung.

# Appendix

## A.1	Propagation delay estimated at the satellite reception time

We consider DtTUL is the UL propagation delay estimated at time tT.

DtTUL=tRSat-tTUE=rtRECEF-UEtTECEFc

We rewrite rtRECEF-UEtTECEF=c ⋅DtTUL with the relation rtRECEF=rtTECEF+vtTECI⋅DtTUL. That is,

rtTECEF+vtTECI⋅DtTUL-UEtTECEF=c⋅DtTUE

We should emphasize the satellite velocity vectors, vtTECI, shall be inertial reference frame, given by:

vtTECI=vtTECEF+00ωE×rtTECEF

By defining, ρtT ECEF:=rtTECEF-UEtTECEF

c⋅DtTUE=rtTECEF+vtTECI⋅DtTUL-UEtTECEF

c2DtTUL2=rtTECEF+vtTECI⋅DtTUL-UEtTECEF2

c2DtTUL2=rtTECEF-UEtTECEF+vtTECI⋅DtTUL2=ρtT ECEF+vtTECI⋅DtTUL2

c2DtTUL2≈ρtTECEF2+2vtTECI⋅ρtTECEF⋅DtTUL

By taking a square root of the equation,

c⋅DtTUL=ρtTECEF2+2vtTECI⋅ρtTECEF⋅DtTUL

=ρtTECEF1+2vtTECI⋅ρtTECEFρtTECEF2⋅DtTUL

≈ρtTECEF1+vtTECI⋅ρtTECEFρtTECEF2⋅DtTUL=ρtTECEF+vtTECI⋅ρtTECEFρtTECEF⋅DtTUL

Finally, we get the equation below.

c⋅DtTUL≈ρtTECEF+vtTECI⋅ρtTECEFρtTECEF⋅DtTUL

Solve the above equation by Δt,

c-vtTECI⋅ρtTECEFρtTECEF⋅DtTUL=ρtTECEF

DtTUL=ρtTECEFc-vtTECI⋅ρtTECEFρtTECEF=ρtTECEF/c1-1cvtTECI⋅ρtTECEFρtTECEF