3GPP TSG-RAN WG4 Meeting #116	R4-2510834

Bangaluru, India, 25 August – 29 August 2025

Agenda Item:	7.6.5.3

Source:	Ericsson

Title:	UE demodulation requirements with time varying Doppler shift and propagation delay model

Document for:	Discussion

# 1	Introduction

RAN4#115 agreed with the way forward on the TE-emulated channel modeling for NGSO NTN testing [1], where RAN4 agreed with the NR NTN and IoT-NTN demodulation requirements to which the time-varying Doppler shift and propagation delay model is applied.

This contribution discusses the open issue on the UE demodulation requirements with time-varying Doppler shift and propagation delay model.

# 2	NGSO NTN test environment

Figure 1	Assumed test environment for NGSO NTN UE testing.

Figure 1 is the overview of the NGSO NTN test environment. During the test, DUT (UE) estimates the Doppler shift and propagation delay based on the satellite position/velocity (ephemeris) information and UE’s location information at a given time. The UE location is stationary during the test, and the UE location information is pre-configured or provided via AT command, as specified in TS 38.508 (See C.1). The UE uses the estimated Doppler shift and propagation delay to perform the pre-compensation before the baseband processing.

Figure 2 shows an example of DL Doppler shift and DL propagation delay using LEO-600 maximum positive Doppler test parameters where the initial evaluation angle is 30 degrees.

Figure 2	DL Doppler shift and DL propagation delay for LEO-600 maximum positive Doppler scenario.

# 3	UE demodulation requirements

The remaining open issue is the impact on UE demodulation requirements with the time-varying Doppler shift and propagation delay model.

Impact on RAN4 demodulation requirements with TE-emulated time vary doppler shift and propagation delay models agreement - Launch simulation campaigns to assess the impact on demodulation requirements from time-varying propagation delay and Doppler shift. Considering the residual frequency error and timing error to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. - Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models - residual frequency offset and time delay - option 1: Fix value - option 1a: frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s - option 1b: frequency error and timing error as 2Hz/s and 0.05us/s - option 2: time-varying propagation delay and Doppler shift. - FFS - Companies are encouraged to provide the simulation results during RAN4#116 meeting

In RAN4#115, we provided our analysis on the residual frequency offset and propagation delay compensation errors [2]. With the assumption that UE receives ephemeris information every 10.24 seconds and UE uses Quadratic (second order) interpolation or Cubic spline interpolation to derive the Doppler shift and propagation delay due to the satellite moving, we have observed that the residual errors with the quadratic interpolation or cubic spline interpolation are at most 2Hz/0.05us and 1Hz/0.035us for the quadradic interpolation and for the cubic spline interpolation methods, respectively. Since these residual errors are small enough compared with the UE Tx frequency error requirements of 0.1ppm (=200Hz@2GHz) and Tx timing error of 0.944 us (=29*64*Tc), we proposed that it is possible to set the same PDSCH demodulation requirements without extra margin even if the time-varying Doppler shift and propagation delay model is applied.

| (a) Test 1-1 QPSK 0.33, NTN-TDLA100-200 | (b) Test 1-2 16QAM 0.5, NTN-TDLC5-200 |
| --- | --- |

Figure 3:	PDSCH throughput with/without residual frequency offset and time delay.

In RAN4#115, RAN4 agreed with the way forward to investigate the PDSCH performance impact due to the residual frequency offset 200Hz@2GHz and time delay 9.44e-7s. Figure 3 shows our simulation results of NR PDSCH with Test 1-1 and 1-2 in TS 38.101-5 table 8.2.1.2.2.1.1-3 with/without fixed frequency offset of 200Hz and timing delay of 0.944us.

Our simulation results show the performance degradations of with the residual frequency offset and time delay of 0.16dB and 0.35dB for Test 1-1 and Test 1-2, respectively, regarding the SNR to achieve 70% of the peak rate.

Observation 1: The degradation of PDSCH with residual frequency offset and time delay is about 0.16dB for NR NTN PDSCH Test 1-1 (QPSK 0.33, NTN-TDLA100-200).

Observation 2: The degradation of PDSCH with residual frequency offset and time delay is about 0.35dB for NR NTN PDSCH Test 1-2 (16QAM 0.5, NTN-TDLC5-200).

When RAN4 defined the NR PDSCH demodulation requirements and eMTC/NB-IoT (N)PDSCH demodulation requirements, RAN4 first averaged companies’ impairment results and then added the additional margin 0.5 to 1.0dB according to the modulation order. As we discussed before, the assumed residual frequency offset and timing delay in Figure 3 is much larger than the practical residual errors, we expect the actual degradation is smaller than the results shown in our simulations. RAN4 should keep the same requirements without extra margin even if the time-varying Doppler shift and propagation delay model is applied.

Proposal 1: Set the same NR NTN PDSCH requirements (SNR) even if the test applies the time-varying Doppler shift and propagation delay model.

Proposal 2: Set the same IoT-NTN (N)PDSCH requirements (SNR) even if the test applies the time-varying Doppler shift and propagation delay model.

# 4	Summary

Observations:

Observation 1: The degradation of PDSCH with residual frequency offset and time delay is about 0.16dB for NR NTN PDSCH Test 1-1 (QPSK 0.33, NTN-TDLA100-200).

Observation 2: The degradation of PDSCH with residual frequency offset and time delay is about 0.35dB for NR NTN PDSCH Test 1-2 (16QAM 0.5, NTN-TDLC5-200).

Proposals:

Proposal 1: Set the same NR NTN PDSCH requirements (SNR) even if the test applies the time-varying Doppler shift and propagation delay model.

Proposal 2: Set the same IoT-NTN (N)PDSCH requirements (SNR) even if the test applies the time-varying Doppler shift and propagation delay model.

# References

- R4-2508625, “Way Forward for [115][324] NTN_testing_NGSO_channel_model_and_demod”, Samsung.

- RP-2507104, “UE demodulation requirements with time varying Doppler shift and propagation delay model”, Ericsson.