3GPP TSG-RAN WG4 Meeting #116	R4-2510835

Bangaluru, India, 25 August – 29 August 2025

Agenda Item:	7.17.3

Source:	Ericsson

Title:	Discussion on CSI prediction

Document for:	Discussion

# 1	Introduction

RAN4#115 agreed with the way forwards on AI/ML for NR air interface [1][2]. This contribution discusses the remaining issues on the CSI reporting requirement and testing framework for CSI prediction.

# 2	Performance monitoring

CSI prediction: - CSI prediction, the details are subject to RAN1 decision - RAN4 will further study the feasibility to specify the corresponding delay and/or accuracy requirements for UE sided monitoring

RAN1 agreed that UE reports the following two SGCS values in CSI Prediction Accuracy Indicator (CSI-PAI) for the UE-assisted performance monitoring metric:

- First SGCS (SGCSk1): calculated from the predicted PMI for one inference reporting and ground truth PMI. UE shall report SGCS for each MIMO layer k.

- Second SGCS (SGCSk2): calculated from the ground truth PMI and non-predicted PMI corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance. UE shall report SGCS for each MIMO layer k.

Figure 1 illustrates the relation between the first SGCS and second SGCS values.

Figure 1	Illustration of first SGCS and second SGCS reported in CSI-PAI.

RAN1 also agreed that the reported SGCS values range from 0 to 15 (4 bits), where 15 codepoints mapped to uniformly quantize in range [0.3 1] in a linear scale with 0.05 step size, and one codepoint is used to indicate the value range of (0 0.3].

We think it is difficult to verify the absolute accuracies of the reported SGCS1 and SGCS2 themselves, because TE does not know the predicted PMI, non-predicted PMI from the last CSI-RS resource, and ground-truth PMI (PMI derived from the measured CSI-RS). One test metric is that, like the CQI fading test, TE takes a statistic of reported SGCS1 and set the requirements that SGCS1 should be more than X with 90% of the test time, e.g., X=0.85.

Another test metric could be a ratio of the first SGCS and second SGCS, that is, SGCS1/SGCS2. Figure 2 and Figure 3 show our simulation results of the ratio of CSI-PAI SGCS1 and SGCS2 (after quantization with 0.05 step size) for 32 CSI-RS ports with TDLD30-20 and TDLA30-50, respectively.

From the simulation results, the ratio SGCS1/SGCS2 is 1.06 at 10%-ile for 20Hz, that is, the ratio SGCS1/SGCS2 is more than 1.06 for 90% of the test time. This implies the AI/ML-based predicted PMI is close to the non-predicted PMI. On the other hand, the ratio SGCS1/SGCS2 is 1.52 at 10%-ile for 50Hz, which implies the predicted PMI becomes higher SGCS compared with non-predicted PMI. Since we have agreed to define AI/ML-based PMI reporting performance requirements with more than one scenario, RAN4 should consider using the statistics of the ratio SGCS1/SGCS2 as a test metric.

Figure 2	Ratio of CSI-PAI SGCS1 and SGCS2 for 32 CSI-RS ports with TDLA30-20.

Figure 3	Ratio of CSI-PAI SGCS1 and SGCS2 for 32 CSI-RS ports with TDLA30-50.

Observation 1:	CSI-PAI SGCS1 and SGCS2 are close for TDLA30 with 20Hz scenario. Ratio of SGCS1 and SGCS2 > 1.06 for 90% of test time.

Observation 2:	CSI-PAI SGCS1 is higher compared with SGCS2 for TDLA30 with 50Hz scenario. Ratio of SGCS1 and SGCS2 > 1.52 for 90% of test time.

Proposal 1: During the AI/ML-based predicted PMI reporting test, TE also collects SGCS1 and SGCS2 using CSI-PAI reporting.

Proposal 2: Study the feasibility to set the CSI-PAI reporting requirements with that the SGCS1 should be more than X in [90]% of the test time, e.g., X=0.85.

Proposal 3: Study the feasibility to set the CSI-PAI reporting requirements with that the radio SGCS1/SGCS2 should be more than Y in [90]% of the test time, e.g., Y=1.0.

# 5	Summary

Observations:

Observation 1:	CSI-PAI SGCS1 and SGCS2 are close for TDLA30 with 20Hz scenario. Ratio of SGCS1 and SGCS2 > 1.06 for 90% of test time.

Observation 2:	CSI-PAI SGCS1 is higher compared with SGCS2 for TDLA30 with 50Hz scenario. Ratio of SGCS1 and SGCS2 > 1.52 for 90% of test time.

Proposals:

Proposal 1: During the AI/ML-based predicted PMI reporting test, TE also collects SGCS1 and SGCS2 using CSI-PAI reporting.

Proposal 2: Study the feasibility to set the CSI-PAI reporting requirements with that the SGCS1 should be more than X in [90]% of the test time, e.g., X=0.85.

Proposal 3: Study the feasibility to set the CSI-PAI reporting requirements with that the radio SGCS1/SGCS2 should be more than Y in [90]% of the test time, e.g., Y=1.0.

# References

- R4-2508080, “WF on NR_AIML_air_part1”, Qualcomm Incorporated.

- R4-2508051, “WF on AI/ML part 2”, Ericsson.