3GPP TSG-RAN WG4 Meeting #116	R4-2510838

Bengaluru, India, 25 August – 29 August 2025

Agenda Item:	7.28.6

Source:	Ericsson

Title:	SAN demodulation requirements for IoT-NTN

Document for:	Discussion

# 1	Introduction

RAN4#115 agreed with the way forward on Rel-19 IoT-NTN phase 3 SAN demodulation requirements [1], where RAN4 are going to define NPUSCH format 1 demodulation requirements with OCC2. This contribution provides our simulation results.

# 2	Simulation results

The main purpose of the enhancement of NPUSCH format 1 in Rel-19 IoT-NTN Phase 3 is to enhance the capacity of NPUSCH format 1 by multiplexing two UEs for NPUSCH format 1 via orthogonal cover codes (OCC). RAN4#115 agreed with the simulation assumption of NPUSCH format 1 with OCC2, but RAN4 did not discuss whether to configure the co-scheduled UE or not.

Considering the realistic scenario, we propose that NPUSCH format 1 demodulation requirement configures co-scheduled UE, e.g., NPSUCH format 1 for the target UE transmitted with OCC code [1 1], and NPUSCH format 1 for the co-scheduled UE transmitted with OCC code [1 -1].

Proposal 1: Configure the co-scheduled UE for OCC2 NPUSCH format 1 demodulation requirements.

Table 1 shows our simulation result for NPUSCH format 1, where we show the performance with and without co-scheduled UE. In the case of existence of co-scheduled UE, we have derived SNR from NPUSCH with OCC code [1 1]. From the simulation results, we do not see the significant performance difference between two cases as far as we use NTN-TDLA100-1.

Observation 1: Impact of the co-scheduled UE with OCC2 NPUSCH format 1 is not significant in the case of NTN-TDLA100-1.

Table 1	NPUSCH format 1 simulation results with OCC-2.

| Tx antennas | Rx antennas | SCS | Sub-carriers | Propagation channel | FRC | Repetition number | SNR to achieve 70% of the peak rate (without Co-scheduled UE) | SNR to achieve 70% of the peak rate (with Co-scheduled UE) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 (Note 1) | 2 | -4.2 | -4.3 |
| 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 (Note 1) | 2 | -7.3 | -7.6 |
| 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-2 (Note 2) | 8 | -13.0 | -13.1 |
| 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-2 (Note 2) | 8 | -15.2 | -15.2 |

# 3	Summary

Observations:

Observation 1: Impact of the co-scheduled UE with OCC2 NPUSCH format 1 is not significant in the case of NTN-TDLA100-1.

Proposals:

Proposal 1: Configure the co-scheduled UE for OCC2 NPUSCH format 1 demodulation requirements.

# References

- R4-250, “Way Forward for [115][334] IoT_NTN_Ph3_demod”, ZTE Corporation.

# Appendix

Table A.7.1-1: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A7-1 | A7-2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 |
| Number of allocated subcarriers | 1 | 12 |
| Diversity | No | No |
| Modulation | BPSK | QPSK |
| ITBS / IRU | 0 / 1 | 9 / 0 |
| Payload size (bits) | 32 | 136 |
| Allocated resource unit | 2 | 1 |
| Code rate (target) | 1/3 | 2/3 |
| Code rate (effective) | 0.29 | 0.56 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 288 |
| Total symbols per resource unit | 96 | 144 |
| Channel estimation length (ms) Note 1 | 16 | 2 (when repetition = 2) 4 (when repetition > 2) |

Table A.16.1-1: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A16-1 | A16-2 | A16-3 | A16-4 | A16-5 | A16-6 |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 | 15 | 15 | 15 | 15 |
| Number of allocated subcarriers | 1 | 1 | 3 | 6 | 12 | 12 |
| Diversity | No | No | No | No | No | No |
| Modulation | BPSK | BPSK | QPSK | QPSK | QPSK | QPSK |
| ITBS / IRU | 0 / 1 | 0 / 1 | 3 / 0 | 7 / 0 | 9 / 0 | 5/4 |
| Payload size (bits) | 32 | 32 | 40 | 104 | 136 | 424 |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 | 5 |
| Code rate (target) | 1/3 | 1/3 | 1/3 | 1/3 | 2/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 | 0.22 | 0.44 | 0.56 | 0.34 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 288 | 288 | 288 | 1440 |
| Total symbols per resource unit | 96 | 96 | 144 | 144 | 144 | 720 |
| Channel estimation length (ms) Note 1 | 16 | 4 | 4 | 4 | 2 (when repetition = 2) 4 (when repetition > 2) | 4 |
| Note 1: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |  |