3GPP TSG-RAN WG4 Meeting #116	R4-2510840

Bangaluru, India, 25 August – 29 August 2025

Agenda Item:	7.22.8

Source:	Ericsson

Title:	Device demodulation requirements for Ambient IoT

Document for:	Discussion

# 1	Introduction

This contribution shows our view on the device 1 demodulation performance requirements for Ambient IoT (A-IoT).

# 2	Rel-19 Ambient-IoT

## 2.1	Overview

The scope of Rel-19 WI Ambient IoT is as follows [1]:

- Deployment scenario 1 (D1): Indoor micro BS and indoor A-IoT devices.

- Topology 1 (T1): A-IoT device directly communicates with BS (reader)

- Device is a passive device without an amplifier, which relies on backscattering a carrier wave (CW)

- FR1 licensed spectrum in FDD

  - RAN4 RF assumed band 8 (900MHz) as an example band.

## 2.2	Physical layer design

RAN1 introduced two new physical channels dedicated to A-IoT, which are PRDCH and PDRCH.

### 2.2.1	PRDCH

PRDCH is a Reader to Device (R2D) physical channel that transmits higher layer R2D information. PRDCH is transmitted on FDD DL carrier frequency. PRDCH uses OOK modulation with multiple (M) chips in each OFDM symbol, where M = (2, 6, 12, 24). RAN1 also assumes only SCS=15kHz can be used for PRDCH.

In the channel coding, the CRC is attached to the information bits. Then the information bits are performed the line coding (Manchester coding), followed by the OOK modulation. The applied CRC algorithm is CRC-6 or CRC-16 depending on the information bits, i.e., use CRC-6 if the information bit length is 24 or less. Otherwise, CRC-16 is used.

RAN1 agreed that BS (reader) transmits R2D timing acquisition signal (R-TAS) before PRDCH, which is assumed to be used by the device for the time/frequency synchronization. R-TAS consists of Start indication part (SIP) and Clock acquisition part (CAP), where SIP is fixed ON/OFF pattern using 2 OFDM symbols. On the other hand, CAP is variable length that is the same value as M used for PRDCH.

RAN1 also agreed that the reader transmits the post-amble signal just after the PRDCH to indicate the end of PRDCH.

### 2.2.2	PDRCH

PDRCH is a Device to Reader (D2R) physical channel that transmits higher layer D2R information. PDRCH is transmitted on FDD UL carrier frequency, where the carrier frequency applied the small frequency shift (SFS) to the carrier frequency of carrier wave (CW). PDRCH uses OOK or BPSK modulation. RAN1 assumes PDRCH only uses SCS=15kHz.

In the channel coding, the CRC is attached to the information bits. Then the information bits are performed the block repetitions, convolutional coding, and OOK/PBCK modulation. The applied CRC algorithm is the same as PRDCH, that is, CRC-6 or CRC-16 depending on the number of information bits. The block repetitions are indicated by the reader whether to apply no repetition or one repetition. Convolutional coding scheme is same as LTE channel coding, specified in TS 36.212, where the constraint length of 7 and coding rate of 1/3. It is also possible to bypass the convolutional coding block by control signal.

RAN1 agreed that the device transmits D2R preamble signal just before PDRCH so that the reader can use them for timing acquisition and time/frequency tracking. The reader can also indicate to insert D2R mid-amble signal in between PDRCH transmission. The D2R preamble and mid-amble signals use M-sequence, and M-sequence can be longer or shorter one.

In summary, PDRCH transmission parameter can be:

- Apply convolutional coding or not

- Apply repetition or not

- D2R preamble/mid-amble length (long or short)

- Apply mid-amble or not, and its interval

## 2.3	Access procedure

Rel-19 Ambient IoT supports two traffic types [1], considering the application of indoor inventory and indoor command:

- Device-oriented traffic following device-terminated triggering (OD-DTT)

- Device terminated (DT)

Since the Rel-19 A-IoT device assumes the very low complexity passive device, there is no RRC state like IDLE and CONNECTED. Therefore, the A-IoT device access procedure is simply completed within 4 steps:

- Message 0 (R2D)

  - Reader sends a paging message to indicate the device to initiate access to the network.

  - In this paging message, the reader signals the time-domain/frequency-domain resources used in the following Message 1.

- Message 1 (D2R)

  - The device sends the 16 bits generated device ID to the reader.

  - This is like the random access preamble transmission.

- Message 2 (R2D)

  - When the reader detects the device ID, it echoes the device ID and provides Message 3 scheduling information.

    - Message 3 scheduling information includes whether to apply convolutional coding or not and apply the repetition or not.

  - This is like the random access response (RAR).

- Message 3 (D2R)

  - The device transfers the application layer information.

# 3	Demodulation performance requirements

## 3.1	Scenario

The WID mentions the Ambient IoT is operated in FR1 FDD licensed spectrum [1], and RAN1 assumes PDRCH/PRDCH uses only SCS=15kHz. If RAN4 defines PRDCH demodulation requirements, RAN4 should define the requirements with FR1 FDD SCS=15kHz only.

Proposal 1: If RAN4 defines PRDCH demodulation requirements, the requirements should be defined only with FR1 FDD SCS=15kHz.

The WID also mentions the Rel-19 Ambient IoT targets indoor deployment scenario (D1). In such a scenario, we expect the device to be stationary or very low mobility. We therefore propose considering TDLA30-10 for PRDCH demodulation requirements.

Proposal 2: If RAN4 defines PRDCH demodulation requirements, use TDLA30-10.

According to RAN4 RF discussion, Rel-19 Ambient IoT device has no antenna port. This means the performance test should be conducted OTA. Moreover, RAN4 RF session assumes Ambient IoT device has one Tx/Rx chain and BS also have one antenna element. We propose the demodulation requirements follow the RF assumption.

Observation 1: Ambient IoT device demodulation requirements are conducted OTA.

Proposal 3: Ambient IoT device demodulation requirements are set based on 1Tx/1Rx antenna configuration.

## 3.2		Device demodulation

A candidate for the device demodulation performance requirement is the PRDCH demodulation. PRDCH can be transmitted for Message 0 and Message 2. In our understanding, PRDCH for Message 0 transmits a preamble and PRDCH for Message 2 contains control messages. From the testability point of view, we think PRDCH demodulation for Message 0 is simple. From the demodulation performance point of view, PRDCH demodulation for Message 2 is more interesting but the test procedure is complex, that is, device needs to transmit Message 1 in advance. Considering the testability, we propose to specify the PRDCH demodulation requirement at least for Message 0. We are also open to specifying the PRDCH demodulation requirement for Message 2.

Proposal 4: Define PRDCH demodulation requirement at least for Message 0 reception.

Proposal 5: Discuss the feasibility of defining PRDCH demodulation requirement for Message 2 reception.

According to the access procedure, when the device receives the paging message from the reader, the device needs to send Message 1 to the reader. We think it is possible to define the test metric of PRDCH demodulation requirements as the probability of miss-detection of the paging (Message 0).

If RAN4 defines PRDCH with Message 2, the test metric can be defined as the probability of miss-detection of Message 2, which can be derived by counting the number of Message 3.

RAN4 usually considers the MDR target of 1% for the missed detection performance requirements. However, RAN4 may need to discuss the target MDR value, because it may require longer test time.

Proposal 6: Test metric of PRDCH demodulation requirement is the SNR to achieve [1] % of probability of miss-detection of the paging (Message 0). TE can count the number of Message 1 to derive the miss-detection probability.

For PRDCH, RAN4 agreed that the BS transmits M chips in each OFDM symbol, and M = {2, 4, 12, 24}. RAN4 should choose one or more M values according to the simulation results.

Proposal 7: RAN4 should choose one or more M values used for PRDCH based on the simulation results.

## 3.3	Specification structure

RAN plenary agreed to create new specifications to define the Ambient IoT device and BS demodulation performance requirements [1]. If RAN4 agree to define the device 1 demodulation performance requirements, they should be specified in the new specification “Ambient IoT device radio transmission and reception”.

Observation 2: 3GPP creates new specifications for Ambient IoT device and BS performance requirements.

Proposal 8: RAN4 should specify the Ambient IoT device demodulation requirements in “Ambient IoT device radio transmission and reception”, if RAN4 agree to define the Ambient IoT device demodulation requirements.

# 4	Summary

Observations:

Observation 1: Ambient IoT device demodulation requirements are conducted OTA.

Observation 2: 3GPP creates new specifications for Ambient IoT device and BS performance requirements.

Proposals:

Proposal 1: If RAN4 defines PRDCH demodulation requirements, the requirements should be defined only with FR1 FDD SCS=15kHz.

Proposal 2: If RAN4 defines PRDCH demodulation requirements, use TDLA30-10.

Proposal 3: Ambient IoT device demodulation requirements are set based on 1Tx/1Rx antenna configuration.

Proposal 4: Define PRDCH demodulation requirement at least for Message 0 reception.

Proposal 5: Discuss the feasibility of defining PRDCH demodulation requirement for Message 2 reception.

Proposal 6: Test metric of PRDCH demodulation requirement is the SNR to achieve [1] % of probability of miss-detection of the paging (Message 0). TE can count the number of Message 1 to derive the miss-detection probability.

Proposal 7: RAN4 should choose one or more M values used for PRDCH based on the simulation results.

Proposal 8: RAN4 should specify the Ambient IoT device demodulation requirements in “Ambient IoT device radio transmission and reception”, if RAN4 agree to define the Ambient IoT device demodulation requirements.

# References

- RP-250796, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA Inc.