3GPP TSG-RAN WG4 Meeting # 116						        R4-2510845

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.22.3.2

Source: 	OPPO

Title: 	On AIOT device requirement

Document for:	Approval

- Introduction

During the last RAN4 meeting, the AIOT device requirement has been discussed and a WF [1] has been agreed. In this discussion paper we try to give further discussion and finalize the device RF requirements.

- Discussion

During the RAN4#115 meeting, the agreed WF is captured as below:

Issue 3-1-1: Transmit output power

Agreement in RAN4#115:

- Performance metric: EIRP

  - FFS on how to determine device’s peak antenna gain direction

  - FFS on whether to measure multiple directions

- Take [10] dB backscatter loss for OOK as starting point, based on the assumption that the incoming CW signal and the backscatter signal are aligned with the peak gain direction of the device antenna.

  - FFS for BPSK

The power of the backscattered signal shall include only the 1st lower sideband and the 1st upper sideband, excluding the carrier itself.

The peak gain direction from our understanding can be declared by the vender. For the multiple directions, companies want to test the device antenna pattern in different directions and also considering the REFSENS requirement has been proposed with different directions, it is proposed to only test the peak gain direction of device.

Proposal 1: For the device peak antenna gain direction, vendor declaration is used.

Proposal 2: Only test the peak gain direction of device for transmit output power.

In our last meeting discussion paper, the regulation test of ERP is illustrated to calculate the backscattering loss as shown in figure 1. We have missed the 1m path loss of the measuring receiver to the tag and hence the result is quite different. Here we try to calculate it again.

From the EU regulation EN 302208, the CW input power will be 33dBm/200kHz from the CW output antenna connector. Simply use the free space path loss equation to estimate as

PL (dB)=92.45+20log10​(dkm​)+20log10​(fGHz​)

For the distance, use the 20cm distance between CW and the tag as illustrated in regulation test setup. And for band n8 the UL frequency will be 880-915MHz and we use 900MHz to calculation. Hence the PL will be around 17.6dB. The 33dBm/200kHz is E.R.P hence the CW antenna gain is already considered. With that, the input power of the device is around 15.4dBm/200kHz. Also as compared to the RF ID regulation, the output power of the tag is limited as -25dBm/100kHz corresponding to -22dBm/200kHz. The PL for 1m is around 31.5dB and hence the reflected power at the TAG antenna is around 9.5dBm/200kHz. Hence the backscattering loss is 9.5 – 15.4 = -6.1dB.

Figure 1 Regulation test setup

Observation 1: From regulation perspective, the backscattering loss of EN 302208 can be translated as around -6.1dBi on the peak gain direction.

As captured above, during the last meeting, -10dB is proposed to be the backscattering loss. The -6.1dB backscattering loss has considered the DC reflection power while it has been agreed that only first side lobe will be considered hence a 3dB difference is added. In such case, -9.1dBi will be the exact number. Hence it is proposed to use -9dBi for OOK and for -3dB for BPSK which accounts for DC difference of 3dB and modulation loss difference for additional 3dB.

Proposal 3: For OOK, the backscattering loss is -9dB and for BPSK, the backscattering loss is -3dB.

Issue 3-3-1: SEM requirements

Agreement in RAN4#115:

- Use EIRP as performance metric

  - Device’s peak antenna gain direction is based on declaration, i.e. same peak gain direction for backscattering loss testing

- The applicable frequency range for SEM: from the CBW edge to [OOB boundary]

- Define flat requirements for SEM as starting point, [9dBc~14dBc] based on achievable harmonic performance

- Define requirements applicable to all CBWs, FFS on how to reduce the test burden.

- Use the [maximum] CW input power level, FFS on the value.

Issue 3-3-1: spurious emission requirements

- Use EIRP as performance metric

  - Device’s peak antenna gain direction is based on declaration, i.e. same peak gain direction for backscattering loss testing

- OOB boundary

  - Option 1: D2R channel bandwidth plus minimum D2R channel bandwidth

  - Option 2: max of 500kHz and 10 times NB where NB equals to D2R CBW

  - Other options are not precluded

- Define requirements applicable to all CBWs, FFS on how to reduce the test burden.

- Use the [maximum] CW input power level, FFS on the value.

For the emission requirement, the agreement in the WF [1] is captured as above.

Figure 2 Emission levels from EU regulation

Figure 3 Resolution bandwidth of the EU emission requirement

From above figure 2 and 3, the emission requirement apply fc ± 400kHz. In such case the OOB range is set as fc ± 400kHz to fc ± 3000kHz with the signal  is around 320kHz. So around 10 times signal bandwidth is proposed as the OOB range.

Observation 2: The EU regulation has proposed 10 times signal bandwidth as OOB range.

Based on the last meeting agreement, flat SEM for device is proposed and also the OOB range is to be further discussed. Below figure 4 is the simulation of the D2R and we can see that for 6th harmonic, the compression can be up to 22dB. For 2nd harmonic the compression is around 11dB. When considering the regulation, we look back to the calculation of the transmit output power, the tag output power is -25dBm/100kHz compared to the OOB emission requirement as -36dBm/100kHz, so around 9dBc is required which is aligned with 11dB as mentioned previously. For the spurious emission, the compression needs to be up to 29dB with -54dBm/100kHz ERP in the spurious domain. From figure 4 below around 27dB can be reached in the 9th harmonic and hence 10 times harmonic can achieve around 29dB required compression.

Observation 3: For OOB domain, the 9dB required compression can be reached at around 11dB considering the 2nd harmonic.

Observation 4: For spurious domain, the 29dB required compression can be reached considering the 10th harmonic.

With observation 3, it is proposed to use 10dBc as the SEM flat requirement in the OOB domain.

Figure 4 PSD of D2R

Proposal 4: 10dBc is proposed to be the SEM requirement.

For the input CW signal power for SEM, it is proposed to use the same test setting which means the same CW input power of back scattering loss test. Further test setup can be discussed in OTA part.

Proposal 5: For the SEM requirement, the input CW power is the same as transmit output power test.

For the spurious emission, the requirement is based on regulation and the OOB range has been discussed. It is proposed to follow the regulation. For the OOB boundary, it has been proposed to use ITU requirement as “max of 500kHz and 10 times NB where NB equals to D2R CBW” in the last meeting. As also observation 2 shows the EU regulation has used the 10 times bandwidth for the OOB boundary. In such case, it is proposed to reuse the 10 times NB where NB equals to D2R CBW as the OOB boundary which also in line with observation 4.

Proposal 6: It is proposed to reuse the 10 times NB where NB equals to D2R CBW as the OOB boundary.

For the REFSENS requirement, the WF is captured as below:

Issue 3-4-1: Reference sensitivity

Agreement in RAN4#115:

- Using [-34dBm] as starting point to define REFSENS requirements, based on the peak gain direction of device antenna.

- Consider to define orientation requirements with following options:

  - Option 1: min sensitivity at peak antenna gain direction and EIS spherical coverage requirements, XdBm at Xth percentage

  - Option 2: spatial averaging EIS

  - Option 3: EIS in the peak direction

- Testing metric: 10% miss detection rate, FFS on whether to consider false alarm

- FFS for the FRC, consider following as starting point:

| SCS | 15 | 15 | 15 | 15 |
| --- | --- | --- | --- | --- |
| PRB | 1 | 2 | 3 | 4 |
| TBS | 20 | 20 | 20 | 20 |
| CRC | 6 | 6 | 6 | 6 |
| M | 6 | 12 | 24 | 24 |

To test the REFSENS with different directions as to consider the tag antenna pattern and its coverage performance, it has been proposed to test a hemi-sphere considering the theta angle and that couple of points on the flat plane. As shown below in figure 5. From the deployment perspective, usually the tag will be placed on top of the inventory and the base station will be at the roof. For some cases if the goods are stacked then a large theta angle cannot be avoided. In such case we propose 60 degrees to test the extreme condition.

Figure 5 REFSENS test points

Proposal 7: To test the 60° hemi-sphere REFSENS.

3	Conclusions

In this paper, we give discussion on the OTA aspect, the observations and proposals are as below:

Observation 1: From regulation perspective, the backscattering loss of EN 302208 can be translated as around -6.1dBi on the peak gain direction.

Observation 2: The EU regulation has proposed 10 times signal bandwidth as OOB range.

Observation 3: For OOB domain, the 9dB required compression can be reached at around 11dB considering the 2nd harmonic.

Observation 4: For spurious domain, the 29dB required compression can be reached considering the 10th harmonic.

Proposal 1: For the device peak antenna gain direction, vendor declaration is used.

Proposal 2: Only test the peak gain direction of device for transmit output power.

Proposal 3: For OOK, the backscattering loss is -9dB and for BPSK, the backscattering loss is -3dB.

Proposal 4: 10dBc is proposed to be the SEM requirement.

Proposal 5: For the SEM requirement, the input CW power is the same as transmit output power test.

Proposal 6: It is proposed to reuse the 10 times NB where NB equals to D2R CBW as the OOB boundary.

Proposal 7: To test the 60° hemi-sphere REFSENS.

4 References

[1] R4-2508116	WF on A-IoT_device, CMCC