3GPP TSG-RAN WG4 Meeting # 116						      R4-2510847

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.22.2

Source: 	OPPO

Title: 	On system parameter

Document for:	Approval

- Introduction

The ambient-IoT topic has been fully discussed during the last RAN4 meeting. A WF [1] has been agreed and further discussion on the system parameter is provided in this paper.

- Discussion

During the RAN4#115 meeting, the agreed WF [1] has been captured as below:

Issue 2-1-1: R2D transmission bandwidth

Agreement in RAN4#115:

- Define symmetric guard band as 10kHz, 20kHz, 30kHz and 40kHz for 1PRB, 2PRB, 3PRB and 4PRB channel bandwidth configuration

Issue 2-2-1: D2R bandwidth

Agreement in RAN4#115:

- Define D2R CBW based on all Tb, Tc, R combinations as finally approved in RAN1.

  - Assumed 10% SFO for device

  - For Reader, [90%] filter spectrum utility ([10%] guard band) is considered.

- The value of transmission BW and channel BW will be explicitly listed.

- Using following equation for BS as starting point:

D2R CBW for BS (KHz)=(2SB Transmission BW*(1/2)+2* Small frequency shift)/0.9

=(2000*(1+R)/Tb)* (1+∣SFO∣)/0.9

=(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)/0.9

  - Define [200kHz]，[3.54MHz] D2R CBW for AIoT BS. Other D2R CBWs for BS are FFS

- Using following equation for device as starting point:

D2R CBW for device (KHz)=2SB Transmission BW*(1/2)+2* Small frequency shift

=(2000*(1+R)/Tb)* (1+∣SFO∣)

=(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)

- The following are for information:

- Figure 1-1 Small frequency shift diagram of FDMA without SFO - Figure 1-2. SFO impact of transmission bandwidth and small frequency shift of FDMA - As the figures above indicate, SFO not only affects the D2R transmission bandwidth but also scales the small frequency shift value . - The 2SB transmission BW and small frequency shift _both measured in kHz——with and without SFO are as follows: - Note : BW and Small frequency shift are in kHz;Tc and Tb are in μs 2SB Transmission BW (kHz); Small frequency shift (kHz). Without SFO; With SFO; Without SFO; With SFO. Btx,D2R_ without SFO= 4*1000/Tb=2*1000/(R*Tc); Btx,D2R_ with SFO =Btx,D2R_ without SFO×(1±∣SFO∣); SFSwithout SFO=±1000*R/Tb= ±1000/(2*Tc); SFSwith SFO =SFSwithout SFO×(1±∣SFO∣).

Issue 2-3-1: R2D channel raster

Agreement in RAN4#115:

-	Define 10KHz channel raster

-	FFS on channel raster offset

  - R2D Channel bandwidth

Below figure 1 illustrates the 1PRB with 12 Sub-carriers configuration. The A-IOT R2D channel bandwidth is the transmission bandwidth configuration plus the guard band. The transmission bandwidth can be any number of sub-carriers, i.e. 10 sub-carriers as shown in figure 1. As the SU has been agreed to be defined as 90% for A-IoT R2D channel bandwidth, hence the guard band should be 10kHz, 20kHz, 30kHz and 40kHz for 1PRB, 2PRB, 3PRB and 4PRB channel bandwidth configuration.

Figure 1 A-IOT R2D channel bandwidth

Proposal 1: Define the A-IoT R2D channel bandwidth as figure 1 above.

2.2 D2R Channel bandwidth

For the D2R channel bandwidth, in the last RAN1 meeting, 2SB has been agreed. For the small frequency shift, it has been discussed in RAN1 and the conclusion is captured as below. Compare to the last meeting agreement, some of the Tb and Tchip has been down selected hence to be further simplified. From our last meeting paper, the corresponding transmission bandwidth and SFS to this Tb and R values are shown below in table 1 and table 2:

The background also captures the relationship between Tb, Tc, R and BTX,D2R. The corresponding numbers are also shown in above table. Also, from the background, the frequency shift can be calculated as frequency shift to CW=1/(2Tchip) and the Btx,D2R = 4/Tb.

The three critical equation has been marked as yellow above and captured again below:

-Tb = Tc×2R

-Btx,D2R = 4/Tb

-frequency shift to CW Fs=1/(2Tchip)

So together consider the RAN1 agreement, the frequency shift and transmission bandwidth are defined as below table 1 and table 2.

Table 1 frequency shift compared to Tchip

| Freq. shift to CW =1/(2Tchip) | 3.75 kHz | 7.5 kHz | 15 kHz | 30 kHz | 60 kHz | 120 kHz | 240 kHz | 480 kHz | 720 kHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tchip (μs) | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.69 |

And the transmission bandwidth of D2R when compared to Tb can be calculated as below table 2.

Table 2 transmission bandwidth of D2R compared to Tb

| Btx,D2R | 15 kHz | 30 kHz | 60 kHz | 120 kHz | 240 kHz | 480 kHz | 960 kHz | 2880 kHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb (μs) | 266.67 | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 1.39 |

From RAN4 perspective, it is suggested not to include the Tb and Tchip in the RAN4 specification but the exact frequency shift and transmission bandwidth. Hence below table 3 is proposed:

Proposal 2: The D2R channel bandwidth with its transmission bandwidth and corresponding feasible frequency shift is proposed as table 3.

Table 3 Frequency shift and transmission bandwidth

| Transmission bandwidth | Frequency shift (kHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3.75 kHz | 7.5 kHz | 15 kHz | 30 kHz | 60 kHz | 120 kHz | 240 kHz | 480 kHz | 720 kHz |
| 15 kHz | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 |  |
| 30 kHz |  | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 |  |
| 60 kHz |  |  | 15 | 30 | 60 | 120 | 240 | 480 |  |
| 120 kHz |  |  |  | 30 | 60 | 120 | 240 | 480 |  |
| 240 kHz |  |  |  |  | 60 | 120 | 240 | 480 |  |
| 480 kHz |  |  |  |  |  | 120 | 240 | 480 |  |
| 960 kHz |  |  |  |  |  |  | 240 | 480 |  |
| 2880 kHz |  |  |  |  |  |  |  |  | 720 |

The agreed calculation of the 2SB transmission BW and the small frequency shift can be also listed in the specification together with the table 3 above and it is not needed to give all the possible bandwidth and the frequency shift of all the combinations.

| 2SB Transmission BW (kHz) |  | Small frequency shift (kHz) |  |
| --- | --- | --- | --- |
| Without SFO | With SFO | Without SFO | With SFO |
| Btx,D2R_ without SFO= 4*1000/Tb=2*1000/(R*Tc) | Btx,D2R_ with SFO =Btx,D2R_ without SFO×(1±∣SFO∣) | SFSwithout SFO=±1000*R/Tb= ±1000/(2*Tc) | SFSwith SFO =SFSwithout SFO×(1±∣SFO∣) |

From testing perspective, especially for the calculation of the OOB boundary, the frequency shift together with the transmission bandwidth will be considered as illustrated in our companion paper. Also the base station receiving bandwidth will cover the whole range as combination of the small frequency shift as well as the transmission bandwidth. From this perspective, a “channel bandwidth” liked concept is still needed for D2R. So the channel bandwidth for 2SB can be defined as:
D2R channel bandwidth=Transmission bandwidth2*Small frequency shift+Transmission bandwidthR=1R>1

R is the repetition as illustrated above.

Proposal 3: To define the D2R channel bandwidth as:

D2R channel bandwidth=Transmission bandwidth2*Small frequency shift+Transmission bandwidthR=1R>1

The transmission bandwidth and small frequency shift both consider SFO.

Agreement

For D2R transmission,

- The minimum Tb is 1.39μs.

- Support at least the following values of Tb, Tchip, and R from the table agreed in RAN1#120bis.

|  | Tchip (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 133.33 | 66.67 | 33.33 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 | 0.69 | 0.52 |
| 266.67 | R=1 | R=2 | R=4 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |  | R=256 |
| 133.33 |  | R=1 | R=2 | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |
| 66.67 |  |  | R=1 | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 |
| 33.33 |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 |
| 22.22 |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  | R=16 |  |
| 16.67 |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 |
| 11.11 |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  |
| 8.33 |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 |
| 5.56 |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  |
| 4.17 |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |
| 2.78 |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  |
| 2.08 |  |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |
| 1.39 |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |  |
| 1.04 |  |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |

2.3 Channel raster

For the channel raster of A-IoT, it has been proposed to use 10kHz with the NR enhanced channel raster to A-IoT system. The enhanced channel raster of TS 38.101-1 has been captured as below. Since currently only band n8 has been used for example band, we did not captured all the NR bands. From the table below we can see, the 10kHz channel raster are mandatory support in some of the NR bands while for the rest they are optional supported, especially for band n8, it is not mandatory support.

From our understanding, directly use 10kHz channel raster for A-IoT system regardless of the band will have problem. For stand-alone operation, there is no limitation of the A-IoT system channel raster point. However, if the band is with 15kHz/30kHz channel raster, then use 10kHz as channel raster for such band is for sure not appropriate. If we have to use 10kHz as channel raster, then it only apply to NR bands with 100kHz channel raster and in stand-alone operation.

Proposal 4: 10kHz channel raster is used for NR bands with 100kHz channel raster for A-IOT.

For the frequency shift, companies have different views. Back to NB-IoT, the shift comes from the difference between 100kHz channel raster of LTE and the SCS of 15kHz. If in A-IOT, the 10kHz channel raster is used, then the whole channel raster grid is in 10kHz. For stand-alone operation, any of the channel raster of 10kHz defined for the band n8 for NR can be used for A-IOT since there is no co-existence problem exist. But for in-band operation if there is simultaneous NR and AIOT operation on band n8, then the channel spacing of A-IOT and NR needs to be defined as integer multiple of 15kHz to avoid inter sub-carrier interference. So instead of defining the offset, the channel spacing can be used which is quite similar to intra-band Non-contiguous CA cases that the in-band A-IOT and NR are SCS level aligned. Above discussion are all based on the channel raster to RE mapping of AIOT and NR are the same.

Proposal 5: The channel raster to RE mapping of AIOT and NR are the same.

Proposal 6: For stand-alone AIOT deployment, no frequency offset is needed.

Proposal 7: For in-band AIOT simultaneous NR transmission, channel spacing is used to keep the AIOT and NR centre frequency SCS level aligned.

3	Conclusions

In this paper, we give discussion on the system parameter, the observations and proposals are as below.

Figure 1 A-IOT R2D channel bandwidth

Proposal 1: Define the A-IoT R2D channel bandwidth as figure 1 above.

Proposal 2: The D2R channel bandwidth with its transmission bandwidth and corresponding feasible frequency shift is proposed as table 3.

Table 3 Frequency shift and transmission bandwidth

| Transmission bandwidth | Frequency shift (kHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3.75 kHz | 7.5 kHz | 15 kHz | 30 kHz | 60 kHz | 120 kHz | 240 kHz | 480 kHz | 720 kHz |
| 15 kHz | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 |  |
| 30 kHz |  | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 |  |
| 60 kHz |  |  | 15 | 30 | 60 | 120 | 240 | 480 |  |
| 120 kHz |  |  |  | 30 | 60 | 120 | 240 | 480 |  |
| 240 kHz |  |  |  |  | 60 | 120 | 240 | 480 |  |
| 480 kHz |  |  |  |  |  | 120 | 240 | 480 |  |
| 960 kHz |  |  |  |  |  |  | 240 | 480 |  |
| 2880 kHz |  |  |  |  |  |  |  |  | 720 |

Proposal 3: To define the D2R channel bandwidth as:

D2R channel bandwidth=Transmission bandwidth2*Small frequency shift+Transmission bandwidthR=1R>1

The transmission bandwidth and small frequency shift both consider SFO.

Proposal 4: 10kHz channel raster is used for NR bands with 100kHz channel raster for A-IOT.

Proposal 5: The channel raster to RE mapping of AIOT and NR are the same.

Proposal 6: For stand-alone AIOT deployment, no frequency offset is needed.

Proposal 7: For in-band AIOT simultaneous NR transmission, channel spacing is used to keep the AIOT and NR centre frequency SCS level aligned.

4 References

[1] R4-2508116	WF on A-IoT_device, CMCC