3GPP TSG-RAN WG4 Meeting # 116	R4-2510854

# Bengaluru, India, 25th – 29th August, 2025

Title:	Discussion on NR-NTN HPUE Tx requirements

Source:	Samsung

Agenda item:	7.6.3.2.1

Document for:	Approval

Introduction

In last RAN4 #115 meeting, the WF[1] had further progressed in RF requirements for NR-NTN PC2/1.5/1. And in addition, two draft bigCRs [2][3] were endorsed to capture these agreements.

In this paper, we will go through the remaining open issues in [1] and [2], and provide our analysis and proposals for them.

UE power configuration

In [1], one open issue is whether several parameters from TN spec is still applicable to the NTN HPUE. The issue is as follows:

Issue 1-1: Configured Tx power equation Agreement: - Agree to clarify and correct the configured transmitted power equation for NTN - [∆TC,c], ∆TIB,c , [∆TRxSRS] are not applicable to Rel-19 NR-NTN UE - FFS ΔPPowerClass - For ΔPPowerBoost, Rel-18 power boosting should be applicable to NR-NTN. - Regarding how to address these in CR drafting, following approach is agreed. - By referring to 38.101-1 and then exclude non-applicable parameters, e.g. ‘except for...’

By the end of #115 meeting , the WF[4] clearly stated the ΔPPowerClass is applicable, as for NR-NTN it was agreed in [4] to re-use TN. The relevant way-forward was given as follows.

- SAR Related issues in [4] - WF: - Keep the same status as TN for respective base RAT - NR-NTN: Reuse NR TN, - Including ΔPPowerClass in configured transmitted power clause - eMTC based Iot-NTN: Reuse eMTC - Not precluding ΔPPowerClass in configurated transmitted power clause - NB-Iot based Iot-NTN: Reusing NB-Iot

Observation 1: It was agreed in [4] that the ΔPPowerClass can be applicable to NR-NTN.

Besides, for the “[∆TC,c]”, given the fact that there’s no ‘NOTE3’ NOTE 3 in Table 6.2.1-1 in 38.101-1 [6] applied to any NTN bands in TS38.101-5 yet, we believe it’s factual we say the ∆TC,c is not applicable to Rel-19 NR-NTN UE.

For structure-wise, the same fact can also support the view that ∆TC,c is no harm to be kept for NR-NTN HPUE.

Observation 2: It is factual that ∆TC,c is not yet applied to any NTN bands so far in Rel-19.

And for “[∆TRxSRS]”, in the demodulation requirement, the current PDSCH demodulation requirements only specified up to 1T2R, and the NTN CA is still under discussion as future release objectives. In Rel-19, our understanding is that the “∆TRxSRS” is not applicable to NTN UE.

Observation 3: The NTN demod requirements only specified upto 1T2R for PDSCH, and CA is still pending on future release works, so that we believe the ∆TRxSRS seems not applicable to NTN Rel-19 UE spec.

Proposal 1: To confirm ∆TC,c and TRxSRS are not applicable to Rel-19 NTN UE spec.

PC1 MPR

In[1], the TN PC1 MPR table was taken as the starting point for further check. In our view, the MPR would be slightly impacted by the relaxed ACLR, but due to the fact that there’s lack of an alternative commercial PA that from both internal survey and from other contributing companies, we believe the MPR discussion can still be based on the previous TN PC1 PA, which is also aligned with the WID.

Issue 1-8 ~ Issue 1-9 PC1 MPR related Agreement: - Interested companies are encouraged to work on two tables with tentative values for PC1 MPR, one for 1Tx and one for 4Tx - Further check these tentative values, especially for the edge RB results by considering the working assumption of L_CRB as [8]RB. - With the understanding the [4Tx] architecture for PC1 is not agreed yet. - If 4Tx would be agreed, it can be further discussed if we can merge two tables then. Table 1. PC1 MPR [with 1Tx] Modulation; MPR (dB). Edge RB allocations; Outer RB allocations; Inner RB allocations. DFT-s-OFDM; Pi/2 BPSK; [Note 1]; ≤ [0.5]; ≤ [0]. QPSK; [Note 1]; ≤ [1]; ≤ [0]. 16 QAM; [Note 1]; ≤ [2]; ≤ [1]. 64 QAM; [Note 1]; ≤ [2.5]; ≤ [2.5]. CP-OFDM; QPSK; [Note 1]; ≤ [3]; ≤ [1.5]. 16 QAM; [Note 1]; ≤ [3]; ≤ [2]. 64 QAM; [Note 1]; ≤ [3.5]; ≤ [3.5]. NOTE 1: MPR for all modulations for Edge RB allocation is defined as following for two distinguished channel bandwidths groups as: Within the <50MHz channel bandwidth group: MPR= CEIL(7.2 dB- 6 dB∙ CBW100 MHz, 0.5 dB) Within the ≥50MHz channel bandwidth group: MPR= CEIL5.35 dB+3.15 dB∙ CBW100 MHz, 0.5 dB where CEIL(x,0.5 dB) means rounding x upwards to the closest multiple of 0.5 dB.. [Table 2. PC1 MPR with 4Tx] Modulation; MPR (dB). Edge RB allocations; Outer RB allocations; Inner RB allocations. DFT-s-OFDM; Pi/2 BPSK; ≤ [9.5]; ≤ [2.0]; ≤ [0.5]. QPSK; ≤ [10.0]; ≤ [2.5]; ≤ [0.5]. 16 QAM; ≤ [10.0]; ≤ [3.5]; ≤ [1.5]. 64 QAM; ≤ [10.0]; ≤ [4.0]; ≤ [3.5]. CP-OFDM; QPSK; ≤ [10.0]; ≤ [4.5]; ≤ [2.0]. 16 QAM; ≤ [10.0]; ≤ [4.5]; ≤ [2.5]. 64 QAM; ≤ [10.0]; ≤ [5.0]; ≤ [5.0].

Observation 4: Internal survey and feedback from contributing companies both claim there’s lack of alternative 1Tx PC1 PA besides the one used for TN PC1 1Tx PA.

Proposal 2: To re-use the assumption of TN PC1 1Tx PA for PC1 1Tx MPR analysis, and document it in the TR.

Proposal 3: Then confirm to re-use the TN PC1 1Tx MPR table for NTN PC1 1Tx HPUE.

A-MPR discussion

In last meeting, the [1] has documented very initial discussion over PC2 A-MPR below. And suggest offline efforts before ths upcoming #116 meeting.

Issue 1-4: PC2 AMPR Way forward: - To harmonize the AMPR regions in offline activates before next meeting. - Target in next meeting: Come up with numbers in square brackets, and confirmed by other companies. - Offline efforts before next meeting is expected.

In the summer, there’s offline activity arranged to collect A-MPR analysis for all power classes for both n256 and n255 bands, which were mentioned in the WID as example bands. And in this section, we will go through the available analysis that we have reviewed up to date of drafting for each power class.

As a general observations over the discussion during offline, we believe all contributed A-MPR analysis, under different assumptions, can be documented into the TR as long as their assumptions are clearly explained, e.g. filter rejection level, PA models, calibration point, etc.

Observation 5: All A-MPR analysis contributed, though some of them are different to what we have agreed or commonly assumed, are valuable for the standardization work for this and future NTN works.

Proposal 4: In general, all contributed A-MPR analysis, similar to the co-ex study, can be documented to the TR 38.863 but the associated assumptions should be clearly stated especially for those different to the agreement/WF, e.g. filter rejection level, PA models, calibration point, etc.

And a separate tdoc carrying the summary of the offline contributed A-MPR is also submitted to this meeting in [5].

4.1 NS_02N for n255

For n255 A-MPR assumption, in the offline, there was discussion over the previously agreed WF about the filter rejection assumed for NS_02N. In [1], the WF captured agreement as follows:

Issue 1-3: AMPR assumption for n255 Agreement: - Agree to use 10dB as filter rejection assumption for GNSS frequencies 1559-1610 MHz for PC2/PC1.5/PC1 AMPR analysis for n255.

In our view, this 10dB filter rejection was carried over from previous PC3 n255 A-MPR analysis [6], which is the baseline assumption for not having A-MPR for n255 PC3 and also it was documented as an essential method to comply with the emission requirements for this band. In this regards, we support the previous agreement

[6] For n255, based on regulatory requirements, a new NS is required. The following observations were made from the measurement results using an LTE modem and transceiver supporting 1626.5 – 1656.5 MHz: - For 5 MHz channel, PA emissions for DFT-s-OFDM are below the spurious emission limits between 1559 and 1606 MHz. - For 10 MHz channel, PA emissions for DFT-s-OFDM are exceeding the spurious emission limits for 1 MHz MBW by 5 dB between 1604 and 1606 MHz. - For 15 MHz channel, PA emissions for DFT-s-OFDM are exceeding the spurious emission limits for 1 MHz MBW by 9 dB between 1599 and 1606 MHz. - For 20 MHz channel, PA emissions for DFT-s-OFDM are exceeding the spurious emission limits by 9 dB between 1584 and 1606 MHz. - For 700 Hz MBW, PA emissions for all channel bandwidths are below the spurious emission limits between 1559 and 1606 MHz. A n255 duplexer with rejection of more than 10 dB will be a sufficient to satisfy the spurious emission limits specified in Table 7.4.2.2.8-1. Based on this measurement observations, the expected filter attenuation of more than 10 dB below 1610 MHz is going to be adequate to meet the additional emissions requirements between 1559 – 1610 MHz, and therefore, no A-MPR needs to be specified for n255.

Observation 6: An n255 duplexer with rejection of more than 10dB was assumed in PC3 AMPR analysis for n255, which is the basis of the existing AMPR analysis for n255.

Proposal 5: For n255 NS_02N A-MPR analysis for PC2/PC1.5/PC1, expected filter attenuation of more than 10 dB below 1610 MHz needs to be assumed following the previous n255 PC3 A-MPR analysis and agreement in [1].

By reviewing the summary in [5], for NS_02N all contributing companies proposed the A-MPR regions depicted in the following figure. The major difference is whether Region D in the following figure is required, as one sourcing company indicate it’s needed while the other company results didn’t have any A-MPR for this region.

Figure 4.1-1 Common A-MPR regions for NS_02N in [5]

Observation 7: In [5], all sourcing companies proposed similar regions as showed in Figure 4.1-1 above.

Proposal 6: For n255 NS_02N, to use the above common regions summarized from multiple sourcing companies to decide A-MPR values based on analysis.

4.2 NS_24N for n256

There’re more companies proposed analysis for n256 NS_24N A-MPR in [5]. And in the offline discussion, it is agreeable to almost all contributing companies to re-use PC3 region as starting point. Also, it’s worth to have further discussion on how to define new regions based on previous PC3 regions to make more RB allocation useful for NTN HPUE.

Observation 8: It is agreeable to most contributing companies to re-use PC3 A-MPR regions to discuss A-MPR values.

Proposal 7: For NS-24N for n256, re-use PC3 A-MPR regions as baseline to discuss A-MPR results. The regions can be updated if to make more RB allocations useful for NTN HPUE.

With this proposal, we further analysed the results summarized in [5], and we come up with the proposed starting point for each of the power class for NS_24N n256.

PC2 NS_24N:

Proposal 8: Take below A-MPR as starting point for NS_24N PC2.

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | 14 | 10.5 | 9.5 | 9.5 | 25.5 | 15 | 5 |
| DFT-s-OFDM QPSK | 14 | 10.5 | 9.5 | 9.5 | 25.5 | 15 | 5 |
| DFT-s-OFDM 16 QAM | 14 | 10.5 | 9.5 | 9.5 | 25.5 | 15 | 5 |
| DFT-s-OFDM 64 QAM | 14 | 10.5 | 9.5 | 9.5 | 25.5 | 15 | 5 |
| CP-OFDM QPSK | 15.5 | 11.5 | 10 | 12.5 | 26.5 | 17.5 | 6 |
| CP-OFDM 16 QAM | 15.5 | 11.5 | 10 | 12.5 | 26.5 | 17.5 | 6 |
| CP-OFDM 64 QAM | 15.5 | 11.5 | 10 | 12.5 | 26.5 | 17.5 | 6 |

PC1.5 2Tx NS_24N:

Proposal 9: Take below A-MPR as starting point for NS_24N PC1.5 2Tx.

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | 16.5 | 13 | 13 | 11.5 | 29 | 20 | 7 |
| DFT-s-OFDM QPSK | 16.5 | 13 | 13 | 11.5 | 29 | 20 | 7 |
| DFT-s-OFDM 16 QAM | 16.5 | 13 | 13 | 11.5 | 29 | 20 | 7 |
| DFT-s-OFDM 64 QAM | 16.5 | 13 | 13 | 11.5 | 29 | 20 | 7 |
| CP-OFDM QPSK | 18.5 | 14 | 14 | 13 | 29 | 21 | 8 |
| CP-OFDM 16 QAM | 18.5 | 14 | 14 | 13 | 29 | 21 | 8 |
| CP-OFDM 64 QAM | 18.5 | 14 | 14 | 13 | 29 | 21 | 8 |

4.3 PC1 1Tx for both NS_24N and NS_02N

For PC1 1Tx, there’s no NS_24N or NS_02N analysis received in [5]. It is due to the fact that there’s lack of PC1 1Tx RF component in the market for the contributing companies to perform analysis and/or measurement. Given the facts that the 3GPP discussion is contribution-driven, and market interests oriented, we believe it is pre-mature to define A-MPR for PC1 1Tx including both NS_24N and NS_02N for n256 and n255 respectively.

Observation 9: There is lack of RF component supporting PC1 1Tx based on the feedback from contributing companies, thus no analysis received for PC1 1Tx A-MPR for both NS_24N and NS_02N.

This is not an exceptional case that happens only to NTN bands, when we review the TN spec TS 38.101-1 [7], there are around 10 NS and associated A-MPR defined for approximately 14 TN bands which support PC1 operation. Among these 10 A-MPR requirements for those TN bands, 8 of them do not have dedicated A-MPR requirements for PC1.

Observation 10: Due to the same facts we are facing in NTN, for most of the TN A-MPR requirements, it hasn’t provided PC1 1Tx A-MPR requirements for multiple bands.

To be responsible for the specifications we are developing, without sufficient contributions and to follow the same practice of TN, we believe it is better that we do not define any A-MPR requirements for NS_24N and NS_02N for PC1 1Tx NTN UE.

Proposal 10: For PC1 1Tx, no A-MPR requirements to be specified for NS_24N and NS_02N.

Conclusion

In this contribution, we provide following proposals on NR-NTN HPUE Tx requirements:

Observation 1: It was agreed in [4] that the ΔPPowerClass can be applicable to NR-NTN.

Observation 2: It is factual that ∆TC,c is not yet applied to any NTN bands so far in Rel-19.

Observation 3: The NTN demod requirements only specified upto 1T2R for PDSCH, and CA is still pending on future release works, so that we believe the ∆TRxSRS seems not applicable to NTN Rel-19 UE spec.

Proposal 1: To confirm ∆TC,c and TRxSRS are not applicable to Rel-19 NTN UE spec.

Observation 4: Internal survey and feedback from contributing companies both claim there’s lack of alternative 1Tx PC1 PA besides the one used for TN PC1 1Tx PA.

Proposal 2: To re-use the assumption of TN PC1 1Tx PA for PC1 1Tx MPR analysis, and document it in the TR.

Proposal 3: Then confirm to re-use the TN PC1 1Tx MPR table for NTN PC1 1Tx HPUE.

Observation 5: All A-MPR analysis contributed, though some of them are different to what we have agreed or commonly assumed, are valuable for the standardization work for this and future NTN works.

Proposal 4: In general, all contributed A-MPR analysis, similar to the co-ex study, can be documented to the TR 38.863 but the associated assumptions should be clearly stated especially for those different to the agreement/WF, e.g. filter rejection level, PA models, calibration point, etc.

Observation 6: An n255 duplexer with rejection of more than 10dB was assumed in PC3 AMPR analysis for n255, which is the basis of the existing AMPR analysis for n255.

Proposal 5: For n255 NS_02N A-MPR analysis for PC2/PC1.5/PC1, expected filter attenuation of more than 10 dB below 1610 MHz needs to be assumed following the previous n255 PC3 A-MPR analysis and agreement in [1].

Observation 7: In [5], all sourcing companies proposed similar regions as showed in Figure 4.1-1 above.

Proposal 6: For n255 NS_02N, to use the above common regions summarized from multiple sourcing companies to decide A-MPR values based on analysis.

Observation 8: It is agreeable to most contributing companies to re-use PC3 A-MPR regions to discuss A-MPR values.

Proposal 7: For NS-24N for n256, re-use PC3 A-MPR regions as baseline to discuss A-MPR results. The regions can be updated if to make more RB allocations useful for NTN HPUE.

PC2 1Tx NS_24N:

Proposal 8: Take below A-MPR as starting point for NS_24N PC2 1Tx.

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | 14 | 10.5 | 9.5 | 9.5 | 25.5 | 15 | 5 |
| DFT-s-OFDM QPSK | 14 | 10.5 | 9.5 | 9.5 | 25.5 | 15 | 5 |
| DFT-s-OFDM 16 QAM | 14 | 10.5 | 9.5 | 9.5 | 25.5 | 15 | 5 |
| DFT-s-OFDM 64 QAM | 14 | 10.5 | 9.5 | 9.5 | 25.5 | 15 | 5 |
| CP-OFDM QPSK | 15.5 | 11.5 | 10 | 12.5 | 26.5 | 17.5 | 6 |
| CP-OFDM 16 QAM | 15.5 | 11.5 | 10 | 12.5 | 26.5 | 17.5 | 6 |
| CP-OFDM 64 QAM | 15.5 | 11.5 | 10 | 12.5 | 26.5 | 17.5 | 6 |

PC1.5 2Tx NS_24N:

Proposal 9: Take below A-MPR as starting point for NS_24N PC1.5 2Tx.

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | 16.5 | 13 | 13 | 11.5 | 29 | 20 | 7 |
| DFT-s-OFDM QPSK | 16.5 | 13 | 13 | 11.5 | 29 | 20 | 7 |
| DFT-s-OFDM 16 QAM | 16.5 | 13 | 13 | 11.5 | 29 | 20 | 7 |
| DFT-s-OFDM 64 QAM | 16.5 | 13 | 13 | 11.5 | 29 | 20 | 7 |
| CP-OFDM QPSK | 18.5 | 14 | 14 | 13 | 29 | 21 | 8 |
| CP-OFDM 16 QAM | 18.5 | 14 | 14 | 13 | 29 | 21 | 8 |
| CP-OFDM 64 QAM | 18.5 | 14 | 14 | 13 | 29 | 21 | 8 |

Observation 9: There is lack of RF component supporting PC1 1Tx based on the feedback from contributing companies, thus no analysis received for PC1 1Tx A-MPR for both NS_24N and NS_02N.

Observation 10: Due to the same facts we are facing in NTN, for most of the TN A-MPR requirements, it hasn’t provided PC1 1Tx A-MPR requirements for multiple bands.

Proposal 10: For PC1 1Tx, no A-MPR requirements to be specified for NS_24N and NS_02N.

Reference

[1] R4-2508008, WF on [115][128] NR_IoT_NTN_HPUE_part1, Samsung

[2] R4-2507164, Draft bigCR to TS 38.101-5 for NR-NTN HPUE, Samsung, xiaomi, Huawei, HiSilicon, Qualcomm, China Telecom, Nokia, OPPO

[3] R4-2507163, Draft bigCR to TR 38.863 for NTN HPUE, Samsung, Xiaomi, CATT, China Telecom, Ericsson, LGE, Mediatek, Nokia, OPPO, Thales, vivo, ZTE, Sanechips

[4] R4-2508007, WF on IoT-NTN HPUE, vivo

[5] R4-2510855, Summary on NTN HPUE A-MPR offline anlaysis, Samsung

[6] TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects, 3GPP

[7] TS 38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, 3GPP