3GPP TSG-RAN WG4 Meeting # 116	R4-2510857

# Bengaluru, India, 25th – 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | draft CR | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-5 for section 6.1 and 6.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Core |  |  |  |  |  | Date: |  |  | 2025-05-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce NR-NTN HPUE RF requirements to the TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To update the content for A-MPR and UE configure power section. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Tx requirement for NTN HPUE is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

6	Conducted transmitter characteristics

6.1	General

Unless otherwise stated, the transmitter characteristics for satellite access UEs are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

All requirements in this section are applicable to devices supporting GSO and/or NGSO satellites.

6.2	Transmitter power

6.2.1	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1-1: UE Power Class

| NR satellite band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n256 | 31 | +2/-3 | 293 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n255 | 31 | +2/-3 | 293 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n254 |  |  |  |  |  |  | 23 | ±2 |
| n252 |  |  |  |  |  |  | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 3 is default power class unless otherwise stated. NOTE 3: Achived via 2Tx in this version of the specification. NOTE 4: Generally, PC1 and PC1.5 UE are not targeted for smartphone form factor in this version of the specification. |  |  |  |  |  |  |  |  |

The UE shall meet the following additional requirements for maximum mean transmission power density specified in Table 6.2.1-2 when NS is signaled and when the configured channel overlaps with any portion of the specified frequency range.

Table 6.2.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum power density |
| --- | --- | --- | --- | --- |
| n254 | NS_04N | 5 | 1610 - 1618.25 | 27dBm/4kHz (mean) |
|  | NS_05N | 5 | 1618.25 - 1626.5 |  |
|  |  | 10, 15 | 1610 – 1626.5 |  |
|  | NS_11N | 5 | 1610 - 1618.25 | 15dBm/4kHz (peak limit) |
|  | NS_12N | 5 | 1618.25 - 1626.5 |  |
|  |  | 10, 15 | 1610 – 1626.5 |  |

6.2.2	UE maximum output power reduction

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 2 and 3, the allowed maximum power reduction (MPR) is defined as Table 6.2.2-2, Table 6.2.2-1 in 3GPP TS 38.101-1[5] clause 6.2.2 respectively, except for 256QAM. For UE power class 1.5 with 2Tx and power class 2 with 2Tx, the allowed maximum power reduction (MPR) is defined as Table 6.2D.2-3 and Table 6.2D.2-1 in 3GPP TS 38.101-1[5] clause 6.2D.2 respecitvely, except for 256QAM. For UE power class 1, the allowed maximum power reduction (MPR) is defined in Table 6.2.2-1. [And for UE power class with 4Tx, the allowed maximum power reduction (MPR) is defined as Table 6.2.2-1a.]

Table 6.2.2-1 Maximum power reduction (MPR) for power class 1 with 1Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w/ Rel-15 DMRS | [(NOTE 1)] | ≤ [0.5] | [0] |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | [(NOTE 1)] | [0] | [0] |
|  | QPSK | [(NOTE 1)] | ≤ [1] | [0] |
|  | 16 QAM | [(NOTE 1)] | ≤ [2] | ≤ [1] |
|  | 64 QAM | [(NOTE 1)] | ≤ [2.5] |  |
| CP-OFDM | QPSK | [(NOTE 1)] | ≤ [3] | ≤ [1.5] |
|  | 16 QAM | [(NOTE 1)] | ≤ [3] | ≤ [2] |
|  | 64 QAM | [(NOTE 1)] | ≤ [3.5] |  |
| [NOTE 1: MPR for all modulations for Edge RB allocation is defined as following for two distinguished channel bandwidths groups as: Within the <50MHz channel bandwidth group: MPR= CEIL(7.2 dB- 6 dB∙ CBW100 MHz, 0.5 dB) Within the ≥50MHz channel bandwidth group: MPR= CEIL5.35 dB+3.15 dB∙ CBW100 MHz, 0.5 dB where CEIL(x,0.5 dB) means rounding x upwards to the closest multiple of 0.5 dB.] |  |  |  |  |

[Table 6.2.2-1a Maximum power reduction (MPR) for power class 1 with 4Tx]

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [9.5] | ≤ [2.0] | ≤ [0.5] |
|  | QPSK | ≤ [10.0] | ≤ [2.5] | ≤ [0.5] |
|  | 16 QAM | ≤ [10.0] | ≤ [3.5] | ≤ [1.5] |
|  | 64 QAM | ≤ [10.0] | ≤ [4.0] | ≤ [3.5] |
| CP-OFDM | QPSK | ≤ [10.0] | ≤ [4.5] | ≤ [2.0] |
|  | 16 QAM | ≤ [10.0] | ≤ [4.5] | ≤ [2.5] |
|  | 64 QAM | ≤ [10.0] | ≤ [5.0] | ≤ [5.0] |

6.2.3	UE additional maximum output power reduction

[Editor’s note: To be amended if additional A-MPR values/NS signaling is required.]

6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR satellite band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [8].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1 except for 256QAM. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in 3GPP TS 38.101-1 [5] clause 6.2.2 except for 256QAM. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR satellite band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-1A.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2.2-1 (NOTE 3) | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_24 | 6.5.3.3.13 in 3GPP TS 38.101-1 [5] | n256 | 5, 10, 15, 20 | Table 6.2.3.15-1 in 3GPP TS 38.101-1 [5] | Clause 6.2.3.15 in 3GPP TS 38.101-1 [5]2 |
| NS_24N | 6.5.3.3.13 in 3GPP TS 38.101-1 [5] | n256 | 5, 10, 15, 20 | Table 6.2.3.8-1 [Table 6.2.3.8-3 Table 6.2.3.8-5] | Clause 6.2.3.8 |
| NS_02N | 6.5.3.3.2 | n255 | 5, 10, 15, 20 | Table 6.2.3.7-1 Table 6.2.3.7-3 [Table 6.2.3.7-5] | Clause 6.2.3.7 |
| NS_100 | 6.5.2.4.2 in 3GPP TS 38.101-1 [5] | n2561 |  |  | Table 6.2.3.1-2 in 3GPP TS 38.101-1 [5] |
| NS_03N | 6.5.3.3.3 | n254 | 5, 10, 15 |  | Clause 6.2.3.2 |
| NS_04N | 6.5.2.3.1 6.5.3.3.4 | n254 | 5 |  | Clause 6.2.3.3 |
| NS_05N | 6.5.2.3.2 6.5.3.3.4 | n254 | 5, 10, 15 |  | Clause 6.2.3.4 |
| NS_06N | 6.5.3.3.5 | n252 | 5, 10, 15, 20 |  | N/A |
| NS_07N | 6.5.3.3.6 | n2524 | 5, 10, 15, 20 |  | Clause 6.2.3.5 |
| NS_08N | 6.5.3.3.7 | n2524 | 5, 10, 15, 20 |  | Clause 6.2.3.6 |
| NS_11N | 6.5.2.3.1 6.5.3.3.4 | n254 | 5 |  | Clause 6.2.3.3 |
| NS_12N | 6.5.2.3.2 6.5.3.3.4 | n254 | 5, 10, 15 |  | Clause 6.2.3.4 |
| NOTE 1: This NS can be signalled for NR satellite bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz of the band is not specified, and therefore shall be used as a guard band. NOTE 3: The NS_01 label with the field additionalPmax [8] absent is default for all NR satellite bands. NOTE 4: The NS_07N and NS_08N is signalled based on coordination between operators and subject to regional/national regulation |  |  |  |  |  |

Table 6.2.3.1-1A: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n256 | NS_01 | NS_24 | NS_100 | NS_24N |  |  |  |  |
| n255 | NS_01 | NS_02N |  |  |  |  |  |  |
| n254 | NS_01 | NS_03N | NS_04N | NS_05N | NS_11N | NS_12N |  |  |
| n252 | NS_01 | NS_06N | NS_07N | NS_08N |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. |  |  |  |  |  |  |  |  |

6.2.3.2	A-MPR for NS_03N

Table 6.2.3.2-1: A-MPR regions for NS_03N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc < 1621.5 |  | <= 6.48 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  | fc = 1621.5 |  | >= 7.2 | A1 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

Table 6.2.3.2-2: A-MPR for NS_03N

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.0 | 1.0 | 4.0 | 6.5 | 1.5 |
|  | QPSK | 2.5 | 4.0 | 2.5 | 6.0 | 7.0 | 2.0 |
|  | 16QAM | 3.0 | 4.5 | 3.0 | 6.5 | 7.5 | 2.5 |
|  | 64QAM | 3.5 | 5 | 3.5 | 7 | 8 | 3 |
| CP-OFDM | QPSK | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 16QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 64QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |

6.2.3.3	A-MPR for NS_04N and NS_11N

Table 6.2.3.3-1: A-MPR regions for NS_04N and NS_11N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |

Table 6.2.3.3-2: A-MPR for NS_04N and NS_11N

|  | Modulation | A1 | A2 | A3 |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.0 | 1.0 |
|  | QPSK | 2.5 | 4.0 | 2.5 |
|  | 16QAM | 3.0 | 4.5 | 3.0 |
|  | 64QAM | 3.5 | 5 | 3.5 |
| CP-OFDM | QPSK | 3.5 | 6.0 | 4.0 |
|  | 16QAM | 3.5 | 6.0 | 4.0 |
|  | 64QAM | 3.5 | 6.0 | 4.0 |

6.2.3.4	A-MPR for NS_05N and NS_12N

Table 6.2.3.4-1: A-MPR regions for NS_05N and NS_12N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1622.4 < fc <= 1624 | >= 3.6 | > 0 | A3 |
|  |  |  | >= 2.88 | A1 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc <= 1621.5 |  | <= 7.2 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  |  | > 7.2 | > 0 | A6 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

Table 6.2.3.4-2: A-MPR for NS_05N and NS_12N

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 1.5 | 5.0 | 1.5 | 6.5 | 6.5 | 2.0 |
|  | QPSK | 1.5 | 5.0 | 1.5 | 6.5 | 7.0 | 2.5 |
|  | 16QAM | 2.0 | 5.5 | 2.0 | 7.0 | 7.5 | 3.0 |
|  | 64QAM | 2.5 | 6 | 2.5 | 7.5 | 8 | 3.5 |
| CP-OFDM | QPSK | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |
|  | 16QAM | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |
|  | 64QAM | 3.0 | 6.5 |  | 8.0 | 10.0 | 4.5 |

6.2.3.5	A-MPR for NS_07N

Table 6.2.3.5-1: A-MPR regions for NS_07N

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | < 2007.5 | ≥ 0 | ≥ 2.16 | A1 |
| 10MHz | < 2010 | ≤ 1.44 | ≥ 0 | A2 |
|  |  | > 1.44 | ≥ 3.6 | A1 |
|  |  | > 7.74 | ≤ 0.72 | A1 |
|  | 2010 ≤ Fc ≤ 2015 | ≥ 0 | ≥ 4.5 | A1 |
| 15MHz | < 2012.5 | ≤ 3.24 | ≥ 0 | A3 |
|  |  | > 3.24 | ≥ 4.5, ≤ 7.2 | A1 |
|  |  | > 3.24 | > 7.2 | A2 |
|  |  | > 10.8 | ≤ 0.72 | A1 |
|  | 2012.5 | ≤ 1.44 | ≥ 0 | A2 |
|  |  | > 1.44 | ≥ 6.48 | A1 |
|  |  | > 12.6 | ≤ 0.72 | A1 |
| 20 MHz | 2010 | ≤ 4.68 | ≥ 0 | A3 |
|  |  | > 4.68 | ≥ 4.5, ≤ 7.2 | A1 |
|  |  | > 4.68 | > 7.2 | A2 |
|  |  | > 12.6 | ≤ 0.72 | A1 |

Table 6.2.3.5-2: A-MPR for NS_07N

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 | ≤ 7 | ≤ 11 |
| DFT-s-OFDM QPSK | ≤ 4.5 | ≤ 7.5 | ≤ 11.5 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 8 | ≤ 12 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 8 | ≤ 12 |
| CP-OFDM QPSK | ≤ 7 | ≤ 9 | ≤ 13 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 9 | ≤ 13 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 9 | ≤ 13 |

6.2.3.6	A-MPR for NS_08N

Table 6.2.3.6-1: A-MPR regions for NS_08N

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5 MHz | - | - | - | - |
| 10MHz | < 2010 | ≤ 1.44 | ≤ 2.7 | A1 |
|  |  | ≤ 1.44 | > 2.7, < 4.5 | A2 |
|  |  | ≤ 1.44 | ≥ 4.5 | A1 |
|  |  | ≥ 1.44 | ≥ 5.4 | A2 |
| 15MHz | < 2012.5 | ≤ 3.24 | ≤ 2.7 | A1 |
|  |  | ≤ 3.24 | > 2.7, < 4.5 | A2 |
|  |  | ≤ 3.24 | ≥ 4.5 | A1 |
|  |  | ≥ 3.24 | ≥ 5.4 | A2 |
|  | 2012.5 | ≤ 1.44 | ≤ 2.7 | A1 |
|  |  | ≤ 1.44 | > 2.7, < 5.4 | A2 |
|  |  | ≤ 1.44 | ≥ 5.4 | A1 |
|  |  | ≥ 1.44 | ≥ 7.2 | A2 |
| 20 MHz | 2010 | ≤ 4.68 | ≤ 3.6 | A1 |
|  |  | ≤ 4.68 | > 3.6, ≤ 5.4 | A2 |
|  |  | ≤ 4.68 | > 5.4 | A1 |
|  |  | > 4.68 | ≥ 7.2 | A2 |

Table 6.2.3.6-2: A-MPR for NS_08N

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 | ≤ 2.5 |
| DFT-s-OFDM QPSK | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 4 |
| CP-OFDM QPSK | ≤ 7 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 4.5 |

6.2.3.7	A-MPR for NS_02N

[Editor’s Note: Place holders for NS_02N A-MPR]

Table 6.2.3.7-1 A-MPR regions for NS_02N (Power Class 2)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5 MHz | TBA | TBA | TBA | TBA |
| 10 MHz | TBA | TBA | TBA | TBA |
| 15 MHz | TBA | TBA | TBA | TBA |
| 20 MHz | TBA | TBA | TBA | TBA |

Table 6.2.3.7-2 A-MPR for NS_02N (Power Class 2)

| Modulation/Waveform | TBA |
| --- | --- |
|  | TBA |
| DFT-s-OFDM PI/2 BPSK | TBA |
| DFT-s-OFDM QPSK | TBA |
| DFT-s-OFDM 16 QAM | TBA |
| DFT-s-OFDM 64 QAM | TBA |
| CP-OFDM QPSK | TBA |
| CP-OFDM 16 QAM | TBA |
| CP-OFDM 64 QAM | TBA |

Table 6.2.3.7-3 A-MPR regions for NS_02N (Power Class 1.5)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5 MHz | TBA | TBA | TBA | TBA |
| 10 MHz | TBA | TBA | TBA | TBA |
| 15 MHz | TBA | TBA | TBA | TBA |
| 20 MHz | TBA | TBA | TBA | TBA |

Table 6.2.3.7-4 A-MPR for NS_02N (Power Class 1.5)

| Modulation/Waveform | TBA |
| --- | --- |
|  | TBA |
| DFT-s-OFDM PI/2 BPSK | TBA |
| DFT-s-OFDM QPSK | TBA |
| DFT-s-OFDM 16 QAM | TBA |
| DFT-s-OFDM 64 QAM | TBA |
| CP-OFDM QPSK | TBA |
| CP-OFDM 16 QAM | TBA |
| CP-OFDM 64 QAM | TBA |

[Table 6.2.3.7-5 A-MPR regions for NS_02N (Power Class 1)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5 MHz | TBA | TBA | TBA | TBA |
| 10 MHz | TBA | TBA | TBA | TBA |
| 15 MHz | TBA | TBA | TBA | TBA |
| 20 MHz | TBA | TBA | TBA | TBA |

Table 6.2.3.7-6 A-MPR for NS_02N (Power Class 1)

| Modulation/Waveform | TBA |
| --- | --- |
|  | TBA |
| DFT-s-OFDM PI/2 BPSK | TBA |
| DFT-s-OFDM QPSK | TBA |
| DFT-s-OFDM 16 QAM | TBA |
| DFT-s-OFDM 64 QAM | TBA |
| CP-OFDM QPSK | TBA |
| CP-OFDM 16 QAM | TBA |
| CP-OFDM 64 QAM | TBA |

]

6.2.3.8	A-MPR for NS_24

Table 6.2.3.8-1 A-MPR regions for NS_24N (Power Class 2)

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | >3.24 | A4 |  |  |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | >1.98 | A1 | >3.6 | >1.08 ≤1.98 | A2 | ≤3.6 | ≤1.98 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1975 < Fc ≤ 1985 | >5.4 |  | A4 |  |  |  |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | >4.32 | A1 | ≥7.2 | >1.08 ≤4.32 | A2 | <7.2 | ≤4.32 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥5.76 |  | A5 | <3.06 |  | A5 | ≥3.06 <5.76 | >1.44 | A6 |
| 15MHz | 1972.5 < Fc ≤ 1987.5 |  | >6.84 | A1 | ≥10.8 | >1.08 ≤6.84 | A2 | <10.8 | ≤6.84 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | A5 | <3.78 |  | A5 | ≥3.78 <8.64 | >1.44 | A6 |
| 20MHz | 1970 < Fc ≤ 1990 | ≥12.96 |  | A5 | <4.68 |  | A5 | ≥4.68 <12.96 | >2.16 | A6 |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | A5 | <5.58 |  | A5 | ≥5.58 <11.52 | >1.44 | A6 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.8-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.8-2 A-MPR for NS_24N (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | 14 | 10.5 | 9.5 | 9.5 | 25.5 | 15 | 5 |
| DFT-s-OFDM QPSK | 14 | 10.5 | 9.5 | 9.5 | 25.5 | 15 | 5 |
| DFT-s-OFDM 16 QAM | 14 | 10.5 | 9.5 | 9.5 | 25.5 | 15 | 5 |
| DFT-s-OFDM 64 QAM | 14 | 10.5 | 9.5 | 9.5 | 25.5 | 15 | 5 |
| CP-OFDM QPSK | 15.5 | 11.5 | 10 | 12.5 | 26.5 | 17.5 | 6 |
| CP-OFDM 16 QAM | 15.5 | 11.5 | 10 | 12.5 | 26.5 | 17.5 | 6 |
| CP-OFDM 64 QAM | 15.5 | 11.5 | 10 | 12.5 | 26.5 | 17.5 | 6 |

Table 6.2.3.8-3 A-MPR regions for NS_24N (Power Class 1.5)

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | >3.24 | A4 |  |  |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | >1.98 | A1 | >3.6 | >1.08 ≤1.98 | A2 | ≤3.6 | ≤1.98 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1975 < Fc ≤ 1985 | >5.4 |  | A4 |  |  |  |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | >4.32 | A1 | ≥7.2 | >1.08 ≤4.32 | A2 | <7.2 | ≤4.32 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥5.76 |  | A5 | <3.06 |  | A5 | ≥3.06 <5.76 | >1.44 | A6 |
| 15MHz | 1972.5 < Fc ≤ 1987.5 |  | >6.84 | A1 | ≥10.8 | >1.08 ≤6.84 | A2 | <10.8 | ≤6.84 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | A5 | <3.78 |  | A5 | ≥3.78 <8.64 | >1.44 | A6 |
| 20MHz | 1970 < Fc ≤ 1990 | ≥12.96 |  | A5 | <4.68 |  | A5 | ≥4.68 <12.96 | >2.16 | A6 |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | A5 | <5.58 |  | A5 | ≥5.58 <11.52 | >1.44 | A6 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.8-4. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.8-4 A-MPR for NS_24N (Power Class 1.5)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | 16.5 | 13 | 13 | 11.5 | 29 | 20 | 7 |
| DFT-s-OFDM QPSK | 16.5 | 13 | 13 | 11.5 | 29 | 20 | 7 |
| DFT-s-OFDM 16 QAM | 16.5 | 13 | 13 | 11.5 | 29 | 20 | 7 |
| DFT-s-OFDM 64 QAM | 16.5 | 13 | 13 | 11.5 | 29 | 20 | 7 |
| CP-OFDM QPSK | 18.5 | 14 | 14 | 13 | 29 | 21 | 8 |
| CP-OFDM 16 QAM | 18.5 | 14 | 14 | 13 | 29 | 21 | 8 |
| CP-OFDM 64 QAM | 18.5 | 14 | 14 | 13 | 29 | 21 | 8 |

[Table 6.2.3.8-5 A-MPR regions for NS_24N (Power Class 1)

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5 MHz | TBA | TBA | TBA | TBA |
| 10 MHz | TBA | TBA | TBA | TBA |
| 15 MHz | TBA | TBA | TBA | TBA |
| 20 MHz | TBA | TBA | TBA | TBA |

Table 6.2.3.8-6 A-MPR for NS_24N (Power Class 1)

| Modulation/Waveform | TBA |
| --- | --- |
|  | TBA |
| DFT-s-OFDM PI/2 BPSK | TBA |
| DFT-s-OFDM QPSK | TBA |
| DFT-s-OFDM 16 QAM | TBA |
| DFT-s-OFDM 64 QAM | TBA |
| CP-OFDM QPSK | TBA |
| CP-OFDM 16 QAM | TBA |
| CP-OFDM 64 QAM | TBA |

]

6.2.4	Configured transmitted power

The requirements for configured transmitted power defined in subclause 6.2.4 of 3GPP TS 38.101-1 [5] clause 6.2.4 shall apply to NTN satellite UE, with the following notes:

- PPowerClass is the maximum UE power specified in Table 6.2.1-1;

- MPRc and A-MPRc for serving cell c are specified in clause 6.2.2 and clause 6.2.3, respectively;

- ∆TC,c, ΔTIB,c, ∆TRxSRS are not applicable to NR-NTN UE;

<End of change>