3GPP TSG-RAN WG4 Meeting # 116	R4-2510860

# Bengaluru, India, 25th – 29th August, 2025

Title:	Discussion on Feature list for NR_IoT_NTN_req_test_enh

Source:	Samsung

Agenda item:	7.6.2

Document for:	Approval

Introduction

In last RAN4 #115 meeting, due to the on-going discussion in several open issues, the feature list was not provided for this work item NR_IoT_NTN_req_test_enh. With more agreements and WFs captured in [1] and [2], and also considering the WFs from the relevant thread, in this paper, we propose the feature list for this WI.

Feature list

In [1], the way forward for feature list discussion is as follows:

Issue 3-1: Feature list Way forward: Encourage companies to check the following feature list proposals: - For NTN less than 5MHz, there’s no need to introduce new UE capabilities with same purpose in Rel-19. - For NR-NTN HPUE, there’s no need to introduce new UE capabilities. - SAR related pending on final agreement. - For IoT-NTN HPUE, further discuss.

For the remaining issues on SAR and IoT-NTN HPUE, [2] have achieved further agreements as follows:

Agreements for SAR in [2]: - WF: - Keep the same status as TN for respective base RAT - NR-NTN: Reuse NR TN, - Including ΔPPowerClass in configured transmitted power clause - eMTC based Iot-NTN: Reuse eMTC - Not precluding ΔPPowerClass in configurated transmitted power clause - NB-Iot based Iot-NTN: Reusing NB-Iot

Agreements for IoT-NTN HPUE that related to feature list in [2]: - WF(NEW): - Restrict the boosting condition to PC2 NB-Iot based Iot-NTN - Increase [0.8] dB MOP or add a power boosting item in Pcmax equation (optional with capability signalling)

With above, we can conclude the following new optional with capability signalling feature to be introduced under this WI.

Proposal:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD diff. | Need of FR1/FR2 diff. | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_IoT_NTN_req_test_enh | 61-1 | Tx PC2 output power limit relaxation for NTN NB-IoT UL | Indicate the supported PC2 Tx output power limit relaxation for NTN NB-IoT UL in each FR1 band | IoT NTN | Yes | N/A | Legacy Tx output power upper limitation still apply | Per (UL) band: UE signals supported upper power limit relaxation | FDD | FR1 only | FDD |  | Optional with capability signalling |

Conclusion

In this contribution, we proposed above new feature as the feature list for NR_IoT_NTN_req_test_enh WI in Rel-19.

Reference

[1] R4-2508008, WF on [115][128] NR_IoT_NTN_HPUE_part1, Samsung

[2] R4-2508007, WF on IoT-NTN HPUE, vivo