3GPP TSG-RAN WG4 Meeting #116	R4-2510863

Bengaluru, India, 25th – 29th August, 2025

Agenda item:	7.1.2.3.2

Source:            	Google

Title:                	Discussion on the remaining issue for 6Rx UE

Document for:	Approval

# Introduction

In RAN#108, a revised WID on RF enhancements for NR FR1/FR2 and EN-DC was approved as one of Release 19 work items [1]. One of the objectives in the WID is to discuss 6Rx requirements for handheld and FWA UEs, which is shown as follows. In this contribution, we will focus on the 6 MIMO layers support for 6Rx handheld and FWA UEs.

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Introduce 6 MIMO layers support as an optional feature only for FWA. - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In RAN4#115, the 6 MIMO layers support for 6Rx handheld and FWA UEs has been discussed and the remaining issue is captured in the approved WF [3]. In WF, the release independency for 6Rx handheld and FWA UEs is still not determined. In this paper, we would like to share our view in the following.

# <Topic #2> MIMO layer evaluation for 6Rx UE ## <Sub-topic 2-1> Release independency for 6Rx UE Agreement: - Option 1: - Support 6Rx with up to 4 MIMO layers for both handheld UE and FWA from Rel-18 - Support 6Rx with up to 6 MIMO layers for FWA from Rel-19 - Option 2: - Support 6Rx from Rel-19 - Option 3: - Support 6Rx from Rel-18

# Discussion

In LS [2], the requirements to support of DL MIMO layers operation for 6Rx UE has been compromised as the following solutions. The 6-layer performance requirements will be introduced for FWA UE device only and 4-layer performance requirements will be introduced for FWA and handheld UE devices.

RAN4 discussed the support of DL MIMO layers operation for UEs equipped with 6Rx chains for FWA and handheld devices and reached the following conclusions in RAN4 #114: - 6-layer performance requirements will be only defined for FWA (Fixed Wireless Access) UE devices. - 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices. - The fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications

In RAN4#115, the release independency for 6Rx UE continue being discussed and three possible options are captured in the WF. The option 1 is to support 6Rx with up to 4-layer for handheld and FWA UEs release independent from Rel-18. The option 2 is to support 6Rx from Rel-19 and the option 3 is to support 6Rx from Rel-18.

In our understanding, release independence captured in TS 38.307 is a mechanism to allows a User Equipment (UE) to support a specific feature defined in a newer release without complying all the requirements of that entire new release. In current specification, 4Rx for handheld UE and FWA UE can be release independent from Rel-15 and 8Rx for FWA UE can be release independent from Rel-17. For example, if a UE want to support 8Rx features, the UE only needs to follow all Rel-17 specifications as minimum requirement but does not need to follow any newer release specification although some 8Rx requirements is finished in Rel-18.

As for 6Rx UE, RAN4 made an agreement in the previous meeting to differentiate the 4-layer and 6-layer performance requirements from different UE form factors, i.e., FWA UE or handheld UE. Based on this consensus, a compromised solution for release independence is proposed to support 6Rx UE with 4-layer from Rel-18 and 6Rx UE with 6-layer from Rel-19 in the WF.

From 6Rx UE with 6-layer perspective, it is pointed out in [2][4] that a new signaling is introduced by RAN2 to indicate 6-layer for PDSCH MIMO layer in Rel-19. For a FWA UE to support 6Rx with 6-Layer, it seems that this UE needs also to implement Rel-19 UE capability signaling to indicate 6-layer to the network. Which means that it is reasonable to introduce 6Rx UE with 6-layer release independent from Rel-19.

FeatureSetDownlinkPerCC-v19xy ::=           SEQUENCE {

maxNumberMIMO-6LayersPDSCH-r19				ENUMERATED {supported}					OPTIONAL

}

From 6Rx UE with 4-layer perspective, both handheld UE and FWA UE can be applied to the performance requirements. Since the existed signaling can be reused for 6Rx with 4-layer, there might not be an issue to support 6Rx with 4-layer release independent from the previous release. However, we have different views for handheld UE from antenna design perspective. So far as we know, there is very limited 6Rx handheld UE without certification in the market. Based on the current commercial UE design, it would be challenging for the handheld UE to support 6Rx without much changes in form factors design. Especially for the high-end UE, it is equipped with physical antennas as many as possible to accommodate all the requested bands from operators in the world. Hence, if the handheld UE supports 6Rx, two additional antennas and/or legacy antenna sharing may be possible solutions which may have much impact to antenna correlation. Although the impact can be reduced for the foldable phone form factor, there still might be an issue that the antenna correlation may not be reduced or even higher than non-foldable handheld if the foldable UE is in the folded/close mode. Hence, we think it would be beneficial for the handheld UE vendor to have more time to investigate 6Rx antenna development and it is preferred to introduce 6Rx with 4-layer release independent from Rel-19.

In our view, based on the above discussion, it is straightforward for 6Rx UE with 6-layer to be release independent from Rel-19 due to Rel-19 new signaling introduction. For 6Rx UE with 4-layer, although the existed signaling can be reused, the antenna implementation complexity would be much higher for the handheld 6Rx UE. Hence, it would be beneficial for the handheld UE vendor to have more time to investigate 6Rx antenna development. Considering that 6 Rx with 6-layer FWA UE requires Rel-19 new signaling and 6 Rx with 4-layer handheld UE face significant antenna implementation challenges, it is preferred to align the introduction of both form factors for 6Rx UE with the same release independent from Rel-19.

Proposal 1: Considering that 6 Rx with 6-layer FWA UE requires Rel-19 new signaling and 6 Rx with 4-layer handheld UE face significant antenna implementation challenges, it is preferred to align the introduction of both form factors for 6Rx UE in the same release independent from Rel-19.

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: Considering that 6 Rx with 6-layer FWA UE requires Rel-19 new signaling and 6 Rx with 4-layer handheld UE face significant antenna implementation challenges, it is preferred to align the introduction of both form factors for 6Rx UE in the same release independent from Rel-19.

# Reference

- RP-251816, “Revised WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, Huawei, RAN#108

- R4-2505225, “LS on DL MIMO layers capabilities for 6Rx UEs”, Intel, RAN4#114-bis

- R4-2420376, “WF on 6Rx and SRS IL imbalance”, Huawei, HiSilicon, RAN4#115

- R4-2507528, “Views on remaining issue for 6Rx UE”, Samsung, RAN4#115