3GPP TSG-RAN WG4 Meeting #116	R4-2510868

# Bengaluru, IN, 25 - 29 Aug, 2025

Title: 	Discussion on SBFD demodulation performance requirements

Source: 	Huawei,HiSilicon

Agenda item:	7.21.6

Document for:	Discussion

Background

During RAN4#115 meeting, Way Forward [1] for NR_duplex_evo_demod was approved. In this contribution, we share our view on SBFD demodulation performance requirements.

Discussion

UE side

Whether to introduce UE demodulation with SBFD carrier (with DL sub-band and UL sub-band configured)

- Way forward - Option 1: Define limited number of additional SBFD cases - Other options are not precluded

From our understanding, a UE that supporting SBFD carrier also support TDD carrier, so it is enough to verify the performance for a UE supporting SBFD by testing the existing TDD cases and limited number of additional SBFD cases. There is no need to define lots of cases independent of TDD.

Define limited number of additional SBFD cases.

Number of DL sub-band if introduced UE demodulation

- Way forward - Option 1 - Consider one DL sub-band and one UL sub-band case as high priority. FFS whether to consider two DL sub-band and one UL sub-band case based on RAN1 agreements - Other options are not precluded

RAN1 is still under discussing whether two DL subband and one UL subband case is an optional feature based on UE capability. So we propose to consider one DL subband and one UL subband case as high priority. FFS whether to consider two DL subband and one UL subband case based on RAN1 agreement.

Consider one DL subband and one UL subband case as high priority. FFS whether to consider two DL subband and one UL subband case based on RAN1 agreement.

TDD pattern

- Way forward - Option 1 - 30khz: XXXXXXXXUU - 120KHz: XXXXU - Other options are not precluded

In RAN4 demodulation performance requirements definition and in the real network deployment, DDDDDDDSUU for 30kHz SCS and DDDSU for 120kHz SCS is typical used. To be compatible with the above TDD patterns, we propose to use XXXXXXXXUU for 30kHz SCS and XXXXU for 120kHz for RAN4 SBFD performance demodulation requirements definition.

Use XXXXXXXXUU for 30kHz SCS and XXXXU for 120kHz for RAN4 SBFD performance demodulation requirements definition.

Channel bandwidth & SCS

- Way forward: - Option 1 - FR1 with 30KHz, 40MHz DL sub-band + 20MHz UL sub-band for 30kHz SCS - FR2-1 with 120KH, 100MHz DL sub-band + 100MHz UL sub-band for 120kHz SCS - Other options are not precluded

For the channel bandwidth and SCS, we propose to use the typical value for the existing TDD cases in TS 38.101-4, i.e. 40MHz DL subband + 20MHz UL subband for 30kHz SCS and 100MHz DL subband + 100MHz UL subband for 120kHz SCS.

Use the typical value for the existing TDD cases in TS 38.101-4, i.e. 40MHz DL subband + 20MHz UL subband for 30kHz SCS and 100MHz DL subband + 50MHz UL subband for 120kHz SCS.

MCS & Rank

- Way forward: - Option 1 - MCS 13 and rank 2 - Other options are not precluded

For the MCS and rank, one typical value can be selected, e.g. MCS 13 and rank 2.

For PDSCH demodulation performance requirements, use MCS 13 and rank 2.

Propagation condition

- Way forward: - Option 1 - TDLC300-100 - Other options are not precluded

For the propagation condition, one typical value can be selected, e.g. TDLC300-100.

For PDSCH demodulation performance requirements, use TDLC300-100.

HARQ feedback

- Way forward: - Option 1 - use the HARQ timeline for XXXXXXXXUU TDD pattern for 30kHz SCS and XXXXU TDD pattern for 120kHz SCS as above Figure 2.1.2-1 and Figure 2.1.2-2 respectively Figure 2.1.2-1 HARQ timeline for XXXXXXXXUU TDD pattern for 30kHz SCS Figure 2.1.2-2 HARQ timeline for XXXXU TDD pattern for 120kHz SCS - Other options are not precluded

Considering half-duplex operation and faster HARQ feedback, the HARQ timeline for XXXXXXXXUU TDD pattern for 30kHz SCS and XXXXU TDD pattern for 120kHz SCS is proposed as Option 1.

For PDSCH demodulation performance requirements, use the HARQ timeline for XXXXXXXXUU TDD pattern for 30kHz SCS and XXXXU TDD pattern for 120kHz SCS as Option 1.

Number of HARQ processes

- Way forward : - Option 1 - Using 4 HARQ processes for both TDD pattern - Other options are not precluded

Based on discussion on section 2.7, 4 HARQ processes is enough for both TDD pattern.

Use 4 HARQ processes for both TDD pattern.

Other parameters

- Way forward : - Option 1 - For PDSCH demodulation performance requirements, use the parameters as above Table 2.1.2-1 Parameter; Unit; Value. Duplex mode; TDD. Active DL BWP index; 1. PDSCH configuration; Mapping type; Type A. k0; 0. Starting symbol (S); 2. Length (L); 12. PDSCH aggregation factor; 1. PRB bundling type; Static. PRB bundling size; 2. Resource allocation type; Type 0. RBG size; Config2. VRB-to-PRB mapping type; Non-interleaved. VRB-to-PRB mapping interleaver bundle size; N/A. PDSCH DMRS configuration; DMRS Type; Type 1. Number of additional DMRS; 1. Maximum number of OFDM symbols for DL front loaded DMRS; 1. - Other options are not precluded

For other parameters, we propose to use the typical PRB bundling value for the existing TDD cases in TS 38.101-4, as Option 1.

For PDSCH demodulation performance requirements, use the parameters as Option 1.

BS side

General Scope

- Way forward - Introduce PUSCH demodulation requirement with UL resource muting configured with interference modelling - FFS on which type(s) of interference will be considered to specify the PUSCH requirement - gNB self-interference - co-site inter-sector interference - inter-site gNB-gNB co-channel inter-subband CLI - FFS on how to model interference - gNB self-interference - Option 1: modelling as additive white gaussian noise with fixed INR=-6dB targeting 1dB desense as starting pointing - Other options are not precluded - inter-sector and inter-site interference - Option 1: modelling as additive white gaussian noise with fixed INR = - X dB based on assumption of co-site isolation - Other options are not precluded - inter-site gNB-gNB co-channel inter-sub-band CLI

For the type of interference to be modelled, based on the WID as following, only CLI is considered, that means gNB self-interference is out of scope.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD.

gNB self-interference is out of scope.

Based on RAN1 discussion, the main use for the muting resource is gNB-gNB CLI handling, e.g. calculate interference covariance for better interference suppression to get better demodulation performance. However, we don’t see RAN1 prohibit the use of resource muting for other types of interference, e.g. inter-sector and inter-site interference in above option.

Agreement If non-transparent UL resource muting is supported for interference covariance matrix measurement for gNB-to-gNB CLI handling, the following are recommended to be specified - Definition and indication of UL resource muting pattern - Collision with DMRS/PTRS - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination - Power allocation in symbols with muted REs considering potential impact to phase continuity - TB size determination Note: The existing reference signal time-frequency resource pattern, e.g., PT-RS, comb-2 SRS, are the candidates for the UL resource muting pattern. Note: Consider pattern without adverse impact on PAPR Note: The potential impact on transmit signal quality/MPR requirement may need to checked with RAN4. Note: The above does not apply for PUSCH transmission during random access procedures.

For the above two type of interference (inter-sector and inter-site interference, inter-site gNB-gNB co-channel inter-sub-band CLI), both of them can be modeled as additive white gaussian noise with fixed INR=-X dB that X can be defined based on the simulation, i.e. find the X that we can observe obvious performance difference between UL resource muting enabled or not.

Model CLI (including inter-sector and inter-site interference, inter-site gNB-gNB co-channel inter-sub-band CLI) as additive white gaussian noise with fixed INR = -X dB that X can be defined based on the simulation, i.e. find the X that we can observe obvious performance difference between UL resource muting enabled or not.

Frequency Range

- Way forward - Option 1 - FR1 and FR2-1 - Other options are not precluded

We preferred to consider 30kHz SCS for FR1 and 120kHz SCS FR2-1 that is more typical in the real network deployment.

Consider 30kHz SCS for FR1 and 120kHz SCS FR2-1 that is more typical in the real network deployment.

Channel Bandwidth

- Way forward - Option 1 - prioritize the minimum bandwidth sizes of SBFD for both FR1 (40 MHz) and FR2-1 (100 MHz) NR TDD bands - if additional bandwidth is required, 100MHz for FR1 - Option 2 - FR1 with 30KHz, 40MHz DL sub-band + 20MHz UL sub-band for 30kHz SCS - FR2-1 with 120KH, 100MHz DL sub-band + 100MHz UL sub-band for 120kHz SCS - Other options are not precluded

For the channel bandwidth and SCS, we propose to use the typical value, i.e. 40MHz DL subband + 20MHz UL subband for 30kHz SCS and 100MHz DL subband + 100MHz UL subband for 120kHz SCS.

Use the typical value, i.e. 40MHz DL subband + 20MHz UL subband for 30kHz SCS and 100MHz DL subband + 50MHz UL subband for 120kHz SCS.

TDD pattern

- Way forward - Option 1 - 30 kHz SCS: XXXXXXXXUU - 120 kHz SCS: XXXXU - Other options are not precluded

In RAN4 demodulation performance requirements definition and in the real network deployment, DDDDDDDSUU for 30kHz SCS and DDDSU for 120kHz SCS is typical used as the default TDD pattern. To be compatible with the above TDD patterns, we propose to use XXXXXXXXUU for 30kHz SCS and XXXXU for 120kHz as the default TDD pattern for RAN4 SBFD performance demodulation requirements definition.

Use XXXXXXXXUU for 30kHz SCS and XXXXU for 120kHz as the default TDD pattern for RAN4 SBFD performance demodulation requirements definition.

Waveform

- Way forward - Option 1 - Both DFT-s-OFDM and CP-OFDM - Other options are not precluded

Based on RAN1 agreements, UL resource muting for PUSCH is supported, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain. So we propose to consider both DFT-S-OFDM and CP-OFDM.

Consider both DFT-S-OFDM and CP-OFDM.

Inclusion in specification

- Way forward - Option 1: New sub-clause for demodulation requirements under same chapter with RF requirements can be considered. If RAN4 agree Not to introduce new BS demodulation requirements for SBFD, only applicability rule can be captured. - Other options are not precluded

We think it is better to discuss this issue later.

For inclusion in specification, discuss this issue later.

Proposals

In this contribution, we share our view on SBFD demodulation performance requirements. Our observations and proposals are:

- Define limited number of additional SBFD cases.

Consider one DL subband and one UL subband case as high priority. FFS whether to consider two DL subband and one UL subband case based on RAN1 agreement.

Use XXXXXXXXUU for 30kHz SCS and XXXXU for 120kHz for RAN4 SBFD performance demodulation requirements definition.

Use the typical value for the existing TDD cases in TS 38.101-4, i.e. 40MHz DL subband + 20MHz UL subband for 30kHz SCS and 100MHz DL subband + 50MHz UL subband for 120kHz SCS.

For PDSCH demodulation performance requirements, use MCS 13 and rank 2.

For PDSCH demodulation performance requirements, use TDLC300-100.

For PDSCH demodulation performance requirements, use the HARQ timeline for XXXXXXXXUU TDD pattern for 30kHz SCS and XXXXU TDD pattern for 120kHz SCS as Option 1.

Use 4 HARQ processes for both TDD pattern.

For PDSCH demodulation performance requirements, use the parameters as Option 1.

gNB self-interference is out of scope.

Model CLI (including inter-sector and inter-site interference, inter-site gNB-gNB co-channel inter-sub-band CLI) as additive white gaussian noise with fixed INR = -X dB that X can be defined based on the simulation, i.e. find the X that we can observe obvious performance difference between UL resource muting enabled or not.

Consider 30kHz SCS for FR1 and 120kHz SCS FR2-1 that is more typical in the real network deployment.

Use the typical value, i.e. 40MHz DL subband + 20MHz UL subband for 30kHz SCS and 100MHz DL subband + 50MHz UL subband for 120kHz SCS.

Use XXXXXXXXUU for 30kHz SCS and XXXXU for 120kHz as the default TDD pattern for RAN4 SBFD performance demodulation requirements definition.

Consider both DFT-S-OFDM and CP-OFDM.

For inclusion in specification, discuss this issue later.

Reference

R4-2508723, Way Forward for [115][330] NR_duplex_evo_demod, RAN4#115, Samsung