3GPP TSG-RAN WG4 Meeting #116	R4-2510878

Bengaluru, India, August 25 – 29, 2025

Agenda Item:	7.18.3

Source:	Huawei, HiSilicon

Title:	Discussion on testing issues for CSI compression two-sided models

Document for:	Discussion and Decision

# Introduction

According to WF in RAN4 #115 [1] and [2], we discuss the remaining issues related to CSI compression two-sided models.

# Simulation results on low-complexity encoders

RAN4 #115 WF [1]: Further simulations (not agreement): In order to further elaborate option 3 track 2 considering lower complexity encoders, interested companies may contribute simulations to RAN4#116. The following guidance, and also the guidance in [2] R4-2508084 is provided and recommended for companies to follow in order to obtained aligned simulations. For option 3, track 2 consider the following investigations for lower complexity encoder structures. - Performance assuming training and testing with the Samsung frozen decoder - Performance achieved with a jointly trained low complexity encoder and decoder - Performance achieved when a frozen decoder is trained based on all of the encoder structures (i.e. different encoder complexity) - Consider transformer, CNN, MLP

According to the low-complexity model structures for encoders and model training procedures assumed in [2], simulation results are provided in the following.

Proposal 1: SGCS results for low-complexity encoders are provided in Table 1.

Table 1. Simulation results for low-complexity encoders

| Option 3 low-complexity encoders with different backbones |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | （1-to-1）Joint training | New encoder training with (Samsung’s) frozen decoder | （N-to-1）Joint training SGCS-4 Step-1 | New encoder training with frozen decoder from Step-1 SGCS-4 Step-2 |
| Encoder-1 | Companies' name: Huawei, Hisilicon | 0.71 (SGCS-0) | 0.704 (SGCS-1) | 0.6759 (simulation results from RAN4#115, where only Encoder-1,2,3 are considered, n1=n2 ) | 0.6957 (RAN4#115) (Results for RAN#116 to be provided during the meeting week) |
|  |  |  |  | 0.7128 |  |
|  |  |  |  | (simulation results updated for RAN4#116, with Encoder-1,2,3,4 considered, n1=0.9, n2=0.1) |  |
| Encoder-2 (TF) | Companies' name: Huawei, Hisilicon | 0.6793 (SGCS-3) | 0.6785 (SGCS-2) | 0.6669 (RAN4#115) | 0.6711 (RAN4#115) (Results for RAN#116 to be provided during the meeting week) |
|  |  |  |  | 0.6841 (RAN4#116) |  |
| Encoder-3 (CNN) | Companies' name: Huawei, Hisilicon | 0.6911 (SGCS-3) | 0.6852 (SGCS-2) | 0.6745 (RAN4#115) | 0.6842 (RAN4#115) |
|  |  |  |  | 0.7019 (RAN4#116) | (Results for RAN#116 to be provided during the meeting week) |
| Encoder-4 (MLP) | Companies' name: Huawei, Hisilicon |  |  |  |  |
|  |  | 0.6782 (SGCS-3) | 0.6346 (SGCS-2) | 0.6712 (RAN4#116) | (To be provided during the meeting week) |

Observation 1: Comparing SGCS-1 (separate training for assumed CNN encoder with fixed CNN decoder) and SGCS-2 (separate training for low-complexity encoders with fixed CNN decoder), SGCS performance loss is observed.

Simulation results show that if the encoder with model structure A and complexity level-x is not considered during TE decoder derivation alignment, where TE decoder is derived with the assumption of model structure B (B is different from A) and complexity level-y (y is higher than x), SGCS performance loss can be observed if DUT employs model structure A with complexity level-x.

Observation 2: Comparing SGCS-3 (joint training for low-complexity encoders with assumed CNN decoder) and SGCS-2 (separate training for low-complexity encoders with fixed CNN decoder), SGCS performance improvement is observed.

Observation 3: Comparing SGCS-3 (joint training for low-complexity encoders with assumed CNN decoder) and SGCS-0 (joint training for assumed CNN encoder with assumed CNN decoder), marginal SGCS performance loss is observed.

However, this is not the fault of the encoder with model structure A and complexity level-x, since comparable SGCS performance compared to the encoder with model structure B and complexity level-y can be obtained, as long as the encoder with model structure A and complexity level-x is considered during the TE decoder derivation.

Proposal 2: Use low-complexity encoders to derive the common test decoder during WI.

# Discussion on model scalability

RAN4 has studied feasibility of aligning a model without considering model scalability. However, model scalability is an essential issue for model specifications. Due to that the combinations of configuration related to model input and model output are numerous, it is impractical to specify specific models for each configuration.

Considering that this is the last meeting for R19 SI, there is no time to perform simulations for scalable model alignment feasibility study. However, RAN4 can try to agree on the steps for how to align on a scalable model to be specified. The procedure can serve as a baseline or guidance during WI.

## 3.1 Baseline solution for achieving model scalability

It is observed that RAN1 has analyzed the model scalability, and the study outcome can be taken into account in RAN4 to avoid duplicate work.

In RAN1#120, several alternatives for realizing the scalability over various aspects, including number of Tx ports, bandwidth and subband size, CSI feedback payload size, are identified.

Observation 4: Scalability aspects include number of Tx ports, bandwidth and subband size, CSI feedback payload size.

Observation 5: For each scalability aspect, several alternative solutions have been identified in RAN1.

RAN1 #120 Agreement Agreement For model structure scalability study for temporal domain Case 0, - For the choice of token dimension and feature dimension, - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. - Alt 2: Use Tx port as the token dimension and subband as a feature dimension - The number of tokens varies with the number of Tx ports. - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. - The number of tokens varies with the number of Tx ports and the number of subbands. - For scalability over the feature dimension, - Alt1: specific embedding layer for each feature size - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) - For scalability over the token dimension, - Alt1: positional embedding specific to each token index - Ntoken,active tokens out of Ntoken token positions are used as input. - Alt2: Padding at the input - For scalability over payload configurations, - Alt1: specific output linear layer for each payload configuration - Alt2: truncation/masking of the output linear layer output - Alt3: by varying quantization parameters - Notes - Other Alternatives are not precluded. - Different Alternatives may be used in combination. - Same/similar approach is applied at the decoder side. - Evaluations to consider: - Case 1 (scalable structure): Scalable model structure described above - Using model structure as indicated in above diagram with fixed hyperparameters, companies may train a single parameter set or different parameter sets across different {number of Tx ports, CSI feedback payload size, bandwidth} assuming a common model structure. - To report whether a single parameter set or different parameter sets were used across different {number of Tx ports, CSI feedback payload size, bandwidth}. (e.g., single parameter set across different payload sizes and bandwidths, different parameter set across different number of Tx ports) - Case 2 (dedicated structure): Using model structure as indicated in above diagram with different hyperparameters, where the input and the output related hyperparameters are chosen optimally corresponding to each specific {number of Tx ports, CSI feedback payload size, bandwidth} without scalability considerations. - Different parameter sets are trained across different number of Tx ports, CSI feedback payload sizes, and bandwidths. - For each scalable model structure choice, to evaluate the SGCS performance of the non-AI/ML benchmark (e.g., Rel-16 eType2), Case 1, and Case 2, for each of {number of Tx ports, CSI feedback payload size, bandwidth}, and report the average gain (%) in SGCS of Case 1 and Case 2 over the non-AI/ML benchmark, as well as the loss (%) in the average gain of Case 1 w.r.t. Case 2. The average is performed by first calculating the SGCS gain (%) for each {number of Tx ports, CSI feedback payload size, bandwidth} and then averaging the SGCS gain (%) values over {number of Tx ports, CSI feedback payload size, bandwidth}. Agreement For model structure scalability study for temporal domain Case 0, for the choice of {number of Tx ports, CSI feedback payload size, bandwidth}, take the following as baseline values. - Number of Tx ports - [N1, N2, P] = [2, 8, 2] (32 CSI-RS ports) - [N1, N2, P] = [4,4,2] (32 CSI-RS ports) - [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports) - CSI feedback payload size - Multiple values, encouraged to pick at least one value from each of small, medium, and large payload regions X, Y, and Z. - Bandwidth & subband size - All subbands - A subset of all the subbands.

Different from CNN assumed in RAN4 for feasibility study during SI, Transformer is selected as a starting point in RAN1. If CNN is selected during WI, then there is no issue with the choice of token dimension and feature dimension. However, the other issues and alternatives also apply. If Transformer is selected during WI, RAN4 can directly reuse the choices identified by RAN1.

Regardless of which model structure is selected for specification, there are many candidate solutions by combining the alternatives identified for each scalability aspect. If RAN4 performs simulations for all combinations of all alternatives, the workload will be high.

Observation 6: The workload is extremely high if all combinations of alternatives for different scalability aspects are considered.

In RAN1 #120bis, simulation results for different alternatives over different scalability aspects are provided. The observations are summarized as follows in RAN1 Session notes. The popular alternatives with acceptable performance loss of the scalable model with respect to the dedicated model are highlighted.

RAN1 #120b Agreement Observation For choice of token/feature dimension in the scalable model design, companies observe that there is small loss in SGCS for Case1 over Case2 under tokenization (Alt1, Alt3) For layer1, under tokenization (Alt1), - 8 sources [Qualcomm, CATT, Xiaomi, vivo, ZTE, Fujitsu, Samsung, Huawei] observe that average gain in SGCS of Case 1 over Case 2 is in the range of -3.34% to 0.33% with median value of -1.10% - 2 sources [CATT, Xiaomi] observe average gain in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 2.04% to 7.84% with median value of 6.61% - 3 sources [CATT, Xiaomi, Ericsson] observe average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.17% to 10.2% with median value of 10.1% For layer1, under tokenization (Alt3), - 1 source [vivo] observes average gain (%) in SGCS of Case 1 over Case 2 is in the range of -0.7% to 0.4% with median value of -0.3% - 1 source [Ericsson] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.4% to 6% For scalability over the feature dimension, companies observe that there is small loss in SGCS for Case1 over Case2 assuming (Alt1, Alt2, Alt1+Alt2) For layer1, under (Alt1), - 4 sources [Qualcomm, CATT, Xiaomi, Fujitsu ] observes average gain (%) in SGCS of Case 1 over Case 2 is in the range of -3.34% to -0.4% with median value of -2.08% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 3% to 7.84% with median value of 6.71% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.17% to 10.2% with median value of 10.1% For layer1, under (Alt2), - 7 sources [Qualcomm, Xiaomi, vivo, ZTE, Fujitsu, Samsung, Huawei] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.2% to 0.1% with median value of -0.85% - 1 source [Xiaomi] observes gain in SGCS of Case 1 (scalable structure) over benchmark [G1] is 2.04% - 1 source [Xiaomi] observes gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For layer1, under (Alt1+Alt2), - 1 source [Qualcomm] observes average gain (%) in SGCS of Case 1 over Case 2 is in the range of -3.3% to -2.9% with median value of -3.1%. For scalability over the token dimension, companies observe that there is small loss in SGCS for Case1 over Case2 assuming (Alt1, Alt2) For layer1, under (Alt1), - 2 sources [Qualcomm, vivo] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.2% to 0.9% with median value of -1.5%. For layer1, under (Alt2), - 3 sources [CATT, vivo, ZTE ] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.34% to 0.4% with median value of -0.75%. - 1 source [CATT] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 6.51% to 7.84% with median value of 6.87% - 1 source [CATT] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 10.12% to 10.2% with median value of 10.1% For scalability over the payload configurations, companies observe that there is small loss in SGCS for Case1 over Case2 assuming (Alt1, Alt2) For layer1, under (Alt1), - 4 sources [Qualcomm, CATT, Xiaomi, Huawei] observe average gain (%) in SGCS of Case 1 over Case 2 is in the range of -0.73% to -3.34% with median value of -2.08% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 3% to 7.84% with median value of 6.71% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.17% to 10.2% with median value of 10.1% For layer1, under (Alt2), - 3 sources [Xiaomi, vivo, ZTE] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.2% to 0.4% with median value of -0.85% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is 2.04% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For scalability over feature dimension, token dimension and payload configuration jointly, For tokenization Alt1, feature scalability Alt1, token scalability Alt2 and payload scalability Alt1, - 1 source [CATT] observes average gain in SGCS of Case 1 over case 2 is in the range of -3.34%~2.03% with median value of -2.93% - 1 source [CATT] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 6.51%~7.84% - 1 source [CATT] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 10.12%~10.23% For tokenization Alt1, feature scalability Alt2, token scalability Alt2, payload configuration Alt2, - 2 sources [ZTE, vivo] observe average gain in SGCS of Case 1 over case 2 is in the range of -2.8%~1.2% with median value of -1.2%, under same parameter set. - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -0.1% ~ 0.8% with median value of 0.1%, under different parameter set For tokenization Alt1, feature scalability Alt2, token scalability Alt1, payload configuration Alt2, - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -1.7% ~ 5.4% with median value of -2.6%, under same parameter set - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -2.6% ~ 0% with median value of -0.9%, under different parameter set For tokenization Alt1, feature scalability Alt1, payload configuration Alt1, - 1 source [Xiaomi] observes average loss in SGCS of Case 1 over case 2 is in the range of -4.76%~-0.82% with median value of -2.08%. - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is 3% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For tokenization Alt1, feature scalability Alt2, payload configuration Alt2, - 1 source [Xiaomi] observes average loss in SGCS of Case 1 over case 2 is in the range of -5.52%~-1.59% with median value of -2.96%. - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is 2.04% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For tokenization Alt1, feature scalability Alt2, token scalability Alt2, - 1 source [ZTE] observes average gain in SGCS of Case 1 over case 2 is in the range of -0.4%~0.9% with median value of -0.23%. For tokenization Alt3, token scalability Alt1, payload configuration Alt2, - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -2.1% ~ 1.6% with median value of -0.5%, under same parameter set - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -0.8% ~ 0% with median value of -0.4%, under different parameter set

To reduce the workload, RAN4 can agree on a baseline solution for dealing with the scalability issue by selecting one alternative for each aspect. For example, RAN4 can select the alternative with the best performance according to RAN1 simulation results, avoiding duplicate work of performance comparison among alternatives. The solutions which are selected based on RAN1 study outcome are summarized in Table 2, which can be taken as a baseline solution for achieving model scalability.

Table 2. Baseline solution for achieving model scalability

|  | Scalability solution |
| --- | --- |
| Choice of token/feature dimension (if Transformer is agreed) | - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. |
| Scalability over the feature dimension | - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) |
| Scalability over the token dimension | - Alt2: Padding at the input |
| Scalability over payload configurations | - Alt2: truncation/masking of the output linear layer output |

In the following, we provide the simulation results for RAN4 current CNN model and mixed dataset, where the scalability for model input and model output is achieved by the solutions in Table 2.

In Table 3, the simulation results for model scalability over model input size are provided using the solutions in Table 2.

Table 3 Simulation results for scalability over model input size with baseline method in Table 2

| Training dataset | Testing dataset | SGCS |
| --- | --- | --- |
| Constructed by (13 x 32), (8 x 24) and (6 x 16) Note: (subband size x port number), padding size is (13x32) | Constructed by (13 x 32), (8 x 24) and (6 x 16) Note: (subband size x port number), padding size is (13x32) | 0.7474 |
|  | (13 x 32) | 0.6821 |
|  | (8 x 24) | 0.7557 |
|  | (6 x 16) | 0.9019 |
| (13 x 32) | (13 x 32) | 0.7099 |
| (8 x 24) | (8 x 24) | 0.7779 |
| (6 x 16) | (6 x 16) | 0.9211 |

In Table 4, the simulation results for model scalability over model output size are provided using the solutions in Table 2.

Table 4 Simulation results for scalability over model input size with baseline method in Table 2

| Training dataset | Testing dataset | SGCS |
| --- | --- | --- |
| Constructed by 64 x 2bit, 48 x 2bit, 32 x 2bit and 16 x 2bit | Constructed by 64 x 2bit, 48 x 2bit, 32 x 2bit and 16 x 2bit | 0.7079 |
|  | 64 x 2bit | 0.7519 |
|  | 48 x 2bit | 0.7352 |
|  | 32 x 2bit | 0.7015 |
|  | 16 x 2bit | 0.6431 |
| 64 x 2bit | 64 x 2bit | 0.7908 |
| 48 x 2bit | 48 x 2bit | 0.7532 |
| 32 x 2bit | 32 x 2bit | 0.7018 |
| 16 x 2bit | 16 x 2bit | 0.6589 |

Observation 7: SGCS performance loss is around 0.023 on average for scalability over model input size with the baseline method for achieving model scalability proposed in Table 2.

Observation 8: SGCS performance loss is around 0.018 on average for scalability over model output size with the baseline method for achieving model scalability proposed in Table 2.

Considering that the performance loss is negligible, RAN4 can agree on the baseline solution to achieve model scalability.

Proposal 3: For each scalability aspect, one alternative among all the identified alternatives is selected to form a baseline solution for deriving the scalable model.

Proposal 4: According to RAN1 simulation results on scalability study, the baseline solution is selected as the combination of the following alternatives in Table 2.

Table 2. Baseline solution for achieving model scalability

|  | Scalability solution |
| --- | --- |
| Choice of token/feature dimension (if Transformer is agreed) | - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. |
| Scalability over the feature dimension | - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) |
| Scalability over the token dimension | - Alt2: Padding at the input |
| Scalability over payload configurations | - Alt2: truncation/masking of the output linear layer output |

## 3.2 Steps for aligning scalable models

Different from aligning a scalable model without scalability, steps for aligning scalable model needs a few modifications.

Observation 9: Principles of designing the procedure to specify a scalable model during WI are summarized as follows.

- Mixed dataset generation: Align on different configurations for Tx port number and subband size. Companies will upload dataset with description of configurations.  Align on configurations for CSI feedback payload size.

- Model structures of both encoder and decoder: Select model structures based on companies’ contributions with an aligned scalability method. A reasonable complexity level range is defined for companies to propose their preferred models. Down-selection is performed if several models are very similar to each other.

- Training procedure: If multiple encoders are considered for deriving the common test decoder, training procedure needs to be aligned, such as N-to-1 joint training and N-to-1 separate training.

- Performance alignment metric: SGCS and throughput gain compared to non-AI under testing channel conditions are used. If models are aligned well but cannot achieve acceptable/testable performance gain, model structures need to be reselected.

- Performance requirement definition: If multiple encoders are considered for deriving the common test decoder, select one encoder to be specified based on the criteria of the trade-off between throughput performance gain and complexity level.

Proposal 5: Steps for aligning the specified model during WI are summarized as follows.

- Step 1: Align on configurations for mixed dataset construction, taking scalability over Tx port number and subband size into account.

- Step 2: Align on the model backbones and model structures based on companies’ contributions with an aligned scalability method, where model complexity is limited to a predefined range. One model structure is selected for decoder, while one or more model structures are selected for encoder.

  - The baseline scalability method is as follows.

| Choice of token/feature dimension (if Transformer is agreed) | Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. |
| --- | --- |
| Scalability over the feature dimension | A common embedding layer with padding (e.g., zero-padding or other techniques for padding values) |
| Scalability over the token dimension | Padding at the input |
| Scalability over payload configurations | Truncation/masking of the output linear layer output |

- Step 3: Align on training procedures (e.g., joint training and separate training, N-to-1 training if several encoders are used), hyperparameters and loss function definition.

- Step 4: Share encoders and decoders and cross-check.

- Step 5: Select one decoder with medium performance, that can achieve a testable performance gain under link-level testing channel conditions compared to non-AI.

  - If no decoders meet the requirement, go back to Step 2 to reselect the model structures.

- Step 6: Retrain encoders with the selected and fixed decoder. Share the encoder and cross-check.

  - If performance is not aligned, go back to Step 5 to reselect the fixed decoder.

- Step 7: Select one encoder from the retrained encoders which can achieve a testable performance gain under link-level testing channel conditions compared to non-AI.

- Step 8: Specify the decoder from Step 5 and the encoder from Step 7.

# Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: SGCS results for low-complexity encoders are provided in Table 1.

Table 1. Simulation results for low-complexity encoders

| Option 3 low-complexity encoders with different backbones |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | （1-to-1）Joint training | New encoder training with (Samsung’s) frozen decoder | （N-to-1）Joint training SGCS-4 Step-1 | New encoder training with frozen decoder from Step-1 SGCS-4 Step-2 |
| Encoder-1 | Companies' name: Huawei, Hisilicon | 0.71 (SGCS-0) | 0.704 (SGCS-1) | 0.6759 (simulation results from RAN4#115, where only Encoder-1,2,3 are considered, n1=n2 ) | 0.6957 (RAN4#115) (Results for RAN#116 to be provided during the meeting week) |
|  |  |  |  | 0.7128 |  |
|  |  |  |  | (simulation results updated for RAN4#116, with Encoder-1,2,3,4 considered, n1=0.9, n2=0.1) |  |
| Encoder-2 (TF) | Companies' name: Huawei, Hisilicon | 0.6793 (SGCS-3) | 0.6785 (SGCS-2) | 0.6669 (RAN4#115) | 0.6711 (RAN4#115) (Results for RAN#116 to be provided during the meeting week) |
|  |  |  |  | 0.6841 (RAN4#116) |  |
| Encoder-3 (CNN) | Companies' name: Huawei, Hisilicon | 0.6911 (SGCS-3) | 0.6852 (SGCS-2) | 0.6745 (RAN4#115) | 0.6842 (RAN4#115) |
|  |  |  |  | 0.7019 (RAN4#116) | (Results for RAN#116 to be provided during the meeting week) |
| Encoder-4 (MLP) | Companies' name: Huawei, Hisilicon |  |  |  |  |
|  |  | 0.6782 (SGCS-3) | 0.6346 (SGCS-2) | 0.6712 (RAN4#116) | (To be provided during the meeting week) |

Observation 1: Comparing SGCS-1 (separate training for assumed CNN encoder with fixed CNN decoder) and SGCS-2 (separate training for low-complexity encoders with fixed CNN decoder), SGCS performance loss is observed.

Observation 2: Comparing SGCS-3 (joint training for low-complexity encoders with assumed CNN decoder) and SGCS-2 (separate training for low-complexity encoders with fixed CNN decoder), SGCS performance improvement is observed.

Observation 3: Comparing SGCS-3 (joint training for low-complexity encoders with assumed CNN decoder) and SGCS-0 (joint training for assumed CNN encoder with assumed CNN decoder), marginal SGCS performance loss is observed.

Proposal 2: Use low-complexity encoders to derive the common test decoder during WI.

Observation 4: Scalability aspects include number of Tx ports, bandwidth and subband size, CSI feedback payload size.

Observation 5: For each scalability aspect, several alternative solutions have been identified in RAN1.

Observation 6: The workload is extremely high if all combinations of alternatives for different scalability aspects are considered.

Observation 7: SGCS performance loss is around 0.023 on average for scalability over model input size with the baseline method for achieving model scalability proposed in Table 2.

Observation 8: SGCS performance loss is around 0.018 on average for scalability over model output size with the baseline method for achieving model scalability proposed in Table 2.

Proposal 3: For each scalability aspect, one alternative among all the identified alternatives is selected to form a baseline solution for deriving the scalable model.

Proposal 4: According to RAN1 simulation results on scalability study, the baseline solution is selected as the combination of the following alternatives in Table 2.

Table 2. Baseline solution for achieving model scalability

|  | Scalability solution |
| --- | --- |
| Choice of token/feature dimension (if Transformer is agreed) | - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. |
| Scalability over the feature dimension | - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) |
| Scalability over the token dimension | - Alt2: Padding at the input |
| Scalability over payload configurations | - Alt2: truncation/masking of the output linear layer output |

Observation 9: Principles of designing the procedure to specify a scalable model during WI are summarized as follows.

- Mixed dataset generation: Align on different configurations for Tx port number and subband size. Companies will upload dataset with description of configurations.  Align on configurations for CSI feedback payload size.

- Model structures of both encoder and decoder: Select model structures based on companies’ contributions with an aligned scalability method. A reasonable complexity level range is defined for companies to propose their preferred models. Down-selection is performed if several models are very similar to each other.

- Training procedure: If multiple encoders are considered for deriving the common test decoder, training procedure needs to be aligned, such as N-to-1 joint training and N-to-1 separate training.

- Performance alignment metric: SGCS and throughput gain compared to non-AI under testing channel conditions are used. If models are aligned well but cannot achieve acceptable/testable performance gain, model structures need to be reselected.

- Performance requirement definition: If multiple encoders are considered for deriving the common test decoder, select one encoder to be specified based on the criteria of the trade-off between throughput performance gain and complexity level.

Proposal 5: Steps for aligning the specified model during WI are summarized as follows.

- Step 1: Align on configurations for mixed dataset construction, taking scalability over Tx port number and subband size into account.

- Step 2: Align on the model backbones and model structures based on companies’ contributions with an aligned scalability method, where model complexity is limited to a predefined range. One model structure is selected for decoder, while one or more model structures are selected for encoder.

  - The baseline scalability method is as follows.

| Choice of token/feature dimension (if Transformer is agreed) | Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. |
| --- | --- |
| Scalability over the feature dimension | A common embedding layer with padding (e.g., zero-padding or other techniques for padding values) |
| Scalability over the token dimension | Padding at the input |
| Scalability over payload configurations | Truncation/masking of the output linear layer output |

- Step 3: Align on training procedures (e.g., joint training and separate training, N-to-1 training if several encoders are used), hyperparameters and loss function definition.

- Step 4: Share encoders and decoders and cross-check.

- Step 5: Select one decoder with medium performance, that can achieve a testable performance gain under link-level testing channel conditions compared to non-AI.

  - If no decoders meet the requirement, go back to Step 2 to reselect the model structures.

- Step 6: Retrain encoders with the selected and fixed decoder. Share the encoder and cross-check.

  - If performance is not aligned, go back to Step 5 to reselect the fixed decoder.

- Step 7: Select one encoder from the retrained encoders which can achieve a testable performance gain under link-level testing channel conditions compared to non-AI.

- Step 8: Specify the decoder from Step 5 and the encoder from Step 7.

# References

- R4-2508051, “WF on AI ML for air interface study”, 3GPP RAN4 #115, May, 2025.

- R4-2508084, “Simulation assumptions on AI/ML CSI compression”, 3GPP RAN4 #115, 2025.

- RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, 3GPP RAN #108, 2025.