# 3GPP TSG-RAN WG4 Meeting #116	  R4-2510881

# Bengaluru, India, August 25th – 29th, 2025

Title: 	Overviews on 6Rx performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.1.4

Document for:	Discussion

Background

A WF [1] for 6Rx performance requirements was agreed in last meeting, this contribution provides our views.

- Discussions

- Release independent for 6Rx requirements

- One issue is release independent for 6Rx, the options are:

Agreement: - Option 1: - Support 6Rx with up to 4 MIMO layers for both handheld UE and FWA from Rel-18 - Support 6Rx with up to 6 MIMO layers for FWA from Rel-19 - Option 2: - Support 6Rx from Rel-19 - Option 3: - Support 6Rx from Rel-18

- It’s proposed not to introduce release independence for any 6Rx requirement and feature. 6Rx requirements target at handheld UE, which is commonly equipped with 4Rx at early release, there is no evidence showing that handheld UEs are widely equipped with 6Rx at early release. It is aggressive to introduce release independence for 6Rx requirements for handheld UE.

- Regarding 6 MIMO layers, there are inter-operation issues between UE and BS to support such feature, early release BS needs to support the new UE capability and change the scheduling strategy by taking into max DL 6 MIMO layers on top of the current scheduling.

Proposal 1: Not to introduce release independent for any 6Rx requirement/feature.

PDSCH requirements

- MIMO correlation for 4 MIMO layers for Handheld UE testing

There are two options for MIMO correlation configuration:

Issue 2-2-1: MIMO correlation for 4 MIMO layers for Handheld UE testing - Agreement - Introduce correlation matrix for 6Rx in the specification: RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β1625*β925*β425*β125*1 - Proposal - Option 1: with α = 0 and β = 0.07179 - Option 2: with α = 0.3 and β = 0.07179 - Option 3: with α = 0 and β = 0

To check the performance for different correlation matrix, a simulation was conducted with MCS 13 assumed.

Figure 2.1-1: Simulation results for different antenna correlation

We are not in favour of defining different requirements for handheld UE and FWA respectively, since supporting 4 layers is basic feature which should be applicable for all UE types and we didn’t differenate UE types when defining basic feature requirements before. It can be observed option 2 is infeasible due to the high SNR range and option 1 gives a reasonable SNR range.

Proposal 2: For 4 layers, RAN4 to define single set of requirements for both handheld UE and FWA, to use following one correlation:

Option 1: with α = 0 and β = 0.07179

Option 2: with α = 0 and β = 0

- MCS for 4 MIMO layer case for FWA/Handheld UE with TDLA30-10 ULA Low

There are two options for MCS for 4layer case:

Issue 2-2-5: MCS for 4 MIMO layer case for FWA/Handheld UE with TDLA30-10 ULA Low - Agreement: Down select the following two options in the next RAN4 meeting - Option 1: MCS 20 (Table 2) - Option 2: MCS 17 (Table 1)

A simulation was conducted to check performance for MCS20, simulation results are shown in figure 2-2:

Figure 2.1-2: Simulation results for MCS20

The test SNR is around 22dB, which is feasible. However, we prefer MCS17 over MCS20 since 64QAM is mandatory without capability signalling but 256QAM needs capability signalling, we prefer more common case for test coverage if there is only one case to be defined.

Proposal 3: RAN4 to use MCS17 (Table 1) for 4 Layer PDSCH test.

SDR requirements

Static channel and precoding

One issue is static channel and precoding for 8T6R, two options are listed below:

- For 8T6R (Fixed precoding with i1,1=i1,2=i2=0): - Option 1: - H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11 - Option 2: H=34*11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

Since the proposed channel is directional, i.e. channel energy will gather in a certain direction, option 1 will cause additional beamforming gain with precoding with i1,1=i1,2=i2=0, the intention of option 2 is to remove the beamforming gain. We don’t see any reason to preclude any one. Both are feasible, we prefer option 1 since it gives higher SNR test point and more opportunity to test high MCS.

Proposal 4: Support following static channel model for 8T6R:

- H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

1024QAM with 4layers

One issue is whether to introduce 1024QAM with 4 layers. To check the feasibility, a simulation was conducted, where the TxEVM is set to 2.5%.  The simulation results are shown in Figure 2.2-1 and Table 2.2-1.

Figure 2.2-1: Simulation for SDR test with 1024QAM/4 layers

Table 2.2-1: Simulation results for SDR test with 1024QAM/4 layers

| MCS | Target SNR(dB)@85% of maxTP |
| --- | --- |
| 23 | 27.4 |
| 24 | 29.4 |
| 25 | 31.4 |
| 26 | N/A |

It can be observed that 1024QAM with 4layers requires quite SNR range, which is even higher (close to 30dB) if considering additional margin. Note that AWGN channel model is assumed for SDR test and SNR may be more unachievable for fading channel. We don’t think such SNR range can occur in the field.

Proposal 5: Don’t introduce SDR requirements for 1024QAM with 4layers.

CSI requirements

CQI test

One of issues is whether to define CQI test, the candidate options are:

Issue 4-1-1: CQI test for 6Rx - Proposals - Option 1: Not define CQI test - Option 2: Only define one CQI requirements with rank 4 under static channel

The motivation of CQI test is not clear. The SINR difference between 4Rx and 6Rx at receiving side is 10log64=1.7dB, which is less than SINR threshold difference between adjacent CQI index. I.e. 2dB. Therefore, we don’t think calculated CQI index will change much between 4Rx and 6Rx.

Observation 1: The SNR increases by 1.7dB when the number of Rx changes from 4 to 6, which is less than SINR threshold difference between adjacent CQI index, i.e. 2dB, which means calculated CQI index will not change much between 4Rx and 6Rx.

Proposal 6: Don’t define CQI test.

RI test

One of issues is whether to define RI test, the candidate options are:

Issue 4-1-2: RI test for only Rank 6 - Proposals - Option 1: Not define RI tests for 6Rx - Option 2: Define RI test for 6Rx

The motivation of RI test is not clear, according to the outcome from core part, only FWA device can support 6 layers. However, 8Rx is also only supported by FWA device, but we didn’t define RI test for 8Rx. We don’t see the technical reason why RI test is not defined for high capability FWA device with 8Rx but will be defined for low capability FWA device with 6Rx. Furthermore, according to the study from core part, there is low probability that 6 layers can be scheduled. Therefore, it’s expected that gain is not high for Rank6.

Proposal 7: Don’t define RI test.

- Conclusion

This contribution provides our overviews on 6Rx performance requirements, the proposals and observations are:

Proposal 1: Not to introduce release independent for any 6Rx requirement/feature.

Proposal 2: For 4 layers, RAN4 to define single set of requirements for both handheld UE and FWA, to use following one correlation:

Option 1: with α = 0 and β = 0.07179

Option 2: with α = 0 and β = 0

Proposal 3: RAN4 to use MCS17 (Table 1) for 4 Layer PDSCH test.

Proposal 4: Support following static channel model for 8T6R:

- H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

Proposal 5: Don’t introduce SDR requirements for 1024QAM with 4layers.

Observation 1: The SNR increases by 1.7dB when the number of Rx changes from 4 to 6, which is less than SINR threshold difference between adjacent CQI index, i.e. 2dB, which means calculated CQI index will not change much between 4Rx and 6Rx.

Proposal 6: Don’t define CQI test.

Proposal 7: Don’t define RI test.

- Reference

[1] R4-2508727 Way Forward for Topic for [115][326] NR_ENDC_RF_Ph4_Demod_6Rx