3GPP TSG-RAN WG4 Meeting #116	R4-2510882

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on demodulation requirements for Ambient-IoT device 1

Source: 	Huawei, HiSilicon

Agenda item:	7.22.8

Document for:	Discussion

Background

According to the TU assignment, RAN4 should start the discussion on demodulation requirements for Ambient-IoT. One objective of WID [1] is “Specify the necessary demodulation performance requirements for device 1 and Ambient BS”

### 4.2 Objective of Performance part WI - Specify the necessary conformance testing for Ambient-IoT BS - Specify the necessary RRM performance requirements for device 1 - Specify the necessary demodulation performance requirements for device 1 and Ambient-IoT BS

This contribution provides our related views for device 1 demodulation requirements.

- Discussions

New physical channel PRDCH is specified for Ambient system which is used to carry high layer information from Reader to device. Corresponding demodulation performance requirements for PRDCH is needed to verify the demodulation performance of the device 1. Furthermore, TS 38.191 implements clause 10 for demodulation specifically, hence all requirements can be captured in TS 38.191.

Proposal 1: Define the demodulation requirements for PRDCH, the demodulation requirements can be captured in clause 10 of TS 38.191.

Test metric

According to RAN1 study, 1% and 10% BLER are considered as test metric. Both metrics are feasible, we slightly prefer 10%. We don’t think false alarm requirement is needed since there the CRC could guarantee the false alarm probability that is low enough.

Proposal 2: Define test metric of 10% BLER and not define false alarm requirements.

Test setup

One issue is test setup, based on RAN1 design, there is no ACK/NACK feedback introduced for device 1, so TE cannot identify whether the transmitted TB is decoded successfully or not, therefore RAN4 need to discuss how to measure BLER.  According to [3], test procedure for RF receiver requirements was agreed:

Issue 4-5-2: Test procedure for receiver RF requirements Agreement: - Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc. - Step 2: Place DUT based on UE positioning guidelines. - Step 3: Device is assumed charging during the measurement according to device declaration on the required energy conditions. - Step 4: Set the target test frequency and transmit power for signal generator and CW signal. The transmit power of signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device. FSS on the CW signal level. - Step 5: Determine whether DUT can send the correct response in D2R channel within timing window, e.g., TR2D_min - Step 6: Move to next measurement point and repeat Step 3-5 until complete all the testing point in the measurement grid. - Step 7: FFS on performance metric. FFS on how to specify the performance metric.

Step 5 means how TE determines whether device successfully decode PRDCH. If a device decodes a PRDCH successfully, then the device will start sending a response PDRCH within an amount of time TR→D after the end of PRDCH, the detailed explanation could be found in [4]:

A device shall upon receiving a PRDCH intended for the device in an R2D transmission ending in chip χendR2D=MchipR2D-1, perform a corresponding D2R transmission with chip χD2R=0 starting an amount of time TR→D after the end of chip χR2D= χendR2D according to the configuration received from higher layers.

TR→D depends on the R2D message type and whether channel coding for PDRCH is applied. For demodulation test, we think test procedure agreed for RF receiver requirements (Reference sensitivity) could be generally reused with the changes of transmit power of signal generator to make the received power at DUT’s antenna close to that is used for demodulation test. FFS for signal power setting.

Furthermore, tested device’s behaviour after receiving PRDCH depends on R2D message type. To specify the test setup, R2D message type should be defined. According to [4], following R2D message types are defined:

Table 6.1-1: R2D Message Type [4]

| R2D Message Type value | R2D message name |
| --- | --- |
| 000 | Reserved |
| 001 | A-IoT Paging message |
| 010 | Access Occasion Trigger message |
| 011 | Random ID Response message |
| 100 | R2D Upper Layer Data Transfer message |
| 101 | Reserved |
| 110 | Reserved |
| 111 | Reserved |

Taking A-IoT paging message as an example, the UE will use contention-free random access (CFRA) procedure with the device pagingID to respond (Msg1) in one of the usable resources (T_b and T_c pair) in correct time window TR->D or use contention based random access (CBRA) procedure and assign AS ID to the device in msg2 and then use that assigned AS ID to command the device to do something in defined resources. The CFRA or CBRA indication is defined in R2D message.

Taking R2D upper layer data transfer message as an example, then the device will response with upper layer data transfer message using scheduled resource indicated in R2D message.

Our preference is upper layer data transfer message since demodulation test is performed in connected mode and switching the device mode between idle mode and connected mode will complicate the test and increase test time.

Proposal 3: Reuse RF receiver test procedure (defined in [3]) for demodulation test as starting point with following changes or additions:

Re-consider the signal power to make the received power at reasonable range used for demodulation test. FFS for signal power.

Define the R2D message type as R2D Upper Layer Data Transfer message

TE to determine whether a PRDCH is decoded successfully by checking whether a response PDRCH is sent. The BLER could be derived with following equation:

- BLER=Number of responsed PDRCHsNumber of transmitted PRDCHs

Test parameter

Table 2-1 summarizes the test parameters which impact the demodulation performance and our related proposals.

Table 2-1: Proposed test parameter for PRDCH requirements

| Test parameter | Proposal |
| --- | --- |
| Channel model | TDLA30-10, Low |
| SCS | 15kHz |
| Antenna configuration | 2T1R |
| Number of bits | 20 bit |
| M (Number of chips per OFDM symbol) | 6 |
| Number of PRB | 2 |

Proposal 4: RAN4 to consider following parameters as starting point:

| Test parameter | Proposal |
| --- | --- |
| Channel model | TDLA30-10, Low |
| SCS | 15kHz |
| Antenna configuration | 2T1R |
| Number of bits | 20 bit |
| M (Number of chips per OFDM symbol) | 6 |
| Number of PRB | 2 |

- Conclusion

In this contribution we provide our views on demodulation for Ambient-Iot BS and device. The proposals are:

Proposal 1: Define the demodulation requirements for PRDCH, the demodulation requirements can be captured in clause 10 of TS 38.191.

Proposal 2: Define test metric of 10% BLER and not define false alarm requirements.

Proposal 3: RAN4 to reuse RF receiver test procedure (defined in [3]) for demodulation test as starting point with following changes or additions:

Re-consider the signal power to make the received power at reasonable range used for demodulation test. FFS for signal power.

Define the R2D message type as upper layer Data Transfer message

TE to determine whether a PRDCH is decoded successfully by checking whether a response PDRCH is sent.

- BLER=Number of responsed PDRCHsNumber of transmitted PRDCHs

Proposal 4: RAN4 to consider following parameters as start point:

| Test parameter | Proposal |
| --- | --- |
| Channel model | TDLA30-10, Low |
| SCS | 15 kHz |
| Antenna configuration | 2T1R |
| Number of bits | 20 bits |
| M (Number of chips per OFDM symbol) | 6 |
| Number of PRB | 2 |

- Reference

[1] R4-2508116 WF on A-IoT_device, RAN4#115, CMCC

[2] R4-2505230 WF on A-IoT device requirements, RAN4#114bis, CMCC

[3] TS 38.291