3GPP TSG-RAN WG4 Meeting #116	R4-2510883

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on demodulation requirements for Ambient-IoT BS

Source: 	Huawei, HiSilicon

Agenda item:	7.22.9

Document for:	Discussion

Background

According to the TU assignment, RAN4 should start the discussions on demodulation requirements for Ambient-Iot. One objective of WID[1] is “Specify the necessary demodulation performance requirements for device 1 and Ambient BS”

### 4.2 Objective of Performance part WI - Specify the necessary conformance testing for Ambient-IoT BS - Specify the necessary RRM performance requirements for device 1 - Specify the necessary demodulation performance requirements for device 1 and Ambient-IoT BS

This contribution provides our related views for BS demodulation requirements.

- Discussions

New physical channel PDRCH is specified for Ambient system which is used to carry high layer information from device to Reader. It is new physical channel with new Base station, it’s proposed to define demodulation requirements for PDRCH to verify the A-IoT BS demodulation performance. Furthermore, TS 38.191 implements clause 10 for demodulation specifically, hence all requirements can be captured in TS 38.191.

Proposal 1: Define the demodulation requirements for PDRCH, the demodulation requirements can be captured in clause 10 of TS 38.191.

Channel bandwidth

According to [2], D2R channel bandwidth definition was agreed:

Issue 2-2-1: D2R bandwidth Agreement in RAN4#115: - Define D2R CBW based on all Tb, Tc, R combinations as finally approved in RAN1.  Assumed 10% SFO for device  For Reader, [90%] filter spectrum utility ([10%] guard band) is considered. - The value of transmission BW and channel BW will be explicitly listed. - Using following equation for BS as starting point: D2R CBW for BS (KHz)=(2SB Transmission BW*(1/2)+2* Small frequency shift)/0.9 =(2000*(1+R)/Tb)* (1+∣SFO∣)/0.9 =(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)/0.9  Define [200kHz]，[3.54MHz] D2R CBW for AIoT BS. Other D2R CBWs for BS are FFS - Using following equation for device as starting point: D2R CBW for device (KHz)=2SB Transmission BW*(1/2)+2* Small frequency shift =(2000*(1+R)/Tb)* (1+∣SFO∣) =(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)

It can be observed that D2R CBW depends on (Tb, Tc, R) combination, our understanding is that final (Tb, Tc) combinations are still not determined yet and RAN4 RF part is discussing whether to make some down-selection from all defined (Tb, Tc) combinations to define channel bandwidth. Hence definition channel bandwidth for demodulation test shall be delayed until RF bandwidth definition is stable.

Proposal 2: RAN4 to postpone the channel bandwidth and (Tb, Tc, R) discussion until related core part is stable.

Channel coding

Channel coding (Tail-biting convolutional code) is configurable, which will improve the performance significantly, we prefer to configure the channel coding with 1/3 Convolutional Code for testing since we believe it is an important baseband processing step.

Proposal 3: RAN4 to consider channel coding with 1/3 Convolutional Code for PDRCH requirements

Modulation

Two modulation types are supported: BPSK and OOK, both modulation orders should be supported by Reader, but one modulation order can be selected for requirements definition, OOK is preferred.

Proposal 4: RAN4 to consider OOK for PDRCH requirements.

SFO

10% SFO is assumed for RAN1 D2R evaluation and is also defined for RF requirements. SFO is a critical issue from device side due to hardware limitation and has big impact on the performance. Reader utilizes preamble and midamble to perform SFO estimation using coherence detection to against performance degradation. Such processing is an important part of demodulation, so we propose to verify this behaviour. It’s proposed to set 10% SFO and assume coherence detection at reader side for simulation alignment. We also propose to set 1.92MHz device sampling rate to be aligned with RAN1 assumption.

Proposal 5: RAN4 to set 10% SFO and assume coherence detection at reader side for simulation alignment.

Proposal 6: RAN4 to consider 1.92MHz device sampling rate.

Amble length

Amble is used for SFO, channel and interference estimation, m sequence with length 7 and 31 are supported. We prefer length 31 since longer sequence could improve estimation accuracy.

Proposal 7: RAN4 to consider Amble length 31.

Others

Table 2-2 summarizes the test parameters which impact the demodulation performance and our proposal on how to set these parameters.

Table 2-2: Proposed test parameter for PDRCH requirements

| Test parameter | Proposal |
| --- | --- |
| Channel model | TDLA30-10, Low |
| SCS | 15 kHz |
| Antenna configuration | 1T2R |
| SFO | 10% |
| Device sampling rate | 1.92 MHz |
| Number of bits | FFS |
| Rblock | None |
| Ibit | FFS |
| Lamble | 31bits m-sequence |
| Iadd | FFS |
| Channel coding indication | 1/3 Convolutional Code |

Proposal 8: RAN4 to consider following parameters as starting point:

| Test parameter | Proposal |
| --- | --- |
| Channel model | TDLA30-10, Low |
| SCS | 15 kHz |
| Antenna configuration | 1T2R |
| SFO | 10% |
| Device sampling rate | 1.92 MHz |
| Number of bits | FFS |
| Rblock | None |
| Ibit | FFS |
| Lamble | 32bits m-sequence |
| Iadd | FFS |
| Channel coding indication | 1/3 Convolutional Code |

- Conclusion

In this contribution we provide our views on demodulation for Ambient-Iot BS and device. The proposals are:

Proposal 1: Define the demodulation requirements for PDRCH, the demodulation requirements can be captured in clause 10 of TS 38.191.

Proposal 2: RAN4 to postpone the channel bandwidth and (Tb, Tc, R) discussion until related core part is stable.

Proposal 3: RAN4 to consider channel coding with 1/3 Convolutional Code for PDRCH requirements

Proposal 4: RAN4 to consider OOK for PDRCH requirements.

Proposal 5: RAN4 to set 10% SFO and assume coherence detection at reader side for simulation alignment.

Proposal 6: RAN4 to consider 1.92MHz device sampling rate.

Proposal 7: RAN4 to consider Amble length 31.

Proposal 8: RAN4 to consider following parameters as starting point:

| Test parameter | Proposal |
| --- | --- |
| Channel model | TDLA30-10, Low |
| SCS | 15 kHz |
| Antenna configuration | 1T2R |
| SFO | 10% |
| Device sampling rate | 1.92 MHz |
| Number of bits | FFS |
| Rblock | None |
| Ibit | FFS |
| Lamble | 32bits m-sequence |
| Iadd | FFS |
| Channel coding indication | 1/3 Convolutional Code |

- Reference

[1] R4-2508101 WF on A-IoT BS and CW requirements, RAN4#115, Huawei

[2] R4-2505097 WF on A-IoT BS and CW requirements, RAN4#114bis, Huawei

[3] TS 38.291