3GPP TSG-RAN WG4 Meeting #116	R4-2510885

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussions on spatial channel model for demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.12.3

Document for:	Discussion

Background

In last meeting, a WF on spatial channel model for demodulation requirements was agreed. This contribution provides our related views.

- Discussions

Beam steering

One issue is that how to handle the problem that there is no beam steering enabled for CDL channel model.

#### Beam Steering Way Forward: For PMI testing, the properties of CDL can be reviewed, and comments and concern may be added to TR through pCRs. Technical discussion of beam-steering will not be brought to the next meeting, only wording for the TR. Technical description of the beam-steering will not be captured in the TR. Companies welcome to review wording for TR regarding properties of CDL for PMI testing at next meeting, or propose approaches if needed to be done in future Releases.

As discussed in [2], CDL channel has the characterize of fixed AOD/ZOD, causing the problem that the power spatial distribution is fixed. The selected PMI depending on the power spatial distribution will be fixed and pre-acquired if channel spatial characterize is fixed. The figure 2-1 shows the power spatial distribution and it has been agreed that the best PMI is (4,0,0) [1], it means UE using fixed precoder (4,0,0) constantly will past the test and may perform better than UE performing PMI selection. Such  phenomenon doesn’t comply with the real deployment.

Figure 2-1: PSD of CDL-C-A4 channel

Same concern is also expressed in 38.901:

#### 7.7.5.1 CDL extension: Scaling of angles The angle values of CDL models are fixed, which is not very suitable for MIMO simulations for several reasons; The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with closed-loop or reciprocity beamforming. Furthermore, a CDL only represents a single channel realization. The predefined angle values in the CDL models can be generalized by introducing angular translation and scaling. By translation, mean angle can be changed to and angular spread can be changed by scaling. The translated and scaled ray angles can be obtained according to the following equation:

Also, we have the following observation:

Observation 1: It has been agreed that PMI (4,0,0) is the optimal precoder for CDL-C-A4 channel and may be used as one parameter for PDSCH requirements definition in the further release. A single CDL channel model for PDSCH and PMI tests will be applied, which means the best PMI has been informed to the tested UE.

There was an agreement that no technical issues will be discussed in this meeting and RAN4 only discuss the TR wording on such concern. Our proposed wording is that:

The angular values of the agreed CDL mode is fixed, which is not suitable for PMI test for the reason that the PMI statistic is biased and a fixed precoder (for example, PMI (i1,1,i1,2,i2)=(4,0,0) for single-panel I codebook) performs better than channel adaptive precoders performed by PMI selection algorithm, leading that UE PMI selection may not be verified correctly. For example, UE constantly reporting PMI (i1,1,i1,2,i2)=(4,0,0), which has been revealed and can be pre-acquired without precoder selection may also past the PMI test. The related improvement should be investigated to ensure the CDL model to be very generic that can be suitable for different performance requirements testing to verify the proper UE / BS performance..

Proposal 1: Capture following wording on beam steering for CDL channel in TR:

- The angular values of the agreed CDL mode is fixed, which is not suitable for PMI test for the reason that the PMI statistic is biased and a fixed precoder (for example, PMI (i1,1,i1,2,i2)=(4,0,0) for single-panel I codebook) performs better than channel adaptive precoders performed by PMI selection algorithm, leading that UE PMI selection may not be verified correctly. For example, UE constantly reporting PMI (i1,1,i1,2,i2)=(4,0,0), which has been revealed and can be pre-acquired without precoder selection may also past the PMI test. The related improvement should be investigated to ensure the CDL model to be very generic that can be suitable for different performance requirements testing to verify the proper UE / BS performance..

Frequency Generalisation

Some companies express the concern that the agreed CDL channel is generalized according to certain frequency (3.5GHz) and cannot be applicable for all bands especially for FDD bands. There are two related options:

Options Option 1: To select one typical band for FDD and TDD respectively and utilize the centre frequency of such two typical FDD band TDD band as target frequency to derive desired angular spread and doppler. The two scaled CDL models can represent TDD and FDD scenario respectively for requirements definition. Option 2: Use 3.5GHz for all FR1 bands and duplexing, following legacy and SCM alignment.

The intention of introduction CDL channel for demodulation test is to make the channel more aligned with the channel that is measured in the real deployment. The agreed CDL channel cannot represent the practical channel for different frequency bands, especially considering the large frequency range for NR and 6GR with more new frequency resource introduction, such as the angular spread for FDD is quite lower than TDD. It is unreasonable to select one single or two typical band/frequency for SCM channel definition for general demodulation performance requirements definition. If we consider to select more frequencies for SCM channel, it means more studies and alignment are needed. What’s more, with new frequency introduction in the future, investigation on suitable SCM channel model will become a heavy workload for the corresponding WI, the completion date for the related WI will not be guaranteed. Also which typical frequency should be selected will also become a controversial part for the WI. Only limited frequencies selection for the SCM channel will obviate the principle of band agnostic of demodulation performance requirements, this will bring trouble for UE verification per band in the GCF for different operators.

Observation 2: Either Option 1 and Option 2 are against the motivation that introducing CDL channel to make the channel model for demodulation test more practical and lack scalability to other frequencies in the future.

Considering only one meeting left and there is no enough time to exploit the issues in the CDL channel under discussion. It’s proposed to capture the concern in the TR:

Proposal 2: Capture following wording on frequency generalisation channel in TR:

- The agreed CDL channel is generalized according to certain frequency (3.5GHz), which is not be aligned with the real deployment and not suitable for demodulation test for other frequencies, especially for FDD with low frequency.

Concern on alignment approach

Currently, the alignment approach is based on aligning SNR point@ 70% of max throughput, where only one set of parameters (e,g, One MCS, one doppler) is agreed for simulation alignment. Such limited number of cases make it dangerous to conclude the alignment. For example, 10Hz doppler is too low to check if higher doppler is correctly modelled in the channel generalisation.

The more reasonable and efficient way is to align the channel properties. I.e. Power spectrum distribution, time coherent and frequency coherent. Not aligned channel property causes the risk that SNR aligned in some certain cases may not be aligned in other cases.

Observation 3: Alignment of SNR point with quite limited number of simulation cases (One MCS and one doppler) is insufficient to conclude the  the alignment and feasibility of channel implementation for demodulation performance requirements for all bands defined in 3GPP for NR and 6GR in the future.

Hence our proposal is to capture channel properties provided by companies such as power spectrum distribution, time coherent and frequency coherent in the alignment part of TR.

Proposal 3: The alignment part of TR should cover channel properties provided by companies such as power spectrum distribution, time coherent and frequency coherent, besides SNR points collection.

- Channel model property

Spatial power spectral distribution

| CDL-C-A4-Tx |  | CDL-C-A4-Rx |  |
| --- | --- | --- | --- |
| Cluster model 1 | Cluster model 2 |  | Cluster model 3 |

Figure 3-1: Power spectral distribution results

Time coherence

Figure 3-2: Time coherence results

Frequency coherence

TBD

- Conclusion

In this contribution we provide our views on SCM. The proposals and observations are:

Observation 1: It has been agreed that PMI (4,0,0) is the optimal precoder for CDL-C-A4 channel and may be used as one parameter for PDSCH requirements definition in the further release. There is single CDL channel model for PDSCH and PMI tests, which means the best PMI has been informed to the tested UE.

Proposal 1: Capture following wording on beam steering for CDL channel in TR:

- One contribution expresses the concern that the angular values of the agreed CDL mode is fixed, which is not suitable for PMI test for the reason that the PMI statistic is biased and a fixed precoder (for example, PMI (i1,1, i1,2, i2)=(4,0,0) for single-panel I codebook) performs better than channel adaptive precoders performed by PMI selection algorithm, leading that UE PMI selection may not be verified correctly. For example, UE constantly reporting PMI (i1,1, i1,2, i2)=(4,0,0), which has been revealed and can be pre-acquired without precoder selection may also past the PMI test. It is expected that the improvement could be made in the future release.

Observation 2: Option 2 against the intention that introducing CDL channel to make the channel model for demodulation test more practical.

Proposal 2: Capture following wording on frequency generalisation channel in TR:

- Some contributions express the concern that the agreed CDL channel is generalized according to certain frequency (3.5GHz), which may not be aligned with the FDD real deployment and may not be reasonably used for FDD demodulation test.

Observation 3: Aligning SNR point with quite limited number of simulation cases (One MCS and one doppler) is not enough to confirm the alignment of channel implementation.

Proposal 3: The alignment part of TR should cover not only SNR points collection but also channel properties if provided by companies such as power spectrum distribution, time coherent and frequency coherent.

- Reference

[1] R4-2508776 Way Forward for [115][321] NR_SCM.

[2] R4-2507044 Discussion on  SCM.