3GPP TSG RAN WG4 Meeting #116	R4-2510895

Bengaluru, India, 25 Aug. – 29 Aug. 2025

Agenda Item:	7.3.4

Source: 	Ericsson

Title:	RRM requirements for low band carrier aggregation via switching

Document for:	Discussion

# Introduction

New WID [1] on Low band carrier aggregation via switching is agreed in RAN#106. In this contribution, we analyze the potential impact to RAN4 RRM requirements based on issues identified in last meeting WF, R4-2508259.

# Discussion

## General method to specify the requirements

In the previous meeting, RAN4 began examining the impact of RRM requirements for SDL SCell and FDD PCell. We believe the existing RRM requirements can be used as baseline. We propose that the RRM impact can be managed using two methods.

Method 1: Discussing a method to extend the delay requirement if SSB/SMTC/MG/CSI-RS/DRX ON periodicity are missed at UE due to switching to another cell. Once we agree on this delay extension principle, it can be applied to all related procedures on PCell and SCell. This approach was similarly utilized in Release 16 for defining requirements on a carrier subject to CCA.

Method 2: for some of the requirements, without extending or relaxing the particular requirement, we can specify an applicability condition under which the requirements are valid for low-band CA via switching.

From our analysis in previous meetings, we found not many requirements need to be specified with Method 2 and we suggest following method 1 for the requirements definition.

- RAN4 to agree on Method 1 for specifying the requirements for LB CA via switching. Where method 1 is:

  - Method1: Discussing a method to extend the delay requirement if SSB/SMTC/MG/CSI-RS/DRX ON periodicity are missed at UE due to switching to another cell. Once we agree on this delay extension principle, it can be applied to all related procedures on PCell and SCell. This approach was similarly utilized in Release 16 for defining requirements on a carrier subject to CCA.

The rest of the discussion is organized as follows:

- RRM requirements impact for SDL SCell

- RRM requirements impact for FDD PCell

- LB CA switching delay and potential interruptions

## RRM requirements impact for SDL SCell

### When the SCell is deactivated

In the last meeting, the following agreement was reached.

  - RAN4 to define the requirement for deactivated SDL SCell L3 measurement.

  - The legacy deactivated SCell measurement delay requirements are applied for UE to perform deactivated SDL SCell measurement.

  - For deactivated SDL SCell measurement, interruptions on active serving cell(s) are allowed with up to X% probability of missed ACK/NACK.

    - FFS: X, and X can be same or different for different SDL SCC measurement cycle (i.e., configured measCycleSCell).

To perform measurements on the SDL SCell, the UE must switch to the SDL SCell, regardless of the measCycleSCell configuration. In addition to the SSB or SMTC duration required for the measurement, the UE needs RF tuning time to switch from the FDD PCell to the SDL SCell. After completing the measurement, the UE must switch back to the FDD PCell, which also requires RF retuning time.

The total number slots for missed receptions in the FDD PCell is: RF tuning time + measurement window + RF retuning time.

This many ACK/NACKs may be missed if all slots contain PDSCH scheduling and the UE is unable to receive PDSCH during the measurement period. Additionally, ACK/NACKs corresponding to PDSCH in previous slots may be scheduled during the RF tuning time + measurement window + RF retuning time.

Although the exact number of missed ACK/NACKs depends on the k1 value (the delay between PDSCH and ACK/NACK), for minimum requirement analysis, assuming all slots carry ACK/NACKs, the UE may miss that many ACK/NACKs.

The total interruption, in terms of missed ACK/NACKs, is: 2 × (RF tuning time + measurement window + RF retuning time). Assuming, UE performs the measurement once evey measCycleSCell, the percentage of missed ACK/NACK can be defined as X = Y/ measCycleSCell, where Y is 2 × (RF tuning time + measurement window + RF retuning time).

RF tuning and retuning times depend on UE capabilities and may be either 35 µs or 140 µs, as per the RF session agreement. In FR1, SSB symbols start from the 3rd symbol of the slot and end at the 11th symbol.

This provides a 2-symbol gap (143 µs) before the start of the PSS, which can be used by the UE for RF tuning and retuning. For UEs supporting 140 µs RF switching delay, even with an MRTD of 3 µs, no additional time beyond the measurement window is needed. In such cases, RF tuning and retuning can be included within the measurement window.

However, as discussed offline with other companies, for neighbor cell measurements, the UE may need to perform cell search to detect the symbol boundary of the neighbor cells. If the UE starts symbol detection at the 3rd symbol, it may not accurately detect the symbol boundary if it supports a 140 µs RF switch delay.

However, for UEs with a 35 µs RF switch delay, there is an additional margin of 105 µs (143 µs − 35 µs − 3 µs), which corresponds to a cell separation of up to 31.5 km. Therefore, for at least UEs supporting 35 µs RF switching time, the interruption requirement should consider only the measurement window.

- For the deactivated SCell measurement, the interruption rate is defined as X = Y/ measCycleSCell. Where Y is:

  - 2 × (RF tuning time + measurement window + RF retuning time) for UE supporting RF switch delay of 140µs.

  - 2 × measurement window for UE supporting RF switch delay of 35µs.

### When the SCell is being activated

In the last meeting, the following agreement was reached.

  - The R18 SSB based FR1 inter-band SCell activation delay requirement can be reused to SDL SCell activation in LB CA via switching if UE supports corresponding feature.

    - R16 Direct SCell activation with HO is not considered for R19 LB CA.

    - R17 PUCCH SCell activation is not considered for R19 LB CA.

    - R18 SSB-less SCell activation for NES is not considered for R19 LB CA.

    - R17 Fast SCell activation is not considered for R19 LB CA.

  - Reuse the existing SCell deactivation delay requirement for SDL SCell deactivation.

  - Known/unknown conditions are reused from existing SCell activation requirements

RAN4 has agreed that the switching pattern does not apply during SCell activation. However, to activate the SCell, the UE must switch to the SDL SCell to receive the required reference signals (RS) prior to activation. Because of this, RAN4 should define either an interruption requirement or a scheduling restriction on the FDD PCell during RS reception on the SDL SCell.

For each RS occasion (such as SSB, TRS, or CSI-RS), the UE must switch from the FDD PCell to the SDL SCell, perform the necessary measurements, and then switch back to the FDD PCell. When defining scheduling restrictions, the network must consider the UE’s switching behavior in both directions to avoid scheduling transmissions/receptions during these switch periods.

As shown in Figure 1, when the switching pattern is applicable, RAN1 has agreed that:

- The switch always occurs at the end of a slot,

Figure 1: Switching from FDD to SDL and SDL to FDD

The switch delay is applied on the carrier the UE is switching from.

To stay consistent with this, we suggest RAN4 apply the same principle during SCell activation, allowing reuse of the RRC-configured switching gap.

- RAN4 to agree that, during the SCell activation, switching gap occurs on the switch from carrier and at the end of the slot.

An alternative approach to proposal 3 is to apply the switch delay on the carrier the UE is switching to, rather than the one it's switching from.

- Switching from FDD to SDL is relatively straightforward, involving mainly RF switch delay. The UE can switch to the SDL SCell after the FDD DL slot ends.

- Switching back from SDL to FDD is more complex, especially when scheduling restrictions are applied. This is because FDD UL typically starts before FDD DL, leading to different symbol-level restrictions for DL and UL.

This approach poses two issues and hence it is not advisable to deifen this way:

- Increased scheduler complexity, making it harder to implement and manage scheduling restrictions.

- Impact on PDCCH on the FDD carrier during the switch from SDL to FDD switch.

Based on the above proposal 3 and the issue with alternative approach to P3, the Scheduling Restrictions can be defined as below:

- When switching from FDD to SDL,

  - a scheduling restriction of N1 symbols (same as RRC configured in the switching pattern) is applied on the FDD carrier.

- When switching back from SDL to FDD,

  - If the RS is in the last N2 symbols (same as RRC configured in the switching pattern) of the slot, the switch happens in the next slot, during last N2 symbols of the next slot.

  - If the RS is not in the last N2 symbols, the switch happens in the same slot as the RS reception.

- RAN4 to define scheduling restriction as following.

- If last RS symbol during the measurement on the SDL carrier occurs within the last N2 symbols of the slot:

    - The UE is not expected to receive/transmit on the FDD PCell for N1 symbols before the RS slot + the remaining RS slot(s), if any, + ceil(N2/14) slot(s)

- If last RS symbol during the measurement on the SDL carrier does not occur within the last N2 symbols of the slot:

    - The UE is not expected to receive/transmit on the FDD PCell for N1 symbols before the first RS slot + the remaining RS slot(s), if any;

- where first RS slot is the slot containing the first symbol of the RS on which the UE measures and the remaining RS slot(s) is all slots after the first RS slot up until the slot containing the last symbol of the RS on which the UE measures.

### When the SCell is activated

When the SCell is activated, RAN4 agreed to deifne the requirements for activated SDL SCell L3 measurements. Further following agreement is reached.

For activated SDL SCell measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern. FFS: - If no SSB occasion of the serving cell is contained within the LB-CA switching pattern, the requirements do not apply (from Nokia proposal) - FFS on details of requirement design.

#### L3 measurements

For the requirement design of L3 measurements, we suggest following the method 1 specified in section 2.1 of this paper.

While there are other type of measurement requirements that are impacted, we consider intra-frequency without gap as an example to discuss the principle for RRM requirement relaxation or extending the delay. The measurement period for L3 measurements is as mentioned below.

Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1)

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | max (200 ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max (200 ms, ceil(1.5x 5 x Kp) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(5 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

According to the measurement period outlined in the above table, the measurement period relies on the parameters SMTC, Kp, and CSSFintra. Due to the switch pattern and the half-duplex nature of the LB CA via switching, these parameters may be affected. We look at the defintion of these parameters for analyzing the RRM requirements impact.

Kp: This factor determines the number of SMTC occasions not overlapping with any MG relative to the total available SMTC occasions, including those overlapped by gaps. Existing defintion of the Kp is shown below.

Kp is the scaling factor for an SSB frequency layer to be measured without GAP. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows: - For a window W of duration max(SMTC period, xRP_max), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion: - Ntotal is the total number of SMTC occasions within the window, including those overlapped with GAP and MUSIM gap occasions within the window, and - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.4, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively. Kp = 1 when Navailable = 0. - xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap. Requirements in this clause do not apply when Navailable = 0 due to fully overlapping between SMTC occasions and MUSIM gap occasions within the window W.

Because of the switch pattern design, introducing a new scaling factor, Kp_LB, is necessary. Kp_LB will account for SMTC occasions on the relevant cell that neither overlap with gaps nor are missed due to switch patterns. This scaling factor can be defined to capture the SMTC occasions available for L3 measurement and same is highlighted in green, as illustrated in Figure 2.

Figure 2: L3 measurement ocassions for LB CA

- RAN4 to define a new Kp_LB to address the impact of switch patterns on the measurement period.

Kp_LB:

To deifne Kp_LB, we need to define the W, Ntotal and Navailble.

Window duration W is deined as max(SMTC period,  xRP_max, Switch pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion. Switch pattern periodcity is the periodcity of the NW configured switch pattern for LB CA via switching.

Ntotal is defined as the total number of SMTC occasions within the window, including those that overlap with GAP occasions, MUSIM gap occasions, and including those that fall within either the Switching Pattern ON or OFF durations.

Navailable is defined as the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions or not overlapped with switching pattern OFF duration within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.4, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively.

Kp_LB is the scaling factor for an SSB frequency layer to be measured without GAP and in the presence of switch pattern. Kp_LB = Ntotal / Navailable,

- Kp_LB is the scaling factor for an SSB frequency layer to be measured without GAP and in the presence of switch pattern. Kp_LB = Ntotal / Navailable

- Window duration W is deined as max(SMTC period,  xRP_max, Switch pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion. Switch pattern periodcity is the periodcity of the NW configured switch pattern for LB CA via switching.

- Ntotal is defined as the total number of SMTC occasions within the window, including those that overlap with GAP occasions, MUSIM gap occasions, and including those that fall within either the Switching Pattern ON or OFF durations.

- Navailable is defined as the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions or not overlapped with switching pattern OFF duration within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.4, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively.

SMTC Period: Certain SMTC occasions may be unavailable for L3 measurement due to switching between cells, and it can result in a prolonged measurement delay. We think the non availability of the SMTC occasion for the L3 measurement can be captured in the Kp_LB design. Due to this, we think, SMTC period defintion need not change.

CSSF: When all the Measurement Gap (MG) and SMTC occasions overlap, the inter-frequency and intra-frequency carriers must share the SMTC occasion. Again, this may dependent on the Kp_LB defintion and we suggest revisiting this after the agreement on Kp_LB.

- RAN4 to further discuss CSSF inside MG for the LB CA.

#### L1 measurement requirements:

We look at the legacy L1-RSRP requirements for the impact analysis for LB CA L1 measurement requirements and later we check if we can apply same principles for other L1 measurements as well.

Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1 Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(M*P)*TDRX. Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting.. - M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise For FR1, - P=1/(1-T_SSB/MGRP), when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and - P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB. TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting.

From the table above, we observe that there is no need to change the value of M, as the number of samples remains consistent with legacy requirements for L1-RSRP measurements. However, switching between FDD and SDL may result in some SSBs being unavailable for L1-RSRP measurement. In legacy, when SSBs overlap with MG, a sharing factor P is used to address the non-availability of SSBs due to overlap of SSB with MG, thus specifying the remaining SSB occasions for L1-RSRP. In LB CA with switching, SSBs might be unavailable for L1-RSRP not only due to switching between FDD and SDL but also because of overlap with MG (similar to legacy). Both factors must be considered when defining SSB occasions for L1-RSRP measurement or a sharing factor for L1-RSRP. A new sharing factor, PLB, can be introduced to account for the non-availability of SSBs due to the switch pattern and overlap with MG. PLB can be defined to capture the SSB occasions available for L1-RSRP measurement and same occasions are highlighted in green, as shown in figure 3. Since PLB addresses SSB availability, we propose reusing the existing TSSB definition, TDRX and TReport.

Figure 3: L1-RSRP measurement occasions for LB CA

We observe that RAN4 followed similar approach for defining the BFD, CBD and L1-SINR requirements. We copy the relevant requirements below for reference.

BFD requirements: Table 8.5.2.2-1: Evaluation period TEvaluate_BFD_SSB for FR1 Configuration; TEvaluate_BFD_SSB (ms). no DRX; Max(50, Ceil(5 ´ P) ´ TSSB). DRX cycle ≤ 320ms; Max(50, Ceil(7.5 ´ P) ´ Max(TDRX,TSSB)). DRX cycle > 320ms; Ceil(5 ´ P) ´ TDRX. Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length.. For FR1, - P=11-TSSBMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB. - P = 1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB. CBD requirements: Table 8.5.5.2-1: Evaluation period TEvaluate_CBD_SSB for FR1 Configuration; TEvaluate_CBD_SSB (ms). non-DRX, DRX cycle ≤ 320ms; Max(25, Ceil(3 ´ P) ´ TSSB). DRX cycle > 320ms; Ceil(3 ´ P) ´ TDRX. Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length.. For FR1, - P=11-TSSBMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB, - P = 1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB. L1-SINR requirements: Table 9.8.4.2-1: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR1 Configuration; TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms). non-DRX; max(TReport, ceil(M*P)*TSSB). DRX cycle ≤ 320 ms; max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)). DRX cycle > 320 ms; ceil(M*P)*TDRX. NOTE 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR channel measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity..

- For L1 measurements, RAN4 to define new sharing factor, PLB to account for the non-availability of SSBs due to the switch pattern and SSB overlap with MG.

#### TCI state switch delay

We summarize different TCI state switch delay supported in the existing spec. We did not consider unified TCI state switch for the analysis, but we expect similar conclusions should be applicable for unified TCI state switch as well.

|  | Known TCI state | Unknown TCI state | Comment |
| --- | --- | --- | --- |
| MAC CE based | n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length | n+ THARQ +3Nslotsubframe,µ + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc) / NR slot length | - Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state |
| DCI based | timeDurationForQCL |  |  |
| RRC based | n+ (TRRC_processing +TOk*(Tfirst-SSB + TSSB-proc)) / NR slot length | n+ (TRRC_processing +TL1-RSRP +TOuk*(Tfirst-SSB + TSSB-proc)) / NR slot length | Tfirst-SSB is time to first SSB transmission after L1-RSRP measurement when TCI state switching involves QCL-TypeD; - Tfirst-SSB is time to first SSB transmission after RRC processing time at the UE for other QCL types; - The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state |
| Active TCI state list update | n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. | NA | As defined in MAC CE based switch |

Among the delay components outlined in the table above regarding legacy requirements, we think THARQ is not impacted due to LB CA via switching as it is configured by the NW using the k1 value and the NW configuration already considers the switch pattern. Among the other components, we believe the parameter that is affected by the switch pattern is Tfirst-SSB. This is because Tfirst-SSB involves the UE accessing SSB signals for measurement, and the time taken to access these signals depends on the switch pattern. We consider other components to be related to UE processing delays, which we believe remain unaffected. Therefore, we propose redefining Tfirst-SSB for LB CA via switching, suggesting it should be defined as the first SSB available on the respective cell.

Due to UE operation on the SDL, and the potential HARQ delay on the FDD carrier, UE may receive the SSB on the SDL carrier before sending the HARQ on the FDD carrier. In this case, we think the delay should be max (HARQ, SSB reception time). RAN4 can further work on the delay equation.

- RAN4 to change the defintion of Tfirst-SSB for the TCI state switch delay and it is to be defined as the first SSB available on the respective cell after the switch command.

- RAN4 to consider changes to TCI state switch requirement equation to consider the SSB being received on SDL before the HARQ being sent on FDD.

## RRM requirements impact for FDD PCell

We think the RRM requirements analysis of L3 measurement, L1 measurements should hold true for PCell as well.

- The principle of scaling factor agreed for SDL SCell, to be applicable for PCell as well. It is noted that the scaling factor may be different for SDL SCell and FDD PCell.

Other than the requirements discussed for the SDL SCell, we think couple of procedures such as RLM and CBD on the PCell needs special attention.

RLM-RS can be SSB and/or CSI-RS. In legacy, RLM was supported only on SpCell. Since the WI says Strive to minimize the RAN1 impact, we do not foresee RAN1 discussing/introducing the RLM on SCell.

Further UE is expected to perform RLM measurement on the RLM-RS at least once in 2 radio frame and send OOS indication to the higher layer if the OOS is detected.

Based on the switch pattern, the availability of RLM-RS on the PCell may be impacted. If the RLM-RS is impacted due to the switch pattern, RLM evaluation period requirements need further discussion. Since the SDL SCell do not have UL support, HARQ feedback is carried on the PCell. If the radio link of PCell fails, even if the SCell DL is good, UE communication with SCell is as good as lost. Hence, we suggest RAN4 to investigate the RLM and faster link recovery on the PCell. For example, if any of the following condition is met, UE could prioritize performing RLM and link recovery on the slots where the RLM-RS or SSB is transmitted by ignoring the pattern on those slots.

- N1 number of out of sync are triggered on PCell; or

- T310 timer is started.

Similarly, if the BFD is detected on the PCell, UE could prioritize performing CBD on the slots where the CBD-RS is transmitted by ignoring the pattern on those slots (i.e., even if the slot containing CBD-RS belonging to SCell, UE prioritize the CBD-RS reception). In the case of RLM, NW do not know when the RLM occurs at the UE and in this case we could only define the interruption requirements.

- RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritise to receive RS for for faster link recovery. Principle of scheduling restriction or interruption requirement agreed for the SCell activation to be reused for the link receiver procedure.

Following the same principles defining the scheduling restriction during SCell activation, we can define interruption requirements as following. That means that, UE switiches back to the FDD PCell is UE is on SDL SCell, receive RLM-RS and switch back to SDL SCell.

If last RLM-RS symbol during the measurement on the FDD carrier occurs within the last N2 symbols of the slot:

- The UE is not expected to receive on the SDL SCell for N1 symbols before the RS slot + the remaining RS slot(s), if any, + ceil(N2/14) slot(s)

If last RLM-RS symbol during the measurement on the FDD carrier does not occur within the last N2 symbols of the slot:

- The UE is not expected to receive on the SDL SCell for N1 symbols before the first RS slot + the remaining RS slot(s), if any

## LB CA switching delay and potential interruptions

RAN4 reached following agreement during the last meeting.

- For SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier.

  - Note: the above agreement can be revisited if not aligned with RAN1 further agreement.

Based on the RAN1 agreements, we do not see a need to update the above agreement.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- RAN4 to agree on Method 1 for specifying the requirements for LB CA via switching. Where method 1 is:

  - Method1: Discussing a method to extend the delay requirement if SSB/SMTC/MG/CSI-RS/DRX ON periodicity are missed at UE due to switching to another cell. Once we agree on this delay extension principle, it can be applied to all related procedures on PCell and SCell. This approach was similarly utilized in Release 16 for defining requirements on a carrier subject to CCA.

- For the deactivated SCell measurement, the interruption rate is defined as X = Y/ measCycleSCell. Where Y is:

- 2 × (RF tuning time + measurement window + RF retuning time) for UE supporting RF switch delay of 140µs.

- 2 × measurement window for UE supporting RF switch delay of 35µs.

- RAN4 to agree that, during the SCell activation, switching gap occurs on the switch from carrier and at the end of the slot.

- RAN4 to define scheduling restriction as following.

- If last RS symbol during the measurement on the SDL carrier occurs within the last N2 symbols of the slot:

    - The UE is not expected to receive/transmit on the FDD PCell for N1 symbols before the RS slot + the remaining RS slot(s), if any, + ceil(N2/14) slot(s)

- If last RS symbol during the measurement on the SDL carrier does not occur within the last N2 symbols of the slot:

    - The UE is not expected to receive/transmit on the FDD PCell for N1 symbols before the first RS slot + the remaining RS slot(s), if any;

- where first RS slot is the slot containing the first symbol of the RS on which the UE measures and the remaining RS slot(s) is all slots after the first RS slot up until the slot containing the last symbol of the RS on which the UE measures.

- RAN4 to define a new Kp_LB to address the impact of switch patterns on the measurement period.

- Kp_LB is the scaling factor for an SSB frequency layer to be measured without GAP and in the presence of switch pattern. Kp_LB = Ntotal / Navailable

- Window duration W is deined as max(SMTC period,  xRP_max, Switch pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion. Switch pattern periodcity is the periodcity of the NW configured switch pattern for LB CA via switching.

- Ntotal is defined as the total number of SMTC occasions within the window, including those that overlap with GAP occasions, MUSIM gap occasions, and including those that fall within either the Switching Pattern ON or OFF durations.

- Navailable is defined as the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions or not overlapped with switching pattern OFF duration within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.4, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively.

- RAN4 to further discuss CSSF inside MG for the LB CA.

- For L1 measurements, RAN4 to define new sharing factor, PLB to account for the non-availability of SSBs due to the switch pattern and SSB overlap with MG.

- RAN4 to change the defintion of Tfirst-SSB for the TCI state switch delay and it is to be defined as the first SSB available on the respective cell after the switch command.

- RAN4 to consider changes to TCI state switch requirement equation to consider the SSB being received on SDL before the HARQ being sent on FDD.

- The principle of scaling factor agreed for SDL SCell, to be applicable for PCell as well. It is noted that the scaling factor may be different for SDL SCell and FDD PCell.

- RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritise to receive RS for for faster link recovery. Principle of scheduling restriction or interruption requirement agreed for the SCell activation to be reused for the link receiver procedure.

# References

[1]		RP-243317, New WID on low band CA via switching

[2]		RP-241537

[3]		R4-2508259, WF on RRM requirements for NR_LBCA_Sw, Moderator (Apple)