3GPP TSG-RAN4 Meeting #116	R4-2510897

Bengaluru, India,  Aug. 25 – Aug. 29, 2025

Source:	Ericsson

Title:	Performance requirements for low-band carrier aggregation via switching

Agenda item:	7.3.5

Document for:	Discussion

# 1	Introduction

RAN4#116 is the first meeting of the performance part of the WI. In this contribution we discuss and propose performance requirements for low-band carrier aggregation via switching and related test cases.

# 2	Discussion

The core requirements for low-band CA are still under discussion, and the initial draft CRs have not yet been submitted. However, based on the work split agreed upon in the last meeting, we can begin analysing which requirements need to be tested for UEs supporting low-band CA via switching. For reference, the agreed work split is included below.

| Index | Requirement | note | Company |
| --- | --- | --- | --- |
| 1 | Activated SDL SCell L3 measurement | Activated and deactivated SCell measurement delay requirement can be in the same section. All details are depending on the discussion. | OPPO |
|  | Deactivated SDL SCell measurement |  |  |
| 2 | [Interruption for deactivated SDL SCell measurement] |  | Xiaomi |
| 3 | SDL SCell activation/deactivation and corresponding interruption | All scope and details are depending on the discussion. | Nokia |
| 4 | [BFD/CBD] for SDL SCell and PCell (if needed) | (1) (If needed) BFD/CBD requirement for SDL SCell. (2) (If needed) The impact of switching pattern to PCell BFD/CBD requirement | Huawei |
| 5 | TCI state switch delay for SDL SCell and PCell (if needed) | (1) TCI state switch delay requirement for SDL SCell. (2) (If needed) The impact of switching pattern to PCell TCI state switch delay requirement | LGE |
| 6 | PL-RS switch delay for PCell (if needed) | (If needed) The impact of switching pattern to PCell PL-RS switching requirement | ZTE |
| 7 | L1-RSRP/SINR for SDL SCell and PCell (if needed) | (1)L1-RSRP/SINR measurement requirement for SDL SCell. (2) (If needed) The impact of switching pattern to PCell L1-RSRP/SINR measurement requirement | Samsung |
| 8 | PCC L3 measurement delay requirement (if needed) | (1) PCC L3 requirement impact can also be captured if needed. (2) PCell RLM requirement impact can also be captured if needed. | Ericsson |
|  | RLM requirement for PCell (if needed) |  |  |

Based on the above table we think RAN4 at least need to define test cases for following.

- Activated SDL SCell L3 measurement

- Deactivated SDL SCell measurement

- SDL SCell activation/deactivation delay and corresponding interruption

The requirements which can be FFS for the test cases are:

- Interruption for deactivated SDL SCell measurement

- [BFD/CBD] for SDL SCell and PCell

- TCI state switch delay for SDL SCell and PCell

- PL-RS switch delay for PCell

- L1-RSRP/SINR for SDL SCell and PCell

- PCC L3 measurement delay requirement

- RLM requirement for PCell

- RAN4 to agree on defining following list of test cases for UE supporting low-band CA via switching

- Activated SDL SCell L3 measurement

- Deactivated SDL SCell measurement

- SDL SCell activation/deactivation delay and corresponding interruption

- FFS:

  - Interruption for deactivated SDL SCell measurement

  - [BFD/CBD] for SDL SCell and PCell

  - TCI state switch delay for SDL SCell and PCell

  - PL-RS switch delay for PCell

  - L1-RSRP/SINR for SDL SCell and PCell

  - PCC L3 measurement delay requirement

  - RLM requirement for PCell

Test configurations:

Based on the WID description, we can derive the following test configuration

-	The cell 1 is on n5 and cell 2 is on n29

-	MRTD = 3µs

-	SCS 15KHz on both carriers

## 2.x	Timing

In existing RRM specification TS 38.133 baseline we have the following tests:

… A.6 NR standalone tests with all NR cells in FR1 … A.6.4 Timing A.6.4.1 UE transmit timing A.6.4.1.1 NR UE Transmit Timing Test for FR1 A.6.4.1.2 NR UE Transmit Timing Test for two TRPs in FR1 … A.6.4.2 UE timer accuracy … A.6.4.3 Timing advance … A.6.4.3.1 SA FR1 timing advance adjustment accuracy …

The WI for low band carrier aggregation via switching has the following deployment considerations [1]:

… - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers …

Based on this one can view low band carrier aggregation via switching as an instantiation of NR standalone tests with all NR cells in FR1, when it comes to RRM timing performance requirements.

Observation 1:	 Low band carrier aggregation via switching as an instantiation of NR standalone tests with all NR cells in FR1.

Proposal 2: For timing test cases, existing NR standalone tests with all NR cells in FR1 are sufficient to cover also Low band carrier aggregation via switching.

# 3 	Summary

In this contribution we analyzed the performance requirements of low-band CA via switching and made the following observations and proposals.

- RAN4 to agree on defining following list of test cases for UE supporting low-band CA via switching

- Activated SDL SCell L3 measurement

- Deactivated SDL SCell measurement

- SDL SCell activation/deactivation delay and corresponding interruption

- FFS:

  - Interruption for deactivated SDL SCell measurement

  - [BFD/CBD] for SDL SCell and PCell

  - TCI state switch delay for SDL SCell and PCell

  - PL-RS switch delay for PCell

  - L1-RSRP/SINR for SDL SCell and PCell

  - PCC L3 measurement delay requirement

  - RLM requirement for PCell

Observation 1:	Low band carrier aggregation via switching as an instantiation of NR standalone tests with all NR cells in FR1.

Proposal 2: For timing test cases, existing NR standalone tests with all NR cells in FR1 are sufficient to cover also Low band carrier aggregation via switching.

# References

[1] 		RP-251315, Revised WID on Low NR band carrier aggregation via switching, TELUS