# 3GPP TSG-RAN WG4 Meeting #116	R4-2510916

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	7.23.4

Source: 	Ericsson

Title:	Discussion on RRM performance part for Rel-19 NES

Document for:	Discussion

# Introduction

In last meeting, most open issues of core requirements on Rel-19 NES were closed. In this contribution, we’ll discuss the test case design and the related test case list[1].

# OD-SSB test case

Test Scope

When OD-SSB feature introduced in Rel-19 NES WI, two different cases are defined for OD-SSB transmission: Case 1 – without AO-SSB; Case 2 – with AO-SSB. Furthermore, three different scenarios are agreed as follow: Scenario 2 - OD-SSB indication before SCell activation, Scenario 2a - OD-SSB indication together with SCell activation and Scenario 3B – OD-SSB configuration update after SCell activation. Therefore, the test cases shall include all three valid scenarios with both Case 1 and Case 2. RAN4 can define the test cases for OD-SSB based SCell activation with deactivated Scell measurement to test both Scenario 2 and Scenario 2a and without deactivated SCell measurement to test Scenario 2 only. At the same time, RAN4 should introduce the measurement test cases to verify the scenario 3B in CONNECTED mode.

Proposal 1: RAN4 to define the OD-SSB based test case for both Case 1 and Case 2(with and without AO-SSB).

Proposal 2: RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B.

Proposal 3: RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell.

Currently, RAN4 defined different UE behaviours for deactivated SCell measurement, SCell activation and L1-RSRP measurement. In deactivated SCell measurement, RAN4 introduced FMW for fast OD-SSB measurement. The test case shall verify UE’s behaviour for FMW. RAN4 spent lots of time to discuss UE’s behvaiour after first OD-SSB activation. However, we don’t think such UE’s behaviour for OD-SSB re-activation before SCell activation related T2 shall be verified which will result in the test case too complex.

Proposal 4: RAN4 to define the test case for deactivated SCell measurement to verify the FMW and corresponding interruption requirement.

Proposal 5: RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

In Case 2, RAN4 agreed to define the requirement for Alt Time-C1 in which the OD-SSB transmission is overlapping with AO-SSB and the union of OD-SSB transmission and AO-SSB meets a legacy SSB periodicity. An effective measurement periodicity is applied to both SCell activation and L1-RSRP after SCell activation. Thus, RAN4 shall define the test case to verify the EMP related requirement in Case 2.

Proposal 6: RAN4 to define the test case to verify the EMP related requirement in Case 2 Alt Time-C1.

In Case 2, there is other two scenarios which are Alt Time-C2 and scenario 3. RAN4 is discussing whether to define the requirements and the detail UE behaviour and requirements are uncertain. To considering the test burden, we think it’s fine not to define the test cases for these two scenarios.

Proposal 7: In Case 2, RAN4 not to define the test case for the following scenarios.

- Alt Time-C2: the center frequency locations of always-on SSB and on-demand SSB are same, and the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern

- Scenario 3: the center frequency locations of always-on SSB and on-demand SSB are different

Furthermore, RAN4 agrees to follow the legacy SSB measurement accuracy requirements for OD-SSB. Thus, we think it’s fine not to define new test case for Measurement Performance requirements of OD-SSB.

Proposal 8: RAN4 not to define the new test case for Measurement Performance requirements of OD-SSB.

General Principle

To simplify the test case setting, we prefer the following configurations:

- Both FR1 and FR2

- Only test OD-SSB periodicities with 5ms, 10ms and 20ms

- Only test non-DRX mode

- without SSB time index detection

- Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

Proposal 9: RAN4 to define the OD-SSB test cases with the following general princinples:

- Both FR1 and FR2

- Only test OD-SSB periodicities with 5ms, 10ms and 20ms

- Only test non-DRX mode

- without SSB time index detection

- Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

Test Configuration

RAN4 agrees the deactivated SCell’s measurement lower bound is 10ms and the lower bound is not applied to SCell activation. Therefore, we propose to define the SCell activation test case with 5ms periodicity. When UE needs the lower bound, the test requirement shall base on 10ms for deactivated SCell measurement. In Case 2, we propose to define two types of deactivated SCell measurement. One is for OD-SSB overlapping with AO-SSB. The OD-SSB periodicity can be set as 5ms. Another test case is for the union of OD-SSB and AO-SSB meets the legacy SSB periodicity. Both OD-SSB and AO-SSB periodicity can be set as 20ms. At the same time, in case 2, two remaining scenarios are discussing in RAN4: RAN1 Alt Time-C2 and scenario 3. We propose not to define any test case for these two scenarios.

Proposal 10: In Case 1, RAN4 to define the OD-SSB based SCell activation test case with 5ms periodicity.

Proposal 11: In Case 2, RAN4 to define the OD-SSB based SCell activation test case as follow.

- OD-SSB(5ms) is overlapping with AO-SSB(20ms)

- The union of OD-SSB(20ms) and AO-SSB(20ms) meets the legacy SSB periodicity(10ms)

Considering OD-SSB is NCD-SSB, RAN4 needs to introduce new OD-SSB(NCD-SSB) configuration for 5ms, 10ms and 20ms periodicity.

Proposal 12: RAN4 to introduce new NCD-SSB configuration for 5ms, 10ms and 20ms periodicity.

Test Case List

To tradeoff the burden of tests and the test coverage, we prefer to consider the following test cases.

Proposal 13: RAN4 to define OD-SSB test cases as follow.

| No. | Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- | --- |
| 1a | SCell Activation and deactivation of known SCell in non-DRX | SCell activation Deactivated SCell measurement Interruption during deactivated SCell measurement Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2 and 2a Known SCell activation |
| 1b | SCell Activation and deactivation of unknown SCell in non-DRX(OD-SSB periodicity is 5ms) | SCell activation | OD-SSB periodicity 5ms | OD-SSB Case 1 OD-SSB scenario 2a Unknown SCell activation |
| 2a | SCell Activation and deactivation of known SCell in non-DRX | SCell activation Deactivated SCell measurement Interruption during deactivated SCell measurement Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms AO-SSB periodicity 20ms | OD-SSB Case 2 Alt Time-C1 - OD-SSB(5ms) is overlapping with AO-SSB(20ms) OD-SSB scenario 2 and 2a |
| 2b | SCell Activation and deactivation of known SCell in non-DRX for 640 ms SCell measurement cycle | SCell activation Deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 Alt Time-C1 - The union of OD-SSB and AO-SSB meets the legacy SSB periodicity OD-SSB scenario 2 and 2a |
| 3 | Multiple SCells Activation and deactivation of unknown SCells in non-DRX | Multiple SCells activation | OD-SSB periodicity 5ms AO-SSB periodicity 20ms | OD-SSB Case 2 OD-SSB scenario 2a |
| 4a | Direct SCell activation at SCell addition of known SCell | Direct SCell activation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2a |
| 4b | Direct SCell activation at SCell addition of known SCell | Direct SCell activation | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 Alt Time-C1 OD-SSB scenario 2a |
| 5b | PUCCH SCell Activation and deactivation of known SCell | PUCCH SCell Activation | OD-SSB periodicity 10ms | OD-SSB Case 2 OD-SSB scenario 2 and 2a Known SCell activation |
| 6a | Intra-frequency measurement(deactivated SCell) | Deactivated SCell measurement | OD-SSB periodicity 10ms | OD-SSB Case 1 FMW |
| 6b | Intra-frequency measurement(deactivated SCell) | Deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 FMW |
| 7a | Intra-frequency measurement- SA event triggered reporting tests without gap for deactivated SCell under non DRX | L3-RSRP measurement | OD-SSB periodicity 10ms | OD-SSB scenario 3B |
| 7b | Intra-frequency measurement- SA event triggered reporting tests without gap under non DRX | L3-RSRP measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB scenario 3B |
| 8 | SSB based L1-RSRP measurement when DRX is not used | L1-RSRP measurement | OD-SSB periodicity 20ms -> 10ms | OD-SSB scenario 3B |

# SSB Adaptation test case

Scenario

During RAN4 meetings, RAN4 agreed to support SSB adaptation in the following scenarios.

- L1 measurement

  - L1-RSRP, L1-SINR, CBD

- L3 measurement

- SCell activation

  - Normal SCell activation, Direct SCell activation, PUCCH SCell activation

From our understanding, RAN4 at least to define the test cases for L1-RSRP, L3-RSRP, and normal SCell activation. In last RAN2 meeting, RAN2 agrees only to use DCI format 2_9 to indicate SSB adaptation. After SSB adaptation, SMTC is also updated.

Proposal 14:  For SSB adaptation, RAN4 to define new test cases to verify SSB adaptation with MAC-CE based Scell activation and PUCCH SCell activation.

Proposal 15:  For SSB adaptation, RAN4 to define new test cases to verify SSB(SMTC) adaptation for L3 measurement.

Proposal 16:  For SSB adaptation, RAN4 to define new test cases to verify SSB adaptation for L1-RSRP, L1-SINR and CBD.

Test Principle

To simplify the test case setting, we prefer the following configurations:

- Both FR1 and FR2

- Only test non-DRX mode

- without SSB time index detection

To guarantee test converge, both SSB adaptation from a sparse SSB periodicity to a dense SSB periodicity and vice vase should be verified. However, RAN4 shall not define any test cases to verify the transition period similar as legacy test cases.

Proposal 17: RAN4 to define the SSB adaptation test cases with the following general princinples:

- Both FR1 and FR2

- Only test non-DRX mode

- without SSB time index detection

Proposal 18: RAN4 not to define the SSB adaptation test case for both L1/L3 transition period.

Test Configuration

To verify SSB adaptation, RAN4 needs to introduce new NCD-SSB configuration with 20ms and 80ms. The legacy SMTC configuration with 20ms and 80ms can be reused.

Proposal 19:  RAN4 to verify both SSB adaptation from a sparse SSB periodicity to a dense SSB periodicity and vice vase.

- SMTC periodicity from 80ms to 20ms

- SSB periodicity from 80ms to 20ms

Proposal 20: For SSB adaptation test, RAN4 to introduce new NCD-SSB configuration with 20ms and 80ms.

Proposal 21: For SSB adaptation test, RAN4 to use legacy SMTC configuration with 20ms and 80ms.

Test Case List

To tradeoff the burden of tests and the test coverage, we prefer to consider the following test cases.

Proposal 22: RAN4 to define SSB adaptation test cae as follow.

| No. | Test category | Test purpose |
| --- | --- | --- |
| 1 | SSB based L1-RSRP measurement when DRX is not used for SSB adaptation | L1-RSRP measurement |
| 2 | L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used for SSB adaptation | L1-SINR measurement |
| 3 | Beam Failure Detection and Link Recovery Test for FR2 SCell configured with SSB-based BFD and LR in non-DRX mode | Candidate beam detection |
| 4 | SA event triggered reporting tests without gap for SSB adaptation | L3-RSRP measurement |
| 5 | SCell Activation and deactivation for SCell in non-DRX | SCell activation |

# OD-SIB1 test case

In last meeting, RAN4 discussed whether to define additional paging interruption requirement for OD-SIB1 acquisition with cell reselection in IDLE mode. Although the discussion is on-going, we think it’s reasonable to define a test case to verify this new OD-SIB1 acquisition procedure in IDLE mode with the following steps.

  - Step 1: The UE evaluates cell reselection based on measurements towards SSB of target NES cell.

  - Step 2: After cell reselection delay, the UE initiates OD-SIB1 procedure.

    - Step 2-1: UE transmits the UL-WUS(PRACH)

    - Step 2-2: UE receives the RAR

    - Step 2-3: UE monitors OD-SIB1 window to decode the SIB1 and OSIs

Proposal 23:  RAN4 to define a new test case to verify the OD-SIB1 procedure in IDLE mode.

- Summary

In this contribution, we have discussed the the performance part of Rel-19 NES. Based on the discussions, we made the following proposals and observations:

Proposal 1: RAN4 to define the OD-SSB based test case for both Case 1 and Case 2(with and without AO-SSB).

Proposal 2: RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B.

Proposal 3: RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell.

Proposal 4: RAN4 to define the test case for deactivated SCell measurement to verify the FMW and corresponding interruption requirement.

Proposal 5: RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

Proposal 6: RAN4 to define the test case to verify the EMP related requirement in Case 2 Alt Time-C1.

Proposal 7: In Case 2, RAN4 not to define the test case for the following scenarios.

- Alt Time-C2: the center frequency locations of always-on SSB and on-demand SSB are same, and the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern

- Scenario 3: the center frequency locations of always-on SSB and on-demand SSB are different

Proposal 8: RAN4 not to define the new test case for Measurement Performance requirements of OD-SSB.

Proposal 9: RAN4 to define the OD-SSB test cases with the following general princinples:

- Both FR1 and FR2

- Only test OD-SSB periodicities with 5ms, 10ms and 20ms

- Only test non-DRX mode

- without SSB time index detection

- Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

Proposal 10: In Case 1, RAN4 to define the OD-SSB based SCell activation test case with 5ms periodicity.

Proposal 11: In Case 2, RAN4 to define the OD-SSB based SCell activation test case as follow.

- OD-SSB(5ms) is overlapping with AO-SSB(20ms)

- The union of OD-SSB(20ms) and AO-SSB(20ms) meets the legacy SSB periodicity(10ms)

Proposal 12: RAN4 to introduce new NCD-SSB configuration for 5ms, 10ms and 20ms periodicity.

Proposal 13: RAN4 to define OD-SSB test cases as follow.

| No. | Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- | --- |
| 1a | SCell Activation and deactivation of known SCell in non-DRX | SCell activation Deactivated SCell measurement Interruption during deactivated SCell measurement Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2 and 2a Known SCell activation |
| 1b | SCell Activation and deactivation of unknown SCell in non-DRX(OD-SSB periodicity is 5ms) | SCell activation | OD-SSB periodicity 5ms | OD-SSB Case 1 OD-SSB scenario 2a Unknown SCell activation |
| 2a | SCell Activation and deactivation of known SCell in non-DRX | SCell activation Deactivated SCell measurement Interruption during deactivated SCell measurement Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms AO-SSB periodicity 20ms | OD-SSB Case 2 Alt Time-C1 - OD-SSB(5ms) is overlapping with AO-SSB(20ms) OD-SSB scenario 2 and 2a |
| 2b | SCell Activation and deactivation of known SCell in non-DRX for 640 ms SCell measurement cycle | SCell activation Deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 Alt Time-C1 - The union of OD-SSB and AO-SSB meets the legacy SSB periodicity OD-SSB scenario 2 and 2a |
| 3 | Multiple SCells Activation and deactivation of unknown SCells in non-DRX | Multiple SCells activation | OD-SSB periodicity 5ms AO-SSB periodicity 20ms | OD-SSB Case 2 OD-SSB scenario 2a |
| 4a | Direct SCell activation at SCell addition of known SCell | Direct SCell activation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2a |
| 4b | Direct SCell activation at SCell addition of known SCell | Direct SCell activation | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 Alt Time-C1 OD-SSB scenario 2a |
| 5b | PUCCH SCell Activation and deactivation of known SCell | PUCCH SCell Activation | OD-SSB periodicity 10ms | OD-SSB Case 2 OD-SSB scenario 2 and 2a Known SCell activation |
| 6a | Intra-frequency measurement(deactivated SCell) | Deactivated SCell measurement | OD-SSB periodicity 10ms | OD-SSB Case 1 FMW |
| 6b | Intra-frequency measurement(deactivated SCell) | Deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 FMW |
| 7a | Intra-frequency measurement- SA event triggered reporting tests without gap for deactivated SCell under non DRX | L3-RSRP measurement | OD-SSB periodicity 10ms | OD-SSB scenario 3B |
| 7b | Intra-frequency measurement- SA event triggered reporting tests without gap under non DRX | L3-RSRP measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB scenario 3B |
| 8 | SSB based L1-RSRP measurement when DRX is not used | L1-RSRP measurement | OD-SSB periodicity 20ms -> 10ms | OD-SSB scenario 3B |

Proposal 14:  For SSB adaptation, RAN4 to define new test cases to verify SSB adaptation with MAC-CE based Scell activation and PUCCH SCell activation.

Proposal 15:  For SSB adaptation, RAN4 to define new test cases to verify SSB(SMTC) adaptation for L3 measurement.

Proposal 16:  For SSB adaptation, RAN4 to define new test cases to verify SSB adaptation for L1-RSRP, L1-SINR and CBD.

Proposal 17: RAN4 to define the SSB adaptation test cases with the following general princinples:

- Both FR1 and FR2

- Only test non-DRX mode

- without SSB time index detection

Proposal 18: RAN4 not to define the SSB adaptation test case for both L1/L3 transition period.

Proposal 19:  RAN4 to verify both SSB adaptation from a sparse SSB periodicity to a dense SSB periodicity and vice vase.

- SMTC periodicity from 80ms to 20ms

- SSB periodicity from 80ms to 20ms

Proposal 20: For SSB adaptation test, RAN4 to introduce new NCD-SSB configuration with 20ms and 80ms.

Proposal 21: For SSB adaptation test, RAN4 to use legacy SMTC configuration with 20ms and 80ms.

Proposal 22: RAN4 to define SSB adaptation test cae as follow.

| No. | Test category | Test purpose |
| --- | --- | --- |
| 1 | SSB based L1-RSRP measurement when DRX is not used for SSB adaptation | L1-RSRP measurement |
| 2 | L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used for SSB adaptation | L1-SINR measurement |
| 3 | Beam Failure Detection and Link Recovery Test for FR2 SCell configured with SSB-based BFD and LR in non-DRX mode | Candidate beam detection |
| 4 | SA event triggered reporting tests without gap for SSB adaptation | L3-RSRP measurement |
| 5 | SCell Activation and deactivation for SCell in non-DRX | SCell activation |

Proposal 23:  RAN4 to define a new test case to verify the OD-SIB1 procedure in IDLE mode.

- References

- R4-2504907, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei