3GPP TSG-RAN4 Meeting # 116	R4-2510934

Bangalore, IN. 25th – 29th, 2025

# Title: 	On terminology and release independent for 6Rx RF requirement

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.1.2.3.1

# Document for:	Approval

# 1 Introduction

In last meeting, some agreements were achieved for 6Rx in the WF [1], where for FWA terminology and release independent, it was:

## <Sub-topic 1-1> FWA terminology Agreement: - Remove “form factor” from the notes in the corresponding tables for 4Rx, 6Rx and 8Rx, and use “FWA CPE” or “FWA” for ΔRIB,6R requirement in the note of the table. - Consider adding the definition of FWA CPE or FWA, and industrial UE in the specifications - Focus on modifying the notes of the receiver requirements for 4Rx, 6Rx and 8Rx for Rel-19 specifications in RAN4#115 - FFS on whether and how to align the existing notes between 4Rx, 6Rx and 8Rx since the notes were specified according to the separate WIDs. - Further modifications will be done in the future meetings - Address the similar issues for previous releases under the maintenance agenda - Address the similar issue for transmission requirements under the maintenance agenda. ### <Sub-topic 2-1> Release independency for 6Rx UE Agreement: - Option 1: - Support 6Rx with up to 4 MIMO layers for both handheld UE and FWA from Rel-18 - Support 6Rx with up to 6 MIMO layers for FWA from Rel-19 - Option 2: - Support 6Rx from Rel-19 - Option 3: - Support 6Rx from Rel-18

In this contribution, we continue discussing the FWA terminology and release independent issue.

# 2	Discussion

## 2.1	FWA terminology

This issue was detail described in [2], wherein some confusions may be existed for the readers. It was already agreed to remove “form factor” from the notes in the corresponding tables for 4Rx, 6Rx and 8Rx, and then the tables are looked like:

-- 4Rx table

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n5, n8, n13, n26, n28, n71, n85, n105 | -2.71 |
| n5, n8, n13, n28, n71, n20, n26 | -2.42 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79, n104 | -2.2 |
| NOTE 1: When 4 Rx operation is supported by [FWA CPE or FWA] form factor NOTE 2: When 4Rx operation is supported by handheld UE. |  |

-- 8Rx table

Table 7.3.2-2a: Eight antenna port reference sensitivity allowance ΔRIB,8R

| Operating band | ΔRIB,8R (dB) |
| --- | --- |
| n1, n3, n7, n25 | -4.5 |
| n34, n39, n41 | -4.3 |
| n77, n78, n79 | -4.0 |
| NOTE 1: 8 Rx operation is targeted for [FWA CPE or FWA] FWA/CPE/Vehicle/Industrial devices form factor. |  |

-- 6Rx table

Table 7.3.2-2aa: Six antenna port reference sensitivity allowance ΔRIB,6R

| Operating band | ΔRIB,6R (dB) |
| --- | --- |
| n41 | -3.41 -3.72 |
| n77, n78, n79, n104 | -3.01 -3.32 |
| NOTE 1: When 6 Rx operation is supported by handheld UE. NOTE 2: When 6 Rx operation is supported by [FWA CPE or FWA]]. |  |

After referring to some materials in the following:

FWA (Fixed Wireless Access) is a wireless broadband technology that delivers high-speed internet to homes and businesses using radio signals (like 4G LTE, 5G, or proprietary wireless tech) instead of cables (like fiber).

FWA CPE is the physical device installed at the user’s location to receive wireless signals and provide internet connectivity.

The FWA form factor refers to the physical design and configuration of the FWA device. It defines how the CPE is structured (indoor/outdoor, integrated antenna, modular design, etc.).

It can be seen that FWA is a wireless access technology, not a user device. FWA CPE, as it known as its name, of course it is a user device which use FWA technology to provide the services, likes vehicle/Industrial devices, handheld UE. With removing the ‘form factor’ in the NOTE, it seems we cannot use FWA anymore.

Observation 1. FWA is a wireless access technology, not a user device, while FWA CPE is a user device.

Observation 2. With removing the ‘form factor’ in the NOTE, it seems we cannot use FWA anymore.

With the above observation, we think it would be fine to use “FWA CPE” terminology in the specification. In addition to that, our understanding is that the "FWA Form Factor” conception includes "FWA CPE" , and the former one also includes some other physical attributes like size, bezels, flat/curved screen, waterproofing, and etc..

Therefore, due to RAN4 had removed ‘form factor’ in the NOTE, so it seems we have no choice but to select if we “FWA CPE” terminology to describe the ΔRIB requirements, and we think it will not impact the existing requirements since it is not the intention to introduce a new UE type.

Proposal 1. To use “FWA CPE” terminology

The definitions for “FWA CPE” can be found in the accompany draft CR.

## 2.2 Release independent

As we mentioned in [3], we think the purpose for the release independent for a feature is to allow this feature can be implemented in an earlier release (even in a earlier frozen release) even though this feature is introduced in later release. The definition for the release independent can be found in TS38.306, which is shown as below:

release independent: applicable to some frozen releases, starting from a certain release Rel-M NOTE 1: Normally, a feature is introduced only in the latest open release Rel-N and future releases are based on the previous one so that future releases inherit the requirements of this feature. Introducing a feature "in a release independent way from Rel-M onwards" (M<N) means it was decided by TSG RAN that this feature would be also beneficial in previous, already frozen releases starting with Rel-M until Rel-(N-1). In order to avoid touching TS 38.101 [2-5] or TS 38.133 [6] of these frozen releases, the corresponding requirements are captured in TS 38.307 via pointers to [2-5] or [6] of the release in which the feature was introduced. NOTE 2: Release independent does not mean applicable to all releases.

From the above definition, due to 6Rx is a feature discussing under Rel-19 and it will be introduced in Rel-19 TS38.101-1 specifications, i.e. 6Rx is a Rel-19 feature. If release independent applies, then it should be from previous release (i.e. Rel-M onwards" (M<N), N=19). For 6Rx, similar as 8Rx where RF requirements for 8Rx UE is release independent from Rel-17, we think it would also be beneficial that this 6Rx feature for both handheld UE and FWA can be release independent from earlier release.

Regarding the SRS antenna switching perspective, we think it should be decoupled with 6Rx feature although ΔTSRSRx requirements for t3r6 are defined, due to the srs-AntennaSwitchingBeyond4RX-r17 capability is an optional capability.

Regarding the different MIMO level, the supportive of 6 layer for handheld UE and FWA undergo a very long controversial discussion in demodulation. Consequently, different performance requirements are defined for 6Rx handheld UE and 6Rx FWA, which are:

-	6-layer performance requirements will be only defined for FWA.

-	4-layer performance requirements will be defined and will apply to FWA and handheld UE devices.

It should noted that whether 4-layer or 6-layer is not considered when RAN4 discuss the RF requirements.

From the above perspective, comparing with the three options in the WF, option 3 is our preference. Moreover, this meeting is the last meeting for 6Rx topic, so if companies have strong concern on option 3, then we think option 1 would be a compromise way.

Proposal 2: Preferring to support 6Rx is release independent from Rel-18 (i.e.option 3)

- Option 1 can be a compromise

# 3 Conclusion

In this contribution, we continue discussing on the release independent for 6Rx RF requirements.

Proposal 1. To use “FWA CPE” terminology

Proposal 2: Preferring to support 6Rx is release independent from Rel-18 (i.e. option 3)

- Option 1 can be a compromise

# 4	Reference

[1] R4-2508086, WF on 6Rx and SRS IL, Huawei

[2] R4-2505649, Discussion on the usage of the terminology “FWA form factor” and its application for 6Rx, 4Rx and 8Rx requirements, Anritsu Limited

[3] R4-2506379 On release independent for 6Rx RF requirement, ZTE Corporation, Sanechips