3GPP TSG-RAN WG4 Meeting #116		R4-2510951

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.22.9

Source: 	CMCC

Title: 			 Discussion on Demodulation performance requirements for Ambient-IoT BS

Document for:	Discussion

- Introduction

In RAN plenary #107 meeting, the revised WID of Solutions for Ambient IoT (Internet of Things) in NR [1] has been approved. The RAN4 performance related scope of this WI is updated as follow:

- Specify the necessary conformance testing for Ambient-IoT BS - Specify the necessary RRM performance requirements for device 1 - Specify the necessary demodulation performance requirements for device 1 and Ambient-IoT BS

Based on the time budget in RP-243326, the performance part of R19 A-IoT should start on August’s meeting. In this contribution, we provide our initial thinking about test scope and simulation parameters.

- Discussion

Test scope

PDRCH (Physical device-to-reader Channel) shall be introduced for A-IOT BS demodulation requirements.

Proposal 1: Introduce new PDRCH channel for A-IOT BS demodulation requirements.

Modulation and Waveform (CW)

Both BPSK and OOK are supported by Device 1, at this stage, we are ok to consider both for simulation assumption. For OOK signal, unmodulated single tone shall be configured.

Proposal 2: Consider both BPSK and OOK for simulation.

Proposal 3: Unmodulated single tone shall be configured for OOK signal.

SFO

In RF reference Sensitivity simulation, a random value selected from the range of [104 ppm~105 ppm] are used as simulation assumption. For demod simulation, we prefer to use the 105 ppm SFO to check the worst case.

Proposal 4: Consider 105 ppm SFO for simulation

Amble configuration and Receiver assumption

We propose to configure preamble of 31bit and midamble 31bit, and assume coherent receiver at reader side to guarantee the performance with 105 ppm SFO.

Proposal 5: Configure 31 bit Preamble and 31 bit Midamble.

Proposal 6: Assume coherent receiver for Reader

Test metric

For now, at least SNR at 10% BLER shall be verified. Whether to introduce false alarm requirement, we prefer to keep it align with RF session, and wait for the conclusion from RF first.

Proposal 7: At least define SNR at 10% BLER requirement.

SCS, Antenna configuration and Propagation condition

We propose 15kHz SCS, 1T2R and TDLA30-10 for initial simulation

Proposal 8: Use 15kHz SCS, 1T2R and TDLA30-10 for initial simulation.

- Conclusion

Proposal 1: RAN4 to agree the latest Demodulation performance part work plan for “Solutions for Ambient IoT (Internet of Things) in NR” as presented in this contribution.

Proposal 1: Introduce new PDRCH channel for A-IOT BS demodulation requirements.

Proposal 2: Consider both BPSK and OOK for simulation.

Proposal 3: Unmodulated single tone shall be configured for OOK signal.

Proposal 4: Consider 105 ppm SFO for simulation

Proposal 5: Configure 31 bit Preamble and 31 bit Midamble.

Proposal 6: Assume coherent receiver for Reader

Proposal 7: At least define SNR at 10% BLER requirement.

Proposal 8: Use 15kHz SCS, 1T2R and TDLA30-10 for initial simulation.

- Reference

- RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc.